Tips Learned from Divers Ever Wiser than I

More Burning Tips courtesy of Cal Preston out of the North Sea

If you are burning through a structure made up of RSJs, such as an old walkway, make your cuts at such an angle that the piece being cut forms a slight wedge. The top of the piece should be a bit wider than the bottom. This will stop it from getting jammed when you eventually come to lifting it out with a crane. It will help to mark the track of your intended angle with a paint stick beforehand.

Talking about wedges, take a few small scrap steel wedges down with you in a pocket with a small hammer. As you go along the length of the cut, bang a wedge into the cut every few feet or so. Again this stops the weight of the top piece from sagging downwards and jamming. It also makes it easier to see if you have left any "holidays" in the cut.

Talking about "holidays", or bridges. If you do leave some, the thing will not come loose under the lifting power of the crane when you claim you are finished. The crane driver will not thank you for making him risk a broken wire. Your vision might be hampered by bad vis, or if the water is clear you will probably blind yourself with the flash at some point, so use a hacksaw blade to confirm the cut is through, and make sure that your section of the cut is complete. Do not expect the next guy to go over your bit again. The wedge tip really helps here.

Check, double check, and check again that O2 will not be trapped in a pocket above the burn site and potentially cause a fatal explosion. If you think there is a chance of a pocket forming, see if you can burn a hole at the top of the area to allow the O2 to escape. You can test the likelihood of a pocket with your breathing gas or pneumo before starting to cut. Take the time to do this because otherwise the "fatal" part might apply to you.

Start the cut at the most awkward spot, and work towards the easy part. Towards the end of the cut, get the crane rigged for the lift and apply a wee bit of tension. Then get yourself into a safe, clear area where you can make the final slice, and watch with satisfaction as that sucker pops slightly upwards and free.(If you have left any holidays you will forego that satisfaction and you will look like a beginner. You will also have to scrabble about under an unstable load looking for the culprit, and be prepared to duck when it does pop free!) Try to sound casual as you inform the supervisor that the load is clear to the surface.

Before you do anything make sure you are in a stable position, get yourself really comfortable, relax, think about the cut and then enjoy yourself. Remember you are the best burner in the North Sea/Gulf of Mexico/Middle East/Far East!
Cal.