- DIVING LAWS:

  - Galileo Galilei (Evangelista Toriceili and Balise Pascal)

Discovered:

PSIA	=   Absolute (in Water PSI and Atmospheric PSI)

PSIG	=   Gauge (in Water PSI Only)

14.7 PSI 	=	1 Atmosphere (Atmospheric PSI Only)

.445 PSI	=	1 ft. of sea water (in Water PSI)

760 mm HG	=	14.7 PSI / 1 ATA (Atmospheric PSI Only)

(mm HG = millimeters of mercury)

2.    Boyle’s Law

Discovered:

P1 V1 	=   P2 V2  	where P = ATA’s and V = Ft.3

NOTE:  	“For any gas at a constant temperature, the volume of the gas will 				  vary inversely with the pressure.”

3.    Charles’ Law

Discovered:

V1  	=   	V2  	      where V = Ft.3 and T = °F

T1		T2

NOTE:	Extension of Boyle’s Law with Temperature

4.     Gay-Lussac’s Law

Discovered:

P1  	=   	P2  	      where P = ATA’s and T = °F

T1		T2

NOTE:	“For any gas at a constant volume, the pressure of the gas will vary 				  directly as the absolute temperature.”

5.    The General Gas Law

Discovered:

P1 V1	    =	P2 V2		where P = ATA’s, V = Ft.3 and T = °F

T1		   T2

NOTE:	Combination of Boyle’s Law, Charles’ Law and Gay-Lussac’s Law.

- Dalton’s Law

Discovered:

Oxygen Partial Pressure while Diving

ATA’s x O2 %  =  PPO2

ATA’s  x  Inert Gas %  =  PPI

760 mm HG  x  CO2 %  =  mm HG		(mm HG = millimeters of mercury)

Safe Diving Parameters:	( .9 – 1.6 ) Max. PPO2’s

1 ATA		=    33 ft. of sea water

2 ATA’s	=    33 fsw

3 ATA’s	=    66 fsw

1 ATA		=    760 mm HG

SAT:  Oxygen Partial Pressure in the DDC (Storage)

Gauge Depth  x  O2 in Gas     +  O2 in Air   =   PPO2

33

Safe Storage Parameters:	SAT ( .25 - .8 ) Max PPO2’s

NOTE:	“The total pressure exerted by a mixture of gases is equal to the sum 			  of the pressures of each of the different gases making up the mixture, 		  with each gas acting as if it alone was present and occupied the total 			  volume.”

- Henry’s Law

Discovered:

The Foundation to Decompression Tables

NOTE:	“The amount of any given gas that will dissolve in a liquid at a given 			  temperature is a function of the partial pressure of the gas in contact 		  with the liquid and the solubility coefficient of the gas in the 				  particular liquid.”

*	Because a large percentage of the human body is water, the law simply states that 	as one dives deeper and deeper, more gas will dissolve in the body tissues and that 	upon ascent, the dissolved gas must be released.

- FIGURING BOTTOM PSI AT DEPTH:

PSIG of Depth  x  .445 PSI per fsw  +  135 PSI minimum manifold PSI  =  Ideal Bottom 	PSI for Diver

- FORMULAS:

Working Dive

Depth ATA’s (x) 1 or 1.5 scf breath per diver per minute (x) minutes of dive time (=) 			Total scf Consumed (x) $ per scf (=) Total $

Floodable Volume for Tankage

Depth Storage ATA’s (x) Tankage scf (=) Blowdown Gas needed for Blowdown scf’s

Cylinder scf

Volume of Cylinder (x) psi of cylinder (÷) 14.7 psi (=) Total scf