# Job Safety Analysis Index

# GENERAL DECK WORK

# Deck Activities

# Surface Maintenance

# LIFTING OPERATIONS, WINCHES AND HOISTS

# Lifting and Moving Equipment

# Load Testing Winch

# Tugger Winches

# Chain Hoists

# Crane Operations

# Crane Testing

# Operating Air and Hydraulic Deck Winches

# ‘A’ Frame Operations

# CUTTING, WELDING, GRINDING AND ASSOCIATED HOT WORK

# Deck Welding Operations

# Welding, Grinding and Burning

# OILS, CHEMICALS ETC.

# Working with Oils

# Working with Chemicals

# WORKING AT HEIGHTS, OVERBOARD OR CONFINED SPACES

# Working at Heights

# Working Overboard

# Manhole Covers

# Fresh Water Tanks Entry

# Ballast Tank Entry

# Fuel Oil Tank Entry

# ELECTRICAL WORK, POWER TOOLS, MOVING MACHINERY

# Working with Lighting Equipment

# Starting Electrical/Hydraulic Power Units

# Operating Workshop Machinery

# Working Inside Main Switchboard

# Job Safety Analysis Index

# Working/Running Machinery

# Working on Electrical Equipment

# Working with Power Tools

# Working with Batteries

# DIVING SYSTEM OPERATIONS

# Bell Control Procedures

# Bell Control Checklists

- Reclaim

- Bell Diving Procedures

- Bell Diving Checklists

- Bell Handling Procedures

- Hot Water System

- Bell Handling Emergency Procedures

- Simultaneous ROV/Diver Operations

- Shallow Water Diving

- Shallow Water Diving Using Extended Umbilicals

- Diving Inside An Anchor Pattern

- Diving Operations Alongside a Platform

- Diving in the Vicinity of a Subsea Structure

- Dynamically Positioned Diving Operations

- Emergency Recovery of the Sat Diving Bell at Water Depths Greater than 50 Meters

SATURATION SYSTEM

- Communications Procedures

- Chamber Control Checklists

- Chamber System Checklists

- Medical and Equipment Lock Operations

- BIBS Operations

- Toilet and Bilge Drain Operations

Job Safety Analysis Index

- Chamber Handover Procedure

- Shutdown Procedures Saturation Control

- Chamber Systems Emergency Procedure

- Hyperbaric Evacuation Phase I

- Hyperbaric Evacuation Phase II

- HRL Checklists

- Dive System Fire Fighting Equipment

- Life Support Package (LSP) Procedures

- Dry-Docking Vessel with Personnel in Saturation

HIGH AND LOW PRESSURE GASES

60. Working with High Pressure Gases

61. Gas Management Procedures

62. Gas Storage Banks and Distribution

63. Transferring Oxygen

64. Transfer of Gas Using Compressors

65. Helium Recovery System

66. Gas Cylinder Charging

67. Pressure Test of the Bell

ROV OPERATIONS

68. ROV Operations

69. ROV Umbilical Winches

70. ROV Cranes

71. Simultaneous ROV/Diver Operations

72. Cutting of ROV Umbilical

73. Towfish Operations

74. Free-Swimming SCV. Launch, Recovery Operations

GENERAL SHIP OPERATIONS, BUNKERING, MOORING ETC.

75. Transferring Fuel to include Bunkering and Deck Equipment

76. Mooring

77. Anchoring, Running Anchor Wires

Job Safety Analysis Index

78. Ballasting

79. Transit Navigation

80. Dynamic Positioning

81. Transponder/Taut Wire

82. Pilot Ladder

83. Gangway

84. Seafastening/Removal of Seafastening

85. Basket Transfer

86. Bunkering at Sea

87. Personnel Transfer at Night using Work Basket

88. Launch/Recovery of Workboat

89. Ship to Ship Transfer of Materials

90. ROV Support Operations Alongside Rig, Platform or Drill Ship

91 .DP Operations Close to a Surface Installation/DP Operations Close to a Subsea Structure

LIFEBOATS, LIFERAFTS, MOB BOAT, HELICOPTER OPERATIONS

92. Lifeboat Drills

93. Liferaft

94. MOB Operations

95. Helicopter Operations

96. Launch/Recovery of HRL

97. Changing Lifeboat Suspension Chains

DOMESTIC CLEANING, GARBAGE COMPACTOR, GALLEY OPERATIONS

98. Domestic Cleaning

99. Galley Operations

100. Garbage Compactor

DIVING AND DECK RELATED OPERATIONAL ACTIVITIES

101. Mobilisation and Demobilisation

102. Dynamically Positioned Diving Operations

103. Shallow Water DP Diving Operations

104. DP Diving Operations Within An Anchor Pattern

Job Safety Analysis Index

105. Diving Operations Alongside a Platform or Structure

106. Diving Operations in the Vicinity of a Subsea Structure

107. Diving Operations In The Vicinity Of Drilling Mud, Cuttings, Etc

108. Overboarding & Deploying Equipment, Concrete Mattresses, Support Piers & Pipeline Spoolpieces From Vessel Deck

to Subsea Worksite

109. Pipeline Flooding Activities

110. Air Diving Operations Utilising Ship’s Crane To Assist In Accessing Worksite

111. Transfer of tubular Lifts From Supply Boat To a Drilling Rig or Platform Whilst Performing Diving Operations

112. Saturation Diving Operations Utilising Extended Diver Excursion Umbilicals To Access The Worksite

OPERATING SUBSEA TOOLS, EQUIPMENT AND DIVER TASKS

113. Water Jets

114. Air Lifting

115. Lifting Bags

116. Dredge Hog Unit

117. Hydraulic Tools

118. Hydratight Equipment

119. Concrete Mattress Installation

120. Oxy-Arc Cutting

121. Diamond Cutter

122. Blind Flange Removal / Installation

123. Riser Clamp Installation

124. Riser Installation

125. Spool Installation

126. Metrology

127. Grout Bag Installation and Filling

128. Move (Sweep) Pipeline

129. Remove (or Install) Start-Up / Lay-Down Head

130. Pigging Pipeline

131. Hydrotesting

132. Cutting and Removing Dummy Spool

133. Tides and Currents

134. Poor Subsea Visibility

135. Flare Boom Radiation

136. Concrete Coating Removal

137. Stinger Checks

STEP 1: SEVERITY CRITERIA

For each identified hazard determine the worst consequence from any of the five headings below (Harm to People, Environment, Damage, Project Schedule, Facility Availability). Multiple consequences may arise in which case the risk is based on the most severe outcome but each consequence should be mitigated accordingly.  For example – ignition of flammable liquids may result in fire/explosion, environmental release, personal injury, delays to schedule and interruption to facility availability. The most severe outcome might be sustained business interruption but measures to mitigate all the other consequences MUST be considered and implemented, as appropriate.  Based upon the worst possible scenario, pick the highest number from any one of the five headings below:

| SEVERITY CRITERIA | HARM TO PEOPLE | ENVIRONMENT | DAMAGE | PROJECT SCHEDULE | FACILITY AVAILABILITY |
| --- | --- | --- | --- | --- | --- |
| 1 NEGLIGIBLE | First Aid Injury | Within site boundary. No significant environmental impact or exceedance of licence conditions. Easily controlled / recovered by worksite | Less than $10,000 Insignificant damage to plant & equipment | Schedule impacted up to 1 hour | Production revenue loss is less than 12 hours |
| 2 MODERATE | Medical Treatment / Restricted Work Case | Within site boundary. Short term environmental impact. Single license exceedance recoverable by worksite. | More than $10,000, less than €50,000 Limited damage to plant & equipment | Schedule impacted more than 1 hour but less than 12 hours. | Production revenue loss is more than 12 hours but less than 1 day |
| 3 SIGNIFICANT | Day Away From Work case (DAFWC) Temporary or permanent partial disability. | Outside the site boundary. Localised pollution giving rise to significant environmental impact but unlikely to last beyond 1month. Repeated exceedance of license conditions. Recovery/rehabilitation may require external assistance. | More than $50,000, less than $250,000 Significant damage to local area or essential plant & equipment | Schedule impacted more than 12 hours but less than 48 hours | Production revenue loss is more than 1 day but less than 1 week |
| 4 SEVERE | Single Fatality. Injury resulting in permanent and severe disability. May prevent Operational Safety Case acceptance | Extended exceedance of license conditions & / or uncontrolled release. Significant environmental impact beyond the site boundary unlikely to last beyond 12 months. Recovery/rehabilitation requires external assistance | More than $250,000 less than $500,000. Damage extending to several areas/significant impairment of installation /equipment integrity | Schedule impacted more than 48 hours but less than 7 days. | Production revenue loss is more than 1 week but less than 1 month |
| 5 CATASTROPHIC | Multiple Fatalities / Multiple Serious Injuries. Likely to prevent operational Safety Case acceptance | Massive & uncontrolled release with significant environmental impact extending well beyond site boundary. Chronic pollution resulting in damage lasting more than 12 months. | Over $500,000 Extensive damage (multiple fires/explosions) or loss of installation | Schedule impacted +7 days | Production revenue loss is more than 1 month |

Step 2 - Probability Criteria

Pick the highest letter based upon any one of the five headings below:

|  | PRE-MITIGATION (Existing control measures) | POST-MITIGATION (With additional control measures) |
| --- | --- | --- |
| A VERY UNLIKELY | To the best knowledge of the risk assessment team the hazard has not occurred within industry | It is considered a freak combination of factors would be required for the hazard to occur at any time |
| B UNLIKELY | To the best knowledge of the risk assessment team the hazard has occurred within industry at least once | A rare combination of factors would be required for the hazard to occur during the course of the work |
| C POSSIBLE | To the best knowledge of the risk assessment team the hazard occurs annually within industry & / or the Company. | Could happen when additional factors are present but otherwise unlikely to occur during the course of the work |
| D LIKELY | To the best knowledge of risk assessment team the hazard regularly occurs more than once a year in the Company. | It is considered likely the hazard could still arise during the course of the work |
| E VERY LIKELY | To the best knowledge of risk assessment team the hazard is predicted to occur at least once during course of the work unless changes are made | Almost inevitable the hazard will occur during the course of the work |

Step 3 – Plot Outcome

Based upon the outcome from section 7.1 & 7.2, plot the severity against the Probability on the table below.

This will determine if the risk is LOW , MEDIUM  or HIGH

|  | PROBABILITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SEVERITY | A | B | C | D | E |
| 1 | 1A | 1B | 1C | 1D | 1E |
| 2 | 2A | 2B | 2C | 2D | 2E |
| 3 | 3A | 3B | 3C | 3D | 3E |
| 4 | 4A | 4B | 4C | 4D | 4E |
| 5 | 5A | 5B | 5C | 5D | 5E |

Step 4 - Risk Tolerability Criteria

| LOW | Tolerable but reasonable mitigation measures should not be ignored |
| --- | --- |
| MEDIUM | Tolerable but ALARP Principles must be demonstrated |
| HIGH | Intolerable if the risk involve Hazards to personnel or environment. Must be mitigated further or the cancellation of the work May be tolerable if it only involves hazards to equipment or schedule. This must be approved by the relevant Manager and/or Client (where relevant) and documented prior to the work going ahead. |

JSA TEMPLATE

| ACTIVITY : | DATE : Reviewed by: |  | PROJECT : |
| --- | --- | --- | --- |
| JSA# | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |

| ACTIVITY : Deck Activities | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA # 01 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Storing gas | - Bottles of different gases stored together - Insecure bottles falling over | - Explosion | H (E4) | - Ensure safe distance kept between different types of gases - Always turn bottles off - Ensure bottles labelled - Keep bottles in storage rack where possible - Tie bottles and hoses securely - Quads to have anti snag device fitted - 1, 2, 3, 4, 5, 8, 9, 11, 12 | L (C1) |
| Working at height | - Person falling - Dropped objects | - Personnel injury | H (E4) | - Wear correct PPE, including safety harness - Remove loose items - Place restricted access barrier as appropriate - For container use ladder to access slings - Access to aft A frame only when in stowed position - 1, 2, 3, 4, 5, 8, 9, 11, 12 | L (C1) |
| Seafastenings | - Insecure load | - Personnel injury - Damage to equipment | H (E4) | - Ensure seafastenings are correct and carried out by a competent person - Inspect all welding - Use certified rigging which has been visually checked - 1, 3, 4, 9, 12 | L (C1) |
| Rigging and slinging | - Rigging failure - Dropped loads - Dropped objects | - Damage to equipment and injury to personnel | H (E5 | - Wear correct PPE - Use certified rigging - Perform visual inspection of rigging - Remove all loose objects - Be extra vigilant - 1, 3, 4, 5, 9, 12 | L (C1) |
| Static rigging - chain hoists - lever hoists - tirfors | - Mechanical failure in appliance | - Damage to equipment - Injury to personnel | H (E5) | - Wear correct PPE - Use certified rigging - Perform visual inspection - Use safety pins in all shackles - 1, 3, 4, 5, 9, 12 | L (C1) |
| Storing steel | - Insecure and unstable steel storage | - Trapped arms or legs | M (C3) | - Store steel in proper rack - Small items to be placed in boxes or containers - 1, 3, 4, 5, 9, 12 | L (B1) |
| Sweeping | - Airborne particles of dirt or dust | - Eye injury | M (C2) | - Sweep upwind if possible - Wear PPE including safety glasses - 1, 3, 4, 5, 9, 12 | L (B1) |
| General lifting and moving equipment by hand | - Straining - Access around deck - Trip hazards - Dropped loads | - Back injury - Pulled muscles - Foot or leg injuries | H (D4) | - General awareness regarding correct lifting techniques - Request assistance if required - Keep walkways clear - General awareness on deck - 1, 3, 4, 5, 9, 12 | L (B1) |
| Washing down using fire main | - Volume of water - Air borne particles | - Eye damage - Man overboard | H (E2) | - Minimum personnel on deck - Hose management - Manned control valve - 1, 5, 12 | L (B1) |
| Washing down using power washer | - Air borne particles - Heat - Chemicals | - Eye damage - Scalding - Skin damage - Injury to personnel | H (E2) | - Minimum personnel on deck - COSHH - 1, 3, 5, 12 | L (B1) |

Note:	All rigging must be certified, colour coded and fit for purpose.

Employ correct lifting techniques when moving heavy or awkward shaped items.

| ACTIVITY : Surface Maintenance | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 02 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Chipping rust | - Excessive noise - Flying particles - Dust containing led, etc. - Damage to electric cables | - Ear damage - Eye injury - Poisoning - Breathing problems - Electric shock - Malfunction of equipment | H (D3) | - Use ear protection - Use safety glasses - Use respiratory protection - Avoid chipping on cables - COSHH - 1, 5, 12 | L (B1) |
| Paintwork outside - brush and rollers | - Exposure to solvents in air - Exposure to solvents on skin - Spillage | - Poisoning by inhalation - Poisoning by skin penetration - Pollution | H (D3) | - Use natural ventilation to carry fumes away - Avoid skin contact - Secure paint tins - Residual paint is restricted garbage - COSHH - 1, 5, 12 | L (B1) |
| Paintwork outside - high pressure paint spraying | - Exposure to solvents in air, fumes and particles - Malfunction, leakage of equipment | - Respiratory poisoning - Skin/eye contamination - Escaping of pressurised paint | H (D3) | - Complete respiratory and body protection - Secure working area - Proper maintenance and checks on equipment - COSHH - 1, 5, 12 | L (B1) |
| Paintwork inside | - Exposure to solvents in air, fumes and particles - Exposure to solvents on skin - Spillage - Accumulating combustible gasses - Lack of oxygen | - Explosion - Suffocation | H (E2) | - Forced ventilation - Limited work periods - Check gas concentration - Respiratory protection - COSHH - 1, 3, 5, 12 | L (B1) |

Note:	Always have adequate ventilation and correct PPE when painting in confined areas.

| ACTIVITY : Lifting & Moving Equipment | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 03 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Lifting and moving equipment with gantry hoist and by hand | - Falling down - Swinging loads - Excessive loads being lifted by hand - Awkward - Heavy loads | - Head injury - Hand injury - Foot injury - Back injury - Damaged equipment | H (D3) | - Safe lifting equipment - Secure the load - Not lifting in rough sea - Not lifting excessive load by hand - Correct lifting procedure - Wear safety equipment - Follow man handling procedures - Use correct stance - 1, 3, 4, 5, 9, 12 | L (B1) |
| Lifting / moving objects at sea with gantry hoist and by hand | - Shifting loads - Objects falling off lifts | - Impact - Equipment damage - Exceeding load limits | H(D4) | - Only certified lifting equipment - Check equipment before use - Experienced personnel - Good communication - Secure the area - Use tag lines on large lifts - Confirm status of cargo to be lifted - Ensure alarm limits are set - 1, 3, 4, 5, 9, 12 | L (B1) |
| Moving equipment / stores with pallet trolley | - Uncontrolled movement | - Crushing injuries | H(D4) | - Only use in stable conditions - 1, 9, 12 | L (B1) |

| ACTIVITY : Load Testing Winch | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 04 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Run A/R wires from sheave to A-frame | - Trapped fingers - Wire falling - Working at heights | - Personnel - Equipment | E 2 | - PPE – Safety Harness - SWL Rigging - Trained rigging crew - Toolbox talk - 1, 2, 3, 4, 5, 8, 9, 11, 12 | E1 |
| Load rigging into A-frame | - Dropped equipment - A-frame/crane/pipe contact | - Personnel - Equipment | B3 | - Good communication between crane/banksman - SWL rigging - Tag lines - 1, 2, 3, 4, 5, 8, 9, 11, 12 | B2 |
| Connect load cell and shackle to A/R wires | - Trapped fingers - Personnel Falling | - Personnel - Equipment | C3 | - Use air tuggers to support shackles - PPE – Safety Harness - Toolbox talk - 1, 2, 3, 4, 5, 8, 9, 11, 12 | C2 |
| Pull load on A/R winches | - Wire/equipment failure | - Personnel - Equipment | E3 | - Clear area in tower and main deck via tannoy from bridge - 1, 2, 3, 4, 5, 8, 9, 11, 12 | E1 |
| De-rig equipment | - Trapped fingers - Personnel Falling | - Personnel - Equipment | C3 | - Use air tuggers to support shackles - PPE – Safety Harness - Toolbox talk - 1, 2, 3, 4, 5, 8, 9, 11, 12 | C2 |

| ACTIVITY : Tugger Winches | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 05 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Operating tugger winches | - Moving parts - Wires under tension | - Trapped limbs - Personnel injury - Material damage | H(C5) | - Follow set up and running procedures - Winch certified and tested - Ensure good communications - Wear correct PPE - Rope guards on winches - Minimum personnel in area - 1, 3, 5, 9, 11, 12 | L (B1) |
|  | - Wire parts | - Whiplash injury to personnel | M(C3) | - Do not use excessive loads or tensions - 1, 3, 5, 9, 11, 12 | L (B1) |
|  | - Bad comms. | - Wire travels in wrong direction | M(B4) | - Radio batteries should be charged before use - 1, 3, 5, 9, 11, 12 | L(A2) |
|  | - Brakes fail | - Free fall of wire or load - Contact with ROV - Injury to diver - Equipment damage | M(B4) | - Maintain load and communicate problem to Supervisor - 1, 3, 5, 9, 11, 12 | L(A2) |

Note:	Always be aware of a wire’s routing and which direction it will go in the event of equipment failure.

| ACTIVITY : Chain Hoists | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 06 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Lifting and lowering loads using chain hoists | - Uncontrolled load | - Personnel injury - Damage to beams, hoist or rigging - Damage to equipment | H(C4) | - Use qualified operators only - Ensure rigging is certified and tested - Wear correct PPE - Operators to be extra vigilant - Routine maintenance - 1, 3, 9, 10, 11, 12 | M(C2) |

| ACTIVITY : Crane Operations | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 07 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Lifting / moving objects in port | - Falling/ swinging loads - Shifting loads - Objects falling off lifts - Limit switches malfunction | - Impact - Equipment damage - Exceeding load limits - Falling load | H(D4) | - Only certified lifting equipment - Check equipment before use - Experienced personnel - Good communication - Secure the area - Use tag lines on all lifts - Confirm status of cargo to be lifted - Ensure alarm limits are set - 1, 3, 4, 5, 8, 9, 10, 11, 12 | L(A4) |
| Lifting / moving objects at sea | - Falling/ swinging loads - Shifting loads - Objects falling off lifts - Limit switches malfunction | - Impact - Equipment damage - Falling load - Exceeding load limits - Injury to person - Conflict with deck operations | H(E5) | - Only certified lifting equipment - Check equipment before use - Experienced personnel - Good communication - Secure the area - Use tag lines on all lifts - Consider sea and wind state - Consider vessel movements - Confirm status of cargo to be lifted - Ensure alarm limits are set - Extra care on lifts from seabed - 1, 3, 4, 5, 8, 9, 10, 11, 12 | L(A4) |
| Lifting/overboarding loads from main deck | - Poor visibility - Poor communications - Command/response delay - Incorrect crane movement - Swinging load | - Impact - Equipment damage - Falling objects - Exceeding load limits - Conflict with deck operations - Injury to personnel | H(E)5 | - Only certified lifting equipment - Check equipment before use - Experienced personnel - Good communication - Secure the area - Use tag lines on all lifts - Consider sea and wind state - Consider vessel movements - Confirm status of cargo to be lifted - Ensure alarm limits are set - Extra care on lifts from seabed - 1, 3, 4, 5, 8, 9, 10, 11, 12 | L(B)2 |
| Lifting/moving objects at sea utilising platform cranes | - Shifting loads - Objects falling off loads - Dropped loads - Swinging loads - Malfunction of limit switches - Damage to ’A’ frame wire - Crane becoming entangled | - Impact - Equipment damage - Exceeding load limits - Injury to personnel - Damage to platform crane | E4=H | - Only certified lifting equipment - Experienced personnel - Good communication between platform and vessel - Minimum personnel on deck - Weather state - Use tag lines on all lifts - Confirm status of cargo to be lifted - Set alarm limits - Ensure loads and slings cannot become trapped - 1, 3, 4, 5, 8, 9, 10, 11, 12 | C1=L |

| ACTIVITY : Crane Testing | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 08 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Operation crane on emergency power | - Dropped object - Burst hoses | - Injury to personnel - Damage to crane - Damage to deck structure - Deck crew - Crane driver - Operational delay - Pollution | H(D3) | - All non-essential personnel to keep clear of deck. - Crane diver to be fully briefed. Hydraulic Tech to be in attendance - Locate suitable test weight within crane parameters - Barrier off deck - Protect deck - Test and inspect hoses - Identify procedure for future hook-up of emergency power pack - 1, 3, 4, 5, 8, 9, 10, 11, 12 | L(A4) |

| ACTIVITY : Operating Air & Hydraulic Deck Winches | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 09 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description |  | Step 2 Hazard |  | Step 3 Who or What Exposed |  | Step 4 Quantify Risk |  | Step 5 Recommended Corrective Actions / Control Measures |  | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating hydraulic winches | - Moving parts - Wires under tension |  | - Trapped limbs - Personnel injury - Material damage |  | D1=M |  | - Follow set up and running procedures - Ensure good communications - Wear correct PPE - 1, 3, 5, 9, 10, 11, 12 |  | C1=L |  |
|  | - Wire parts |  | - Whiplash injury to personnel - Damage to subsea structure - Damage to ROV |  | D1=M |  | - Wire must have in date certificates - Move vessel to safe position during lifting ops. - 1, 3, 5, 9, 10, 11, 12 |  | C1=L |  |
|  | - Bad comms. |  | - Wire travels in wrong direction - Damage to subsea structure |  | D1=M |  | - Radio batteries should be charged before use - Use hard line comms. where possible - 1, 3, 5, 9, 10, 11, 12 |  | C1=L |  |
|  | - Brakes fail |  | - Free fall of wire - Damage to Subsea structure - Damage to ROV |  | D1=M |  | - Routine maintenance - Check brakes before use - 1, 3, 5, 9, 10, 11, 12 |  | C1=L |  |
|  | - HP / LP Fluids |  | - Personnel |  | D1=M |  | - Eye protection - 1, 3, 5, 9, 12 |  | C1=L |  |
| Operating air winches | - Moving parts - Wires under tension |  | - Trapped limbs - Personnel injury - Material damage |  | D1=M |  | - Follow set up and running procedures - Ensure good communications - Wear correct PPE - 1, 3, 5, 9, 10, 11, 12 |  | C1=L |  |
|  | - Wire parts |  | - Whiplash injury to personnel - Damage to subsea structure - Damage to ROV |  | D1=M |  | - Wire must have in date certificates - Move vessel to safe position during lifting ops. - 1, 3, 5, 9, 10, 11, 12 |  | C1=L |  |
|  | - Bad comms. |  | - Wire travels in wrong direction - Damage to subsea structure |  | D1=M |  | - Radio batteries should be charged before use - Use hard line comms. where possible - 1, 3, 5, 9, 10, 11, 12 |  | C1=L |  |
|  | - Brakes fail |  | - Free fall of wire - Damage to Subsea structure - Damage to ROV |  | D1=M |  | - Routine maintenance - Check brakes before use - 1, 3, 5, 9, 10, 11, 12 |  | C1=L |  |
|  | - HP / LP Fluids |  | - Personnel |  | D1=M |  | - Eye protection - 1, 3, 5, 9, 12 |  | C1=L |  |

Note:	Always be aware of a wire’s routing and which direction it will go in the event of equipment failure.

| ACTIVITY : ‘A’-Frame Operations | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 10 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Lifting/moving objects at sea utilising ‘A’ Frame | - Shifting loads - Objects falling off loads - Dropped loads - Swinging loads - Malfunction of limit switches | - Impact - Equipment damage - Exceeding load limits - Injury to personnel | E4=H | - Only certified lifting equipment - Experienced personnel - Good communication - Two independent means of communication, i.e. hand signals / radio / hardwire - Minimum personnel on deck - Weather state - Use tag lines on all lifts - Confirm status of cargo to be lifted - Set alarm limits - Check stability of vessel - 1, 2, 3, 4, 5, 9, 10, 11, 12 | E1=M |
| Lifting/moving objects in port utilising ‘A’ Frame | - Objects falling off loads - Dropped loads - Swinging loads - Malfunction of limit switches - ‘Two blocking’ - Impact damage | - Equipment damage - Exceeding load limits - Falling load - Personnel injury | E2=H | - Only certified lifting equipment - Experienced personnel - Good communication - Minimum personnel on deck - Weather state - Use tag lines on all lifts - Confirm status of cargo to be lifted - Set alarm limits - Check stability of vessel - 1, 2, 3, 4, 5, 9, 10, 11, 12 | C1=L |
| Lubricating ‘A’ Frame wire | - Falling - Dropped objects - Slipping on grease - Unauthorised starting ‘A; frame and winches | - Injury to maintenance crew/ deck personnel | B2=L | - Experienced personnel only - Wear harnesses - 1, 2, 3, 4, 5, 9, 10, 11, 12 | A1=L |
| Routine maintenance | - Falling - Dropped objects | - Injury to maintenance crew/ deck personnel | B2=L | - Experienced personnel only - Wear harnesses - 1, 2, 3, 4, 5, 9, 10, 11, 12 | A1=L |

| ACTIVITY : Deck Welding Operations | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 11 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. FIRE EXTINGUISHER |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Changing oxygen bottles | - Contamination by oil - Sparks or naked flames | - Explosion | H(D4) | - Ensure cleanliness - Isolate bottles - Ensure no hot work in vicinity - 1, 3, 4, 5, 12, 13 | L(A4) |
| Changing bottles acetylene | - Sparks or naked flames | - Explosion | M(C2) | - Ensure no hot work in vicinity - Isolate bottles - 1, 3, 4, 5, 12, 13 | L(A3) |
| Burning | - Flash back - Naked flame | - Explosion - Burns - Eye injury | H(D4) | - Permit to work in place - Ensure flash back arrestors fitted to gauges - Wear correct PPE, including gloves and burning visor - 1, 3, 4, 5, 12, 13 | L(A4) |
| Welding | - Welding arc - Flammable material - Fumes - Electric Cable | - Burns - Eye injury - Fire - Lung damage - Electric shock | H(D4) | - Permit to work in place - Wear correct PPE, including gloves and welding visor - Utilise a fire watcher and have fire blankets to hand - Use extractors and dust mask - Ensure cables insulated - 1, 3, 4, 5, 12, 13 | L(A4) |
| Arc air gouging | - High pressure air hose bursts - Welding arc - Flammable materials - Electric Cable | - Burst hose whips around - Burns, eye injury - Fire - Electric shock | H(D4) | - Permit to work in place - Wear correct PPE - Ensure hose safety straps used on connections - Fire watcher to be utilised - Fire blankets in vicinity - Ensure cables insulated - 1, 3, 4, 5, 12, 13 | L(A4) |
| Grinding | - Sparks - Electrical Cables - Grinding or cutting wheel shatters - Noise | - Fire - Electric shock - Personnel injury - Eye injury - Ear damage | H(D4) | - Permit to work in place - Wear correct PPE - Ensure cables insulated - Qualified personnel only to change wheels - Fire watcher to be utilised - Fire blankets in vicinity - Ear protection - Ensure correct voltage - 1, 3, 4, 5, 12, 13 | L(A3) |
| Welding slag removal | - Hot slag | - Burns - Eye damage | H(D4) | - Eye protection, gloves, overalls - 1, 3, 4, 5, 12, 13 | L(A3) |
| Repair burning leads | - High pressure leak in lead | - Personnel injury | H(D4) | - Wear correct PPE - Ensure leads are correctly stored - Check frequently - 1, 3, 4, 5, 12, 13 | L(A3) |
| Repair welding leads | - Exposed electrical cable due to split lead | - Electric shock | M(C4) | - Wear correct PPE - Ensure leads are correctly stored - Check frequently - Isolate before repair - 1, 3, 4, 5, 12, 13 | L(A3) |

Note:	Ensure that personnel involved in welding/burning operations know the whereabouts of fuel tanks, etc.

Be aware of personnel working nearby when involved in grinding/burning/gauging operations.

Isolate and secure all equipment when not in use.

| ACTIVITY : Welding, Grinding, Burning | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 12 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. FIRE EXTINGUISHER |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Welding, grinding and burning | - Sparks - Fire - Welding flash - Hot metals - Explosion | - Fire - Burns - Eye injury - Hand injury | H(D4) | - Cleaning working area - Safety equipment - Fire equipment - Check that bunkering fuel is not be performed - De-gassify tanks/area - Adequate ventilation - 1, 3, 4, 5, 9, 12, 13 | M(C2) |
| Burning | - Explosion | - Injury - Burns | H(D4) | - Check that flashback arrestors are fitted - 1, 3, 4, 5, 9, 12, 13 | M(C2) |
| Welding, burning | - Working in confined area | - Fumes - Heat - Noxious gases | H(D4) | - Check area prior to starting work - Fume extraction - FiFi equipment to be on site - 1, 3, 4, 5, 9, 12, 13 | M(C2) |
|  | - Hot electrode stubs | - Burns - Damage to equipment | H(D4) | - Welders to discard stubs into bucket or similar container - 1, 3, 4, 5, 9, 12, 13 | M(C2) |
| Welding | - Electrocution | - Heart stoppage - Burns | H(D4) | - Cables to be intact - Inspection prior to start work - 1, 3, 4, 5, 9, 12, 13 | M(C2) |
|  | - Radiation - Fumes | - Burns to operator and crew - Lung damage | H(D4) | - Screens to be erected around - Extraction - Adequate ventilation - 1, 3, 4, 5, 9, 12, 13 | M(C2) |

Note:	Hot work permits are to be issued by bridge crew only.

On completion of work or suspension of permit the person in charge must inform all personnel working under that permit of status.

| ACTIVITY : Working with Oils | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 13 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Storage | - Leakage - Fire - Temperature control | - Pollution - Explosion - Spontaneous combustion - Slipping | M(C4) | - Lockable/secure store - Wear correct PPE - Utilise temperature monitoring device with visual/audio alarm - Use smoke/heat sensors - Have available extinguishing media, CO2/deluge system/dry power/ foam - 1, 3, 6, 11, 12 | L(A4) |
| Transportation | - Leakage - Fire - Temperature changes | - Pollution - Explosion - Spontaneous combustion - Personnel injury | M(C4) | - Specialised handling equipment and transportation systems - Specialised absorbents - Foam/COs/dry power extinguishers available - Temperature monitoring devices - 1, 3, 9, 11, 12 | L(A4) |
| Handling | - Spillage - Handling heavy drums - Contact with skin - Fittings blow off | - Pollution/fire - Personnel injury - Slip hazard - Injury to personnel | H(D4) | - Specialised absorbents - Wear correct PPE - Medical facilities available - Foam/CO2/dry powder extinguishers to be available - Fittings are properly tightened - 1, 3, 9, 11, 12 | L(A4) |
| Operational use | - Equipment malfunction - Atomising/leakage - Overheating | - Pollution - Fire/explosion - Personnel injury | H(D5) | - Wear correct PPE - Specialised absorbents - Medical facilities available - Foam/CO2/dry powder extinguishers to be available - Temperature monitoring devices - 1, 3, 6, 9, 11, 12 | L(A4) |
| Disposal | - Offloading/ craneage - Container leakage | - Pollution - Fire - Personnel injury | H(D4) | - Wear correct PPE - Medical facilities available - Foam/CO2/dry powder extinguishers to be available - Specialised absorbents - COSHH - 1, 3, 6, 9, 11, 12 | L(A3) |

Note:	Use correct lifting method for lifting oil drums.

Keep clear of high pressure hoses when in operation.`

Be aware of COSHH.

| ACTIVITY : Working with Chemicals | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 14 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Working with chemicals | - Leaking pollution - Gasses | - Eye injury - Hand injury - Breathing problems | H(D5) | - Using the correct equipment - Having cleaning material around - Having eye wash available - Working in ventilated area - 1, 2, 3, 4, 5, 8, 9, 11, 12 | L(A4) |
|  | - Wrong chemicals for job | - Potential explosion | H(D5) | - COSHH assessment sheets available | L(A4) |
| Lifting chemicals | - Falling loads - Chemical spillage | - Injury to personnel - Damage to environment | H(D5) | - Correct slinging to be employed - Absorbents to be ready - 1, 2, 3, 4, 5, 8, 9, 11, 12 | L(A4) |

Note:	A COSHH. assessment must be performed on all chemicals.

| ACTIVITY : Working at Heights | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 15 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Painting/cleaning | - Working at heights - Dropped objects - Solvent/ chemical exposure - Hit by radar antenna | - Fall injury - Skin/eye injury - Long term solvent injury | H(D5) | - Second person to watch vicinity - Secure objects - Respiratory protection - Ensure adequate work platform - Do not overstretch from safe area - Signs and barriers in place - 1, 3, 4, 5, 12 | M(E1) |
| Working up mast | - Working at heights - Dropped objects | - Fall injury - Injury to persons below | H(D5) | - Second person - Secure objects - Fall arrestor - 1, 3, 4, 5, 9, 12 | L(A4) |
| Routine maintenance | - Falling - Dropped objects | - Injury to maintenance crew/ deck personnel | H(D5) | - Experienced personnel only - Ensure adequate work platform - Do not over stretch from safe area - 1, 3, 5, 6, 7, 8, 12 | L(A4) |

| ACTIVITY : Working Overboard | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 16 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  |  |  | Project Specific |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. APPROPRIATE PPE |  | 5. PERMIT TO WORK |  | 9. SUPERVISION |  |  | 13. |  |  |
| 2. TASK SPECIFIC PROCEDURE |  | 6. MECHANICAL ISOLATION |  | 10. CERTIFICATION |  |  | 14. |  |  |
| 3. SPECIFIC TRAINING |  | 7. ELECTRICAL ISOLATION |  | 11. TOOL BOX TALKS |  |  | 15. |  |  |
| 4. RESTRICT WORK AREA ACCESS |  | 8. POST BARRIERS / SIGNS |  | 12. COMPETENT PERSON |  |  | 16. |  |  |
| Step 1 Task/Activity Description | Step 2 Hazard |  | Step 3 Who or What Exposed |  | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures |  | Step 6 Residual Risk |  |
| Painting/ cleaning | - Fall overboard into water - Fall overboard onto quay - Falling objects onto quay exposure to solvents and chemicals |  | - Drowning - Impact injury - Eye injury - Chemical burns - Solvent damage |  | H(D5) | - Secure equipment - guard quay area - Respiratory protection to be used - Second person to watch - Adequate work platform - Do not over stretch - Harness and fall arrestor - 1, 3, 5, 9, 12 |  | M(E1) |  |
| Launching mob boat or workboat | - Fall overboard - Rigging failure |  | - Drowning - Impact injury |  | H(D5) | - Enter boat before lifting from deck - Any more than two personnel to enter boat at pilot ladder - Harness and fall arrestor - 1, 3, 5, 9, 12 |  | L(A4) |  |
| Rigging | - Fall overboard - Rigging failure - Dropped tools and equipment |  | - Drowning - Impact injury |  | H(D5) | - Wear life vest/safety harness - Permit - Correct PPE |  | L(A4) |  |
| Burning, welding | - Fall overboard - Rigging failure - Dropped tools and equipment - Burns |  | - Drowning - Impact injury - Burns |  | H(D5) | - Fire extinguisher on hand - See risk assessment burning/welding - Fall arrestor - 1, 3, 4, 5, 9, 11, 12 |  | L(A4) |  |

Note:	If working overboard in a personnel basket ensure that a safety pennant is used and strops are made from nylon.

| ACTIVITY : Manhole covers | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 17 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description |  | Step 2 Hazard |  | Step 3 Who or What Exposed |  | Step 4 Quantify Risk |  | Step 5 Recommended Corrective Actions / Control Measures |  | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Opening Closing | - Flooding of water, fuel, etc. - Leaking - Escape of fumes, vapours or gases |  | - Impact injury - Damaged equipment - Pollution - Fire risk - Explosion risk - Risk of intoxication |  | E1 = M |  | - Check fluid level in tank - Leave 4 bolts for safe opening - Clean surfaces on flanges - Good packing - Tighten all bolts – never leave job part done - 1, 2, 3, 4, 5, 8, 9, 11, 12 |  | A1 = L |  |
| Entering void spaces | - Asphyxia |  | - Death |  | E3=H |  | - When entering void spaces do relevant gas checks before entering - Second person present, inform bridge - 1, 2, 3, 4, 5, 6, 7, 9, 11, 12 |  | E1 = M |  |
| Opening, closing | - Lifting heavy object |  | - Back injury - Trapped fingers |  | C3=M |  | - Use correct lifting techniques - 1, 2, 3, 4, 5, 6, 7, 9, 11, 12 |  | A2 = L |  |
| Opening | - Falling down hole |  | - Injury/death - Deck crew |  | E2 =H |  | - Erect safety barriers around hole - 1, 2, 3, 4, 5, 6, 7, 9, 11, 12 |  | E1 = M |  |

Note:	Do not leave open holes unguarded, or perform any hot work nearby until gas free check has been performed.

| ACTIVITY : Fresh water tanks entry | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 18 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description |  | Step 2 Hazard |  | Step 3 Who or What Exposed |  | Step 4 Quantify Risk |  | Step 5 Recommended Corrective Actions / Control Measures |  | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Inspection/ works | - Lack of oxygen - Slippery surface and poor lighting - Pollution - Being shut in tank - Confirmation from adjacent tanks or pipework |  | - Suffocation - Fall from height - ‘Epidemic’ diseases |  | E2 = H |  | - Ventilation, oxygen test - Two personnel present - Ensure sufficient lighting - Wear good/clean footwear - Walk on dry areas only - Remove all loose material (rags, paint, etc.) - 1, 2, 3, 4, 5, 8, 9, 11, 12 |  | E1 = M |  |
|  | - Slip/fall |  | - Injury - Injured person in inaccessible area |  | C2=M |  | - Have adequate lighting - 1, 2, 3, 4, 5, 8, 9, 11, 12 |  | C1 = L |  |
|  | - Wrong tank |  | - Personnel enter wrong tank |  | E2 = H |  | - Study ships plan before starting works - 1, 2, 3, 4, 5, 8, 9, 11, 12 |  | E1 = M |  |
| - Maintenance | - Leave undesirable items in tank |  | - Blockage of pipework etc. |  | C2 = M |  | - Remove all items after completion - 1, 2, 3, 4, 5, 8, 9, 11, 12 |  | C1 = L |  |

Note:	When entering water tanks work in pairs at all times.

| ACTIVITY : Ballast tank entry | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 19 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description |  | Step 2 Hazard |  | Step 3 Who or What Exposed |  | Step 4 Quantify Risk |  | Step 5 Recommended Corrective Actions / Control Measures |  | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Inspection/ works | - Lack of oxygen - Slippery surface and poor lighting - Pollution - Being shut in tank - Confirmation from adjacent tanks or pipework |  | - Suffocation - Fall from height - ‘Epidemic’ diseases |  | E2 = H |  | - Ventilation, oxygen test - Two personnel present - Ensure sufficient lighting - Wear good/clean footwear - Walk on dry areas only - Remove all loose material (rags, paint, etc.) - 1, 2, 3, 4, 5, 8, 9, 11, 12 |  | E1 = M |  |
|  | - Slip/fall |  | - Injury - Injured person in inaccessible area |  | C2=M |  | - Have adequate lighting - 1, 2, 3, 4, 5, 8, 9, 11, 12 |  | C1 = L |  |
|  | - Wrong tank |  | - Personnel enter wrong tank |  | E2 = H |  | - Study ships plan before starting works - 1, 2, 3, 4, 5, 8, 9, 11, 12 |  | E1 = M |  |
| - Maintenance | - Leave undesirable items in tank |  | - Blockage of pipework etc. |  | C2 = M |  | - Remove all items after completion - 1, 2, 3, 4, 5, 8, 9, 11, 12 |  | C1 = L |  |

| ACTIVITY : Fuel Oil Tanks Entry | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 20 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description |  | Step 2 Hazard |  |  | Step 3 Who or What Exposed |  |  | Step 4 Quantify Risk |  | Step 5 Recommended Corrective Actions / Control Measures |  |  | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inspection/ works | - Lack of oxygen - Explosion/fire - Slippery surface - Poor lighting |  |  | - Suffocation - Burns/death - Structure damage - Fall from height |  |  | E4 = H |  | - Ventilate, oxygen test prior to entry - Second person to be present - Gas detection for entry - Gas free certificate for hot work - Ensure sufficient lighting - Good footwear to be worn - Intrinsically safe equipment to be used (radios/torches) etc. - 1, 2, 3, 4, 5, 8, 9, 11, 12 |  |  | E1 = M |  |
|  | - Slip/fall |  |  | - Injury - Injured person in inaccessible area |  |  | C2=M |  | - Have adequate lighting - 1, 2, 3, 4, 5, 8, 9, 11, 12 |  |  | B2 = L |  |
|  | - Wrong tank |  |  | - Personnel enter wrong tank |  |  | E2 = M |  | - Study ships plan before starting works - 1, 2, 3, 4, 5, 8, 9, 11, 12 |  |  | E1 = M |  |
| Maintenance | - Leave undesirable items in tank |  |  | - Blockage of pipework etc. |  |  | C2=M |  | - Removal of all items after completion - 1, 2, 3, 4, 5, 8, 9, 11, 12 |  |  | B2 = L |  |
| ACTIVITY : Working with Lighting Equipment |  |  | DATE : Reviewed by: |  |  |  |  |  |  |  | PROJECT : Vessel Generic |  |  |
| JSA# 21 |  |  | LOCATION : |  |  |  |  |  |  |  | REF PROCEDURES: |  |  |
| SAFETY – FIRST PRIORITY |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Working with lighting equipment | - Electrical shock | - Heart attack | H(E5) | - PTW - Risk Assessment - 1, 3, 5, 7, 12 | L(A2) |
|  | - Working at heights | - Fall injury - Personal injury | H(E5) | - Safe working platform - Do not over stretch - Secure tools - Restrict access under worksite - 1, 3, 5, 7, 12 | M(C2) |
|  | - Dropped objects | - Personal injury | M(C4) | - Secure tools - Restrict access under worksite - 1, 3, 5, 7, 12 | L(A2) |
|  | - Breaking bulbs | - Personal injury - Hand and eye injury | M(C4) | - Isolations - Restrict access under worksite - 1, 3, 5, 7, 12 | L(A2) |
| Working by light alone | - Loss of light | - Disorientation - Injury | M(C2) | - Signs and barriers - 1, 3, 5, 7, 12 | L(A2) |

Note:	Do not use portable lights unless safety tag is present.

| ACTIVITY : Starting Electrical/Hydraulic Power Units | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 22 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Starting any electric/hydraulic power unit | - HP oil | - Damage to equipment - Destruction of moving parts - Lost time - Injury to personnel | H(C5) | - Ensure start/stop procedure is followed - Ensure all valves, gauges, etc. are clearly marked/tagged - Ensure equipment is maintained - Ensure safety procedures are in place - 1, 3, 5, 7, 12 | L(A2) |
|  | - Fire | - Damage to equipment - Destruction of moving parts - Lost time - Injury to personnel | H(C5) | - Ensure start/stop procedure is followed - Ensure all valves, gauges, etc. are clearly marked/tagged - Ensure equipment is maintained - Ensure safety procedures are in place - 1, 3, 5, 7, 12 | L(A2) |
|  | - Pollution | - Injury to personnel | M(C4) | - Ensure start/stop procedure is followed - Ensure all valves, gauges, etc. are clearly marked/tagged - Ensure equipment is maintained - Ensure safety procedures are in place - 1, 3, 5, 7, 12 | L(A3) |
| Starting any electric/hydraulic power unit (cont) | - Moving parts | - Injury to personnel | H(C5) | - Ensure stop / start procedure is followed - 1, 3, 5, 7, 12 | L(A3) |

Note:	Check all equipment for loose items before starting.

| ACTIVITY : Operating Workshop Machinery | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 23 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Operating workshop machinery | - Moving parts | - Hand injury - Eye injury | H(C5) | - Using safety equipment - Secure material safely - PPE - 1, 3, 6, 12 | L(B2) |
|  | - Metal particles flying around | - Hand injury - Eye injury | H(C5) | - Use safety equipment - Secure material safely - PPE - 1, 3, 6, 12 | L(B2) |
|  | - Hot metals | - Hand injury - Eye injury - Burns | M(C2) | - Use safety equipment - Secure material safely - PPE - 1, 3, 6, 12 | L(B2) |
|  | - Improper use of equipment | - Injury to personnel | H(D4) | - Do not allow untrained personnel to operate equipment - PPE - 1, 3, 6, 12 | L(B2) |

Note:	Check all equipment for loose items before starting workshop machinery.

| ACTIVITY : Working Inside Main Switchboard | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 24 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Working inside main switchboard | - Electrical shock | - Ship damage - Heart attack - Burns - Fire - Eye injury | H(D5) | - Remove fuses - Isolate off power - Inform that you are inside switchboard by posting a notice - Rubber matting - 1, 2, 3, 4, 5, 7, 8, 9, 12 | L(A3) |
|  | - Short circuit | - Ship damage - Heart attack - Burns - Fire - Eye injury | H(D5) | - Isolate power - Remove fuses - Secure switches - Inform that you are inside switchboard by posting a notice - Rubber matting - 1, 2, 3, 4, 5, 7, 8, 9, 12 | L(A3) |

Note:	Where possible always isolate switchboard, or work in pairs.

| ACTIVITY : Working/Running Machinery | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 25 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Working / running machinery | - Moving parts - Hot surfaces | - Hand injury - Burns | M(C4) | - Install guards on exposed machinery - Post hazard notices - 1, 3, 12 | L(A3) |
|  | - Noise | - Damage to hearing | M(C2) | - Wear suitable ear protection - 1, 3, 12 | L(A3) |
|  | - Material fracture/rupture | - Impact injury - Contamination by oil etc. | M(C4) | - Use guards - Correct PPE - 1, 3, 12 | L(A3) |
|  | - Entrapment | - Injury | H(D4) | - Use guards at all times - 1, 3, 12 | L(A4) |

Note:	Personnel should be familiar with all machinery and the dangers associated with them.

| ACTIVITY : Working on Electrical Equipment | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 26 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Working on electrical equipment | - Electrical shock | - Heart attack - Hand injury - Burns - Eye injury | M(C4) | - Switch off main power - Place notice on switch and fuses - Work in pairs - 1, 3, 4, 5, 7, 9, 12 | L(A4) |
|  | - Moving parts | - Heart attack - Hand injury - Burns - Eye injury | M(C4) | - Switch off main power - Place notice on switch and fuses - Work in pairs - 1, 3, 4, 5, 7, 9, 12 | L(A4) |
|  | - Short circuit | - Heart attack - Hand injury - Burns - Eye injury | M(C4) | - Switch off main power - Place notice on switch and fuses - Work in pairs - 1, 3, 4, 5, 7, 9, 12 | L(A4) |

Note:	Where possible always isolate equipment where applicable or work in pairs.

| ACTIVITY : Working with Power Tools | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 27 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Working with high pressure and high revolution tools | - Noise | - Injury to personnel | M(C4) | - Ear protection - Equipment must have ‘safe to use’ tags - 1, 3, 4, 6, 12 | L(A4) |
|  | - Dust | - Respiratory problems - Injury to personnel | M(C4) | - Clean and good working area - Equipment must have ‘safe to use’ tags - Inspect tool before use - 1, 3, 4, 6, 12 | L(A4) |
|  | - Leaking | - Injury to personnel | M(C4) | - Clean and good working area - Safe equipment with a dead man button - Equipment must have ‘safe to use’ tags - Inspect tool before use - 1, 3, 4, 6, 12 | L(A4) |
|  | - Tools slipping | - Injury to personnel | M(C4) | - Safe equipment with a dead man button - Equipment must have ‘safe to use’ tags - Inspect tool before use - 1, 3, 4, 6, 12 | L(A4) |
| Working with high pressure and high revolution tools (cont) | - Electrical shock | - Injury to personnel | M(C4) | - Correct voltage - Equipment must have ‘safe to use’ tags. - Inspect tool before use. - 1, 3, 4, 6, 12 | L(A4) |

Note:	Personnel using power tools must be competent in the use of them.                 Fire watch when using grinding tools.

| ACTIVITY : Working with Batteries | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 28 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Working with batteries | Short circuit | Fire Hand and eye injury | M(C4) | Putting tools in a safe place to avoid short circuit Using the right testing equipment - 1, 2, 3, 4, 5, 6, 7, 8, 12 | L(A3) |
|  | Spilling acid | Fire Hand and eye injury | M(C4) | Keeping the battery clean Using the right testing equipment Good housekeeping COSHH - 1, 2, 3, 4, 5, 6, 7, 8, 12 | L(A3) |
|  | Explosion | Fire Hand and eye injury | M(C4) | Good ventilation Keeping the battery clean Using the right testing equipment - 1, 2, 3, 4, 5, 6, 7, 8, 12 | L(A3) |
|  | Manual handling | Back injury | M(C4) | Using the right testing equipment - 1, 2, 3, 4, 5, 6, 7, 8, 12 | L(A3) |
| Charging batteries | Explosion | Death/injury | H(D4) | Have good ventilation when charging batteries - 1, 2, 3, 4, 5, 6, 7, 8, 12 | L(A3) |

Note:	Special care should be taken when handling chemical batteries.

Know COSHH.

| ACTIVITY: Bell Control Procedures | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 29 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Divers TUP | Loss of pressure | Injury to divers | H(C5) | Check mating faces / ‘O’ seals every TUP Over pressure chamber door to isolate system Decompress trunking 5m Check for leaks - 1, 2, 3, 9, 12 | M(C2) |
| External bell checks | Slips Falls | Personnel injury | M(C4) | Harness to be used if necessary Two persons to perform checks if possible - 1, 2, 3, 9, 12 | M(C2) |
| Locking bell off / on | Trapped fingers Crush injury | Personnel injury | H(C5) | Restrict persons around bell Ensure good visual / audio comms Use trained persons only Audible alarms when spool / clamp is in motion - 1, 2, 3, 9, 12 | M(C2) |
| Closing / opening clamp | Trapped fingers Crush injury | Personnel injury | H(C5) | Restrict persons around bell Ensure good visual / audio comms Use trained persons only Audible alarms when spool / clamp is in motion - 1, 2, 3, 9, 12 | M(C2) |
| Extending / retracting spool | Trapped fingers Crush injury | Personnel injury | H(C5) | Restrict persons around bell Ensure good visual / audio comms Use trained persons only Audible alarms when spool / clamp is in motion - 1, 2, 3, 9, 12 | M(C2) |
| Opening / closing side door | Trapped fingers Crush injury | Personnel injury | H(C5) | Restrict persons around bell Ensure good visual / audio comms Use trained persons only Audible alarms when spool / clamp is in motion - 1, 2, 3, 9, 12 | M(C2) |
| Lowering / raising bell | Trapped fingers Crush injury | Personnel injury | H(C5) | Restrict persons around bell Ensure good visual / audio comms Use trained persons only Audible alarms when spool / clamp is in motion - 1, 2, 3, 9, 12 | M(C2) |
| Pressurising bell / chamber trunking | Unable to obtain seal Loss of pressure | Noise from trunk Injury to personnel | M(C4) | Check sealing faces before bell locks on Ensure interlocks and vent valves are closed before pressurising Stop blowdown at 5 - 10m to check for leaks and interlock is operational - 1, 2, 3, 9, 12 | L(A3) |
| Divers TUP from bell to chamber | Loss of pressure | Injury to personnel | M(E1) | Divers check for audible leaks prior to transfer Transfer through as quickly as possible Keep chamber / bell doors closed where possible Obtain seal on chamber door once bellman has passed through - 1, 2, 3, 9, 12 | L(A3) |
| Blowing bell to depth | Bell internal temperature too hot Incorrect PPO2 | Too hot for divers to do checks Unconscious diver | L(A4) | Pressure bell slowly Analyse blowdown gas before use Analyse bell PPO2 before diver enters bell - 1, 2, 3, 9, 12 | L((A3) |
| Preparing bell for surfacing | Opening / closing wrong valves | Cannot surface the bell | M(B3) | Carry out valve check using checklist - 1, 2, 3, 9, 12 | L(A3) |
| Removal of equipment likely to be damaged when bell is surfaced | Equipment damage due to rapid ascent Rupture of sealed containers | Equipment damage Personnel injury | M(C2) | Remove pressure sensitive items Break comms / TV connections Remove lids from bottles - 1, 2, 3, 9, 12 | L(A2) |
| Flushing bell with air | Insufficient flushing | Pockets of helium Personnel injury caused by unconsciousness | M(C4) | Ensure flushing line fully in bell Flush for at least 5 mins Two people present before entering bellUse PPO2 meter - 1, 2, 3, 9, 12 | L(A3) |
| Entering bell for first time after flushing | Heavy bottom door Water in bilge | Personnel injury Water discharging from bell | H(C5) | - TTwo people to open door if possible - Uuse of PPE - Uuse PPO2 meter - 1, 2, 3, 9, 12 | L(A3) |
|  | Pockets of helium | Collapse of diver | H(C5) | Ensure at least 5 minutes of flushing Two people present before entering bell - 1, 2, 3, 9, 12 | L(A3) |
| Storage of gas in bell onboard bottles / BOBs | Wrong gas mix O2% too high O2% too low | Anoxia / hypoxia Hyperoxia | H(C5) | Plan dive mixes prior to dive Analyse gases used - 1, 2, 3, 9, 12 | L(A3) |
| Storage of gas in bell onboard bottles / BOBs | Loss of time | Loss of time while charging banks | M(C2) | Plan dive in good time - 1, 2, 3, 9, 12 | L(A2) |
| Lowering, raising and securing stand off frame | Uncontrolled drop of frame Winch breakdown Dropped objects Lost equipment Personnel slip/falls Delay | Bell Frame Personnel | B3=M | Check winches before operation Ensure rigging in good order Ensure rigging securely attached - 1, 2, 3, 9, 12 | A3=L |
| Lowering and raising moonpool side door flaps | Uncontrolled drop of frame Winch breakdown Dropped objects Lost equipment Personnel slip/falls Delay | Bell Frame Personnel | B3=M | Check winches before operation Ensure rigging in good order Ensure rigging security attached When raised, all equipment to be secured - 1, 2, 3, 9, 12 | A3=L |

Note:	Personnel should never forget they are working in, around and near high-pressure gas. Warning notices should be displayed in relevant areas.

It is the bellman’s responsibility to secure all loose items in the bell. When transferring to-from diving bells close all doors a.s.a.p. to maintain system integrity.

| ACTIVITY : Bell Control Checklists | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 30 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Bell external checks | Miss out valves/ equipment | No gas to divers Equipment malfunction | M(C4) | Laminated check sheet to be used - 1, 2, 3, 4, 9, 12 | L(A3) |
| Valve checks | Trip - fall - bang head on bell handling structure | - Injury to diver doing checks | H(D4) | Harness to be worn if needed Care to be taken Two persons if possible - 1, 2, 3, 9, 12 | L(A3) |
|  | Sudden vent. Off of gas | - Injury to diver doing checks | H(C5) | Dive supervisor to co-ordinate between divers and surface Warning to be given Short burst principle to be used - 1, 2, 3, 9, 12 | L(A3) |
|  | Working on - near pressure vessels | - Injury to diver doing checks | M(C4) | Care to be taken at all times Visual checks to be carried out - 1, 2, 3, 9, 12 | L(A3) |
| Re-tooling work basket | Diver can slip/fall | - Personnel injury | M(C2) | Work in pairs where possible - 1, 2, 3, 9, 12 | L(A3) |
| Bell internal checks | Accidentally opening a valve that vents to surface | - Divers inside - personnel outside - Rapid decompression - Noise | M(C4) | Divers to be fully familiarised with bell internals Personnel outside to be made aware of any gas venting - 1, 2, 3, 4, 9, 12 | L(A3) |
|  | Oxygen-enriched gas in bell | Oxygen toxicity to diver inside Fire hazard | M(C4) | O2 analysers to be kept on and calibrated at all times Continuous PPO2 monitoring | L(A3) |
|  | Oxygen-deficient gas in bell | Lack of O2 to sustain consciousness | M(C4) | O2 analysers internal and dive controlContinuous PPO2 monitoring | L(A3) |
|  | Cut fingers Grazed knuckles | Wound becomes infected | M(C2) | Take care when opening/closing awkward placed valves | L(A3) |
|  | Confined space with protrusions | Unconsciousness Diver trapped in bell | M(C4) | Always store equipment properly - tie down, etc Take care whilst moving | L(A3) |
|  | Incorrect atmosphere | Diver cannot concentrate on checks | M(C2) | Keep bell at constant temperature comfortable for divers | L(A3) |
| Using / calibrating analysers | Incorrect atmosphere | Unconsciousness of divers due to incorrect gas mix | H(C5) | PPO2 in atmosphere to be monitored Facility to cross-check readings Adequate analysers Calibration - 2, 3, 9, 12 | L(A3) |
| Using smoke diving equipment in dive control | Equipment not functioning Insufficient pressure in cylinders | Dive supervisor Divers in bell if applicable Empty cylinders | M(C4) | Re-test date to be visible Contents gauge to be checked regularly Comms to be checks regularly - 2, 3, 9, 12 | L(A3) |
| Pre-dive gas valve set up | Wrong gas to divers | Unconsciousness of divers | H(C5) | Care to be taken when setting up panel Double check - 2, 3, 12 | M(C2) |
| Pre-dive gas valve set up | No gas to divers | Divers - no gas emergency situation | M(C4) | Double check valves are open - 2, 3, 12 | M(C2) |
| Reclaim console set up | Wrong gas to divers | Unconsciousness of divers | M(C4) | Double check set up Understand full operation of panel - 2, 3, 12 | M(C2) |
| Operation of pneumos | Incorrect depth of divers | Divers - wrong storage depth selected Wrong decompression schedule selected | M(C4) | Gauges should have calibration certs. visible - 2, 3, 12 | L(A3) |
| Bell control shut-down checklist | Bell depth increases/ decreases | Delay diving | M(C2) | Close all valves securely - 2, 3, 12 | L(A3) |
| Onboard O2 topping up | Explosion | Injury to gasman or divers depending on severity | M(E1) | Equipment to be O2 cleaned etc. Safety notices in place - 2, 3, 4, 12 | L(A3) |
| Onboard O2 topping up | Pressure too high | Explosion Relief valve blowoff | H(C5) | Make sure upper pressure limits are in operation - 2, 3, 4, 12 | L(A3) |

Note:	Personnel should never forget they are working in, around and near high-pressure gas. Extra care should be taken when handling HP oxygen. Safety notices should be displayed in relevant areas.

Surface standby diver shall wear safety harness/life jacket when necessary.

Always adhere to checklists. If in doubt stop and report to Shift Supervisor.

| ACTIVITY : Reclaim (Closed Circuit Breathing System) | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 31 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Pre-start checks and settings | - Incorrect settings | - Failure to operate within operational parameters | M(C4) | - Follow correct start up procedures and checklists - 1, 2, 3, 4, 12 | L(A3) |
| Diver breathing from Reclaim System | - Low O2 - High O2 - High CO2 | - Asphyxia - O2 toxicity - Hypothermia - Unconsciousness - Convulsions - Divers | H(E5) | - Ensure adequate analysis and monitoring equipment and close supervision | M(E1) |
|  | - Equipment malfunction | - Incorrect supply of pressure / negative pressure | M(C4) | - Routine maintenance - Know procedure to change to open circuit | L(A4) |
|  | - Water vapour in breathing gas | - Diver feels unwell | M(C4) | - Check rubber diaphragms regularly | L(A3) |

Note:	When operating CCBS particular attention must be paid to O2 injection and analysers.

Always use and adhere to checklist. If unsure stop operations and seek advice.

| ACTIVITY : Bell Diving Procedures | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 32 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| At depth connect and test standby divers hat for gas and comms. | Malfunctioning equipment | Abort dive | L(A3) | Bell checks to be done properly 1, 2, 3, 9, 12 | L(A3) |
| Diver leaves bell | Hot water malfunctions | Diver gets cold (hypothermia) | M(C2) | Hot water temp. is checked before start of bell run Set alarms 1, 2, 3, 9, 12 | L(A3) |
|  | Loss of gas to diver | Diver goes on to bailout / onboard gas | H(C5) | Pre-lock out checks done properly Gas pressures recorded before dive start | L(A4) |
|  | Comms to diver stop working | No comms to diver | M(C2) | Flash hat light, give four pulls Follow procedure | L(A3) |
| Diver descending from bell to seabed | Diver cannot clear his ears | Burst ear drum Diver can become disoriented | M(C4) | Diver to be lowered slowly if ears are sticky | L(A3) |
|  | Diver has a tooth cavity problem | Extreme pain for diver | M(C2) | Diver to stop descent when pain onsets | L(A3) |
| Diver working in water | Build up of CO2 | Diver gets headache, disoriented, etc. | H(D4) | When working hard, flush mask through 1, 2, 3, 9, 12 | L(A4) |
|  | Umbilical is fouled | Diver becomes trapped | M(C4) | Diver regularly checks umbilical back to bell Umbilical management | M(C2) |
| Diver working in water (cont) | Bad gas to diver | Diver becomes unconscious | H(E5) | Pre-dive checks Liase with gasman Set alarms / analysers | M(E1) |
|  | Too much divers umbilical paid out of bell | Entrapment / thruster danger | H(D5) | Bellman / divers / supervisors are familiar with umbilical markings Umbilical management | M(C2) |

Note:	Always use and adhere to checklists.              Be aware of umbilical lengths when shallow saturation diving.

| ACTIVITY : Bell Diving Check Lists | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 33 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Wrong gas mix Wrong gas pressure in O/B bottles Bailouts | - Reduced emergency breathing time - Bailout U/S | - Divers pass out - Wrong mix - Reduced time - Wrong pressure | C3=M | - Gas man records pressures and mix and gives copy to shift supervisor - Analyse gas before going online - 1, 2, 3, 9, 12 | C1 = L |
| Bell external checks | - Miss out valves/ equipment | - No gas to divers - Equipment malfunction | C3=M | - Laminated check sheet to be used - 1, 2, 3, 9, 12 | C1 = L |
| Valve checks | - Trip - fall - bang head on bell handling structure | - Injury to diver doing checks | C3=M | - Harness to be worn if needed - Care to be taken - 1, 2, 3, 9, 12 | C1 = L |
| Valve checks | - Sudden vent. - Off of gas | - Injury to diver doing checks | C3=M | - Dive supervisor to co-ordinate between divers and surface - 1, 2, 3, 9, 12 | C1 = L |
| Valve checks | - Working on - near pressure vessels | - Injury to diver doing checks | C3=M | - Care to be taken at all times - 1, 2, 3, 9, 12 | C1 = L |
| Re-tooling work basket | - Diver can slip/fall into moonpool | - Drowning | C3=M | - Wear a harness - life jacket - Work in pairs where possible - 1, 3, 9, 12 | C1 = L |
| Equipment checks on bell external | - Port on bell smashed | - Injury/death to divers inside and outside the bell | C3=M | - No lifting of heavy/awkward equipment near bell - 1, 2, 3, 9, 12 | C1 = L |
| Bell internal checks | - Accidentally opening a valve that vents to surface | - Divers inside - personnel outside - Rapid decompression | C3 = M | - Divers to be fully familiarised with bell internals - Personnel outside - Pressure vessels - 1, 2, 3, 9, 12 | C1 = L |
| Bell internal checks | - Oxygen-enriched gas in bell | - Oxygen toxicity to diver inside - Fire hazard | C3=M | - O2 analysers to be kept on and calibrated at all times - 1, 2, 3, 9, 12 | C1 = L |
| Bell internal checks | - Oxygen-deficient gas in bell | - Lack of O2 to sustain life/ consciousness | C3=M | - O2 analysers etc. - Hull stops to be kept closed where appl. - 1, 2, 3, 9, 12 | C1 = L |
| Bell internal checks | - Cut fingers - Grazed knuckles | - Wound becomes infected | C3=M | - Take care when opening/closing awkward placed valves using knives - 1, 2, 3, 9, 12 | C1 = L |
| Bell internal checks | - Bang head | - Unconsciousness - Diver trapped in bell | C3=M | - Always store equipment properly - tie down, etc. - 1, 2, 3, 9, 12 | C1 = L |
| Bell internal checks | - Bell internal too hot/cold | - Diver cannot concentrate on checks | C3=M | - Try and keep bell at constant temperature comfortable for divers - 1, 2, 3, 9, 12 | C1 = L |
| Operation of pneumos | - Incorrect depth of divers | - Divers - wrong storage depth selected - Wrong decompression schedule selected | C3=M | - Gauges should have calibration certificates visible - 1, 2, 3, 9, 12 | C1 = L |

Note:	Bell external checks to be done in pairs where possible.

| ACTIVITY : Bell Handling Procedures | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 34 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed |  | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- | --- |
| Personnel movements under pressure | - Depth changes - Environmental changes - O2 parameter changes - Door/hatch opening or closing |  | - Decompression sickness - Extremes of temperature - Asphyxia - O2 toxicity - Personnel injury - Entrapment - Chamber occupants | M(C4) | - Adhere to correct working procedures - 3, 9, 12 | L(A4) |
| Climbing ladder to transfer | - Falling/slipping - Objects dropped |  | - Fall/injury - Chamber occupants | M(C4) | - Adhere to correct working procedures Secure any loose objects or equipment - 3, 9, 12 | M(C2) |
| Establishing a seal | - No seal - ‘O’ ring seal not seated/damaged - Vent valve left open |  | - Prolonged exposure to risk - Unable to transfer - Unable to pressurise trunk - Delay in operations - Decompression sickness - Chamber occupants | H(C5) | - Ensure clean seals - Adhere to correct operating/working procedures - Adhere to maintenance procedures - Good communications - 3, 9, 12 | M(C2) |
| Clamp removal | - Interlock malfunction - Pressure still in trunk - Poor communications - Vent line blocked - Gauge inoperative |  | - Pressure retention - Pressure release - Injury - Decompression sickness - Mechanical and material damage - Bell occupants and people in vicinity | M(C4) | - Adhere to maintenance schedules - Adhere to correct working procedures - Good communication procedure - 1, 2, 3, 4, 9, 12 | M(C2) |
| Starting hydraulic machinery | - Electric’s |  | - Electric shock - Operator and people in vicinity | M(C4) | - Follow correct working and maintenance schedules - Display local warning signs and good ships husbandry - Good communication procedure - 1, 2, 3, 9, 12 | L(A4) |
|  | - Mechanical |  | - Entrapment - Personal injury - Operator and people in vicinity | M(C4) | - Follow correct working and maintenance schedules - Display local warning signs and good ships husbandry - Good communication procedure - 1, 2, 3, 9, 12 | L(A3) |
|  | - Remote start |  | - Entrapment - Personal injury - Operator and people in vicinity | M(C4) | - Follow correct working and maintenance schedules - Display local warning signs and good ships husbandry - Good communication procedure - 1, 2, 3, 9, 12 | L(A3) |
|  | - Release of high pressure hydraulic fluids |  | - Injection of fluids into the body - Personal injury - Contamination of environment - People in vicinity | M(C4) | - Follow correct working and maintenance schedules - Display local warning signs and good ships husbandry - Good communication procedure - 1, 2, 3, 9, 12 | L(A4) |
| Bell deployment | - Moving of bell and ancillary equipment |  | - Entrapment - Personal injury - Mechanical and material damage - Bell occupants and people in vicinity | H(C5) | - Follow correct working and maintenance schedules - Display local warning signs and good ships husbandry - Good communication procedure - 1, 2, 3, 9, 12 | L(A4) |
| Operating clump weight | - Entrapment in moving wires |  | - Injury - Severed umbilical - Bell occupants and bell handling crew | M(C4) | - Follow correct working and maintenance schedules - Inform divers of movements - Display local warning signs and good ships husbandry - 1, 2, 3, 9, 12 | L(A4) |

Note:	Divers must be informed of all movements below water level.

Qualified and competent people only to operate specialised equipment.

| ACTIVITY : Hot Water System | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 35 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Starting hot water machine | - Electricity | - Electric shock - Operator | M(C4) | - Certified equipment - 1, 2, 3, 10, 12 | L(A2) |
| Starting pumps | - Dead heading | - Mechanical damage - Operator and people in vicinity | L(A4) | - Follow correct starting procedures - 1, 2, 3, 10, 12 | L(A3) |
|  | - Electricity | - Electric shock - Operator | M(C4) | - Certified equipment - 1, 2, 3, 10, 12 | L(A3) |
| Operational activity | - Fluctuating temperature | - Hot water burns - Cooling divers in water - Diving operations stopped | M(C4) | - Adequate monitoring and early warning sensors - 1, 2, 3, 10, 12 | L(A3) |

Note:	Hot water system must be operated and maintained by trained and competent personnel.

| ACTIVITY : Bell Handling Emergency Procedures | DATE : 12 Dec 09 Reviewed by: J.Windsor |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 36 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Use of emergency procedures | - Inability to assess the emergency situation | - Inappropriate response to the emergency | H(E5) | - Thorough assessment of situation - Practice emergency procedures - 2, 3, 9, 11, 12 | M(C4) |
|  | - Misunderstanding of procedures | - Confusion among personnel | H(E5) | - Routine practice drills - 2, 3, 9, 11, 12 | M(C4) |
| Personnel movements under pressure | - Depth changes | - Decompression sickness - Chamber occupants | H(D5) | - Adhere to correct working procedures - 2, 3, 9, 11, 12 | M(E1) |
|  | - Door/hatch opening or closing | - Back injury - Entrapment - Chamber occupants | H(D5) | - Adhere to correct working procedures - 2, 3, 9, 11, 12 | M(E1) |
| Establishing a seal | - No seal - ‘O’ ring seal not seated/damaged - Vent valve left open | - Prolonged exposure to risk - Unable to transfer - Unable to pressurise trunk - Delay in operations - Decompression sickness - Chamber occupants | H(D5) | - Ensure clean seals - Adhere to correct operating/working procedures - Adhere to maintenance procedures - 2, 3, 9, 12 | M(C2) |
| Starting hydraulic machinery | - Electric’s | - Electric shock - Operator and people in vicinity | H(D4) | - Follow correct working and maintenance schedules - Display local warning signs and good ships husbandry - 1, 2, 3, 6, 9, 10, 12 | L(A4) |
|  | - Mechanical | - Entrapment - Personal injury - Operator and people in vicinity | M(C4) | - Follow correct working and maintenance schedules - Display local warning signs and good ships husbandry - 1, 2, 3, 4, 7, 10, 12 | L(A3) |
|  | - Remote start | - Entrapment - Personal injury - Operator and people in vicinity | M(C4) | - Follow correct working and maintenance schedules - Display local warning signs and good ships husbandry - 1, 2, 3, 9, 10, 11, 12 | L(A3) |
|  | - Release of high pressure hydraulic fluids | - Injection of fluids into the body - Personal injury - Contamination of environment - People in vicinity | H(D4) | - Follow correct working and maintenance schedules - Display local warning signs and good ships husbandry - 1, 2, 3, 9, 10, 11, 12 | L(A4) |
| Operating clump weight | - Entrapment in moving wires | - Injury - Severed umbilical - Bell occupants and bell handling crew | H(D5) | - Follow correct working and maintenance schedules - Display local warning signs and good ships husbandry - Good communication practices - 1, 2, 3, 9, 10, 11, 12 | L(A4) |
| Operating valves in Hydraulic Powerpack room to emergency position | - Operate wrong valves because of no emergency lighting | - Winches will not operate or be rendered inoperable | H(E5) | - Emergency lighting installed - Torches to be used in meantime - 1, 2, 3, 9, 10, 11, 12 | M(D2) |

| ACTIVITY : Simultaneous ROV/Diver Operations | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 37 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| ROV/diver working in close proximity | Electric shock to diver ROV hits diver ROV umbilical gets tangled in divers umbilical ROV hits bell ROV looses power ROV thrusters | Death/injury to diver Crush injury to diver Diver gets dragged off job/to surface Possible injury to bellman divers Entanglement/ knocks diver Damage to Divers umbilical Trapped fingers Diver entrapment | M(C2) | Test of earth circuit leakage breaker every 24 hrs. min. ROV maintains a safe distance from diver ROV maintains safe distance Good tether management ROV is equipped with sonar etc. in poor visibility ROV keeps down stream of diver in tides Make sure suitable guards are fitted over all thrusters Dive Supv has monitor with ROV pilot vision Open comms Dive / ROV control 3, 9, 11, 12 | L(A3) |

Note:	ROV must adhere to Company safe operations procedure. 
ROV pilot must inform Dive Supervisor of all ROV movements on the seabed.

| ACTIVITY : Shallow Water Diving | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 38 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Shallow water diving operations. | - Ship’s Thrusters. | - Diver Injury. - Umbilical entrapment. - Diver entrapment. - Barotrauma. | E2 = H | - Umbilicals secured in bell at maximum permissible length. - Bellman to inform Supervisor of umbilical length deployed from bell/basket. | B2 = L |
|  | - Exceeding diver’s excursion limits. - Tidal rise and fall. - Excessive subsea current. - Reduced visibility. - DP reference system limitations. | - Reduced DP reference system. - Diver - Operational limits. - Loss of HPR signal. - Diver - Hull interference to Taut Wire. - Diver - Degradation of DP status. - Diver | B2 = L | - Monitor diver’s depth and PPO2. - Cross reference structure/worksite. - Bellman/divers/supervisors are familiar with umbilical markings - Good management of diver umbilical. - Constant tending - Ensure umbilicals are negatively buoyant. - Ensure bailouts are charged to capacity | B1 = L |
| Shallow water diving operations (cont) | - Confused communications - Erratic vessel move. - Close proximity of DP reference systems to the worksite. | - Possible unplanned vessel movement. - Injury to diver. - Increased power consumption. - Excessive thruster wash. - Increased subsea noise. - Interference to divers. | B2 = L | - Generate accurate tidal graph. - Monitor current speed and rise and fall. - Diver only to lock in/out during periods when current speed is workable. - Work within limits of HPR & Taut Wire. - Pre- plan location of HPR transponders and taut wires. | B1 = L |

| ACTIVITY : Shallow Water Diving Using Extended Umbilicals | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 39 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Divers need more umbilical than permitted. | - Umbilical float into thrusters. | - Diver Fatality | D3 = | - Generate procedures using tending divers . - Deploy 2nd diver to tend 1st diver. - Positive communications. - Good umbilical discipline (avoid tangling). - Terminate dive early if current increasing. - All divers briefed before going into saturation. - Copy of procedure in Dive Control, and to the divers (in plastic) - Pre-planning | C1 = L |

| ACTIVITY : Diving Inside an Anchor Pattern | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 40 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Operational activities in vicinity of anchor pattern with diving bell deployed. | - Failure of ships equipment | - Loss of ship’s position. - Contact with anchor wire. - Loss of diving bell. | E3 = H | - Where possible lower nearest anchor wire until below diving bell. - Confirm position and catenary anchor wires utilising ROV. | E1 = M |
|  | - Subsea cables/chains. | - Damage to diving bell. - Loss of diving bell life support services. | E3 = H | - Ensure at least three independent reference systems are on line and a fourth available on standby. | E1 = M |
|  | - Loss of reference system. | - Diver entrapment. - Loss of breathing gas. - Loss of divers hot water. | E3 = H | - DP officers on watch to be extra vigilant. - Identify vessel escape routes in event of emergency. | E1 = M |
|  |  | - Divers. | E3 = H | - Diving bell to be deployed the maximum possible practicable horizontal distance from anchor wire. | E1 = M |
|  |  |  | E3 = H | - Diving bell to be stored at upward extent of ascending excursion. - Diver excursion umbilical to be properly managed at all times. | E1 = M |
| Operational activities in vicinity of anchor pattern with diving bell deployed (cont) |  |  | E3 = H | - Weather to be monitored closely and vessel should not work in excess of 25-30 upwind at Master’s discretion. | E1 = M |
|  |  |  | E3 = H | - Anchored structure requirements to be implemented and adhered to. - Communication with floater to be established and maintained throughout dive. | E1 = M |

| ACTIVITY : Diving Operations Alongside a Platform | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 41 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Diving operations utilising dynamic positioning alongside a Platform or Structure | - DP reference system conflict. - Loss of DP. - Loss of HPR signal. - Confused communications. - Dropped objects. - Dumping of mud/cuttings. - Simultaneous operations. - Strong surface currents. - Platform based emissions. - Foreign objects entangling with thrusters | - Damage to vessel. - Taut wire weight hits diver. - Taut wire weight parts. - Degradation of DP status. - Damaged or broken crane wire or hook. - Collision. - possible unplanned vessel movement. - Bell collides with structure. - Reduced visibility. - Injury to diver. | D3 = H | - Obtain permission to enter 500-metre zone. - Identify platform based external activities. - Use full system redundancy. - Monitor all vessel moves. - Regular checks of taut wire rigging. - DPO must not deploy, reposition or recover taut wire without informing Dive Supv. - Work within limits of reference systems. - Monitor DGPS system. - Identify vessel escape routes. - Confirmation of HPR transponders in use and taut wire locations. - maintain good clear communications. | D1 = M |

| ACTIVITY : Diving in the Vicinity of a Subsea Structure | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 42 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Diving operations in the vicinity of a fixed subsea structure. | - Erratic vessel movement. - DP reference system conflict. - Trapped, snagged crane wire. - Loss of HPR signal. | - Loss of ship’s position. - Damage to equipment being installed. - Bell collides with structure. - Injury to diver. - Taut wire weight entanglement. | D3 = H | - Use full system redundancy. - Monitor weather. - Confirm all vessel moves. - Regular checks of taut wire rigging. - DPO must not deploy, recover or reposition taut wire without informing the Dive Supv. | C1 = L |
| Diving operations in the vicinity of a fixed subsea structure (cont) | - Confused communications. - Deployment of subsea equipment. | - Degradation of DP status. - Damaged or broken crane hook. - Dropped objects. - Collision | D3 = H | - Work within limits of the HPR. - Monitor DGPS system. - Identify vessel escape routes. - Confirmation of HPR transponders in use and taut wire locations. - Locate bell and clump weight above the structure. | C1 = L |

| ACTIVITY : Dynamically Positioned Diving Operations | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 43 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Diving Operations utilising Dynamic positioning. | - DP reference system limitations. - Confused communications. - Erratic vessel move. - Third party shipping. - Rapid change in current, speed and direction. - Rapid change/deterioration in environmental conditions. - Close proximity of DP reference system to the worksite. | - Reduced DP reference system operational limits. - Loss of HPR signal. - Hull interference to Taut Wire. - Degradation of DP status. - Collision. - Possible unplanned vessel movement. - Injury to diver. - Increased power. - Excessive thruster wash. - Increased subsea noise. - Interference to divers. |  | - Operate vessel in accordance with regulatory and statutory requirements. - Utilise available system redundancy and maintain lookout. - Maintain good radar watch. - Confirm all vessel moves. - D.P.O. must not deploy reposition or recover Taut Wire without informing Dive Supv. - Work within limits of HPR & Taut Wires. - Monitor fishing and third party vessel activity. |  |
| Relocation of ship whilst diving bell is deployed. | - Failure of ships equipment. - Loss of power. - Loss of reference system. | - Loss of Ship’s position. - DP reference system exceeding operational limits. - Diver entrapment. | D2 = M | - Ensure sufficient independent reference systems are on line one of which should be a surface reference system. - DP Officers on watch to be extra vigilant. - Good clear concise communications. - Inform Dive Supv. of all vessel movements ie. heading and position. | C1 = L |

| ACTIVITY : Emergency Recovery of the Sat Diving Bell at Water Depths Greater than 50 metres | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 44 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Loss of bell main lift wire: Both guide wires and umbilical intact | - Damage to umbilical - Descending bell - Falling wire - Loss of services/life support to the bell - Trapped or entangled wire | - Injury to divers - Diver services/life support - Entrapment - Trapped bell | C3=M | - Establish communications - Release bell wire (if required) - Recover divers back to bell - Seal bell - Recover bell utilising guide weight secondary recovery system to cursor – ROV or surface diver to check bell prior to locking into cursor - TUP as per normal operations | B2 =L |
| Loss of bell main lift wire and umbilical: Both guide wires intact – Utilising through-water communications | - Damage to umbilical - Descending bell - Falling wire - Loss of services/life support to the bell - Falling umbilical - Flooded bell - Trapped or entangled wire | - Injury to divers - Diver services/life support - Entrapment - Trapped bell - Umbilical fouling vessel’s thrusters | C3=M | - Establish communications (through-water comms) - Release bell wire/umbilical (if required) - Recover divers back to bell - Operate emergency valve shutdown procedure - Seal bell - Recover bell utilising guide weight secondary recovery system to cursor – ROV or surface diver to check bell prior to locking into cursor - TUP as per normal operations | B2=L |
| Loss of bell main lift wire and umbilical: Both guide wires intact - No communications | - Damage to umbilical - Descending bell - Falling wire - Loss of services/life support to the bell - Falling umbilical - Flooded bell - Trapped or entangled wire | - Injury to divers - Diver services/life support - Entrapment - Trapped bell - Umbilical fouling vessel’s thrusters | C3=M | - Try to establish communications (through-water comms) - Release bell wire/umbilical (if required) - Recover divers back to bell - Operate emergency valve shutdown procedure - Seal bell - Confirmation of seal must be established utilising ROV or OUTSIDE ASSISTANCE before recovery of bell can commence - Recover bell utilising guide weight secondary recovery system to cursor – ROV or surface diver to check bell prior to locking into cursor - TUP as per normal operations | B2=L |
| Loss of bell main lift wire, the umbilical and one guide wire: One guide wires intact - Utilising through-water communications | - Damage to umbilical - Descending bell - Falling wires - Loss of services/life support to the bell - Falling umbilical - Flooded bell - Trapped or entangled wires | - Injury to divers - Diver services/life support - Entrapment - Trapped bell - Umbilical fouling vessel’s thrusters | C3=M | - Establish communications (through-water comms) - Release bell wire/umbilical (if required) - Recover divers back to bell - Operate emergency valve shutdown procedure - Seal bell - Recover bell utilising guide weight secondary recovery system on the remaining guide wire to cursor – ROV or Surface diver to check bell prior to locking into cursor - surface diver assistance may be required to enable bell to TUP as per normal operations | B2=L |
| Loss of bell main lift wire, the umbilical and one guide wire: One guide wires intact - No communications | - Damage to umbilical - Descending bell - Falling wires - Loss of services/life support to the bell - Falling umbilical - Flooded bell - Trapped or entangled wires | - Injury to divers - Diver services/life support - Entrapment - Trapped bell - Umbilical fouling vessel’s thrusters | C3=M | - Try to establish communications (through-water comms) - Release bell wire/umbilical (if required) - Recover divers back to bell - Operate emergency valve shutdown procedure - Seal bell - Confirmation of seal must be established utilising ROV or OUTSIDE ASSISTANCE before recovery of bell can commence - Recover bell utilising guide weight secondary recovery system on the remaining guide wire to cursor – ROV or surface diver to check bell prior to locking into cursor - ROV or surface diver assistance may be required to enable bell to TUP as per normal operations | B2=L |
| Lost Bell Situation Loss of bell main lift wire, the umbilical and both guide wires: Utilising through-water communications | - Damage to umbilical - Descending bell - Falling wires - Loss of services/life support to the bell - Falling umbilical - Flooded bell - Trapped or entangled wires | - Injury to divers - Diver services/life support - Entrapment - Trapped bell - Umbilical fouling vessel’s thrusters | C3=M | - Establish communications (through-water comms) - Release bell wire/umbilical (if required) - Recover divers back to bell - Operate emergency valve shutdown procedure - Seal bell - If the bell is stable call for assistance from another vessel for through water transfer of divers - If no other vessel available within 24 hours use ROV to hook up dedicated tugger and follow recovery procedure | B2=L |
| Lost Bell Situation Loss of bell main lift wire, the umbilical and both guide wires: No communications | - Damage to umbilical - Descending bell - Falling wires - Loss of services/life support to the bell - Falling umbilical - Flooded bell - Trapped or entangled wires | - Injury to divers - Diver services/life support - Entrapment - Trapped bell - Umbilical fouling vessel’s thrusters | C3=M | - Try to establish communications (through-water comms) - Release bell wire/umbilical (if required) - Recover divers back to bell - Operate emergency valve shutdown procedure - Seal bell - Confirmation of seal must be established utilising ROV or OUTSIDE ASSISTANCE before recovery of bell can commence - If the bell is stable call for assistance from another vessel for through water transfer of divers - If no other vessel available within 24 hours use ROV to hook up dedicated tugger and follow recovery procedure | B2=L |

| ACTIVITY : Communications Procedures | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 45 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Communicating with bell control | - Not following procedures correctly | - Misinformation - Misdirection - Decompression sickness - Mechanical/ material damage - Pressure release - Bell occupants, divers and deck crew | M(C4) | - Correct operating procedures - Alternative communication method - Maintenance schedules - Familiarisation of all equipment | L(A4) |
| Communicating with chamber control | - Not following procedures correctly | - Misinformation - Misdirection - Decompression sickness - Mechanical/ material damage - Pressure release - Chamber occupants, divers and deck crew | M(C4) | - Correct operating procedures - Alternative communication method - Maintenance schedules - Familiarisation of all equipment | L(A4) |

Note:	Communication equipment must function correctly and be in a good state of repair.

Ensure instructions are clear and concise and everything said is understood.

| ACTIVITY : Chamber Control Checklists | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 46 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Pre-saturation checklists | - Low oxygen in chamber - Obstructions in and around chamber system - Procedures not followed correctly | - Injury - Mechanical/ electrical damage - Operator and persons in vicinity | M(C4) | - Analyse oxygen content of chambers - General awareness - Adhere to checklists and correct operating procedures | L(A4) |
| Calibration and set up of chamber monitoring equipment | - Electricity - Use of high pressure gas | - Injury - Mechanical/ electrical damage - Operator and persons in vicinity | M(C4) | - Adhere to checklists and correct operating procedures - Analyse all gases - Set alarms | L(A4) |
| Set up of gas distribution panel | - Oxygen and high pressure gas - Incorrect regulator settings | - Injury - Mechanical/ electrical damage - Incorrect gas mixes/volume supplied - Operator and persons in vicinity | H(D5) | - Adhere to checklists and correct operating procedures - Check pressures - Analyse all gases | M(E1) |
| Chamber control consoles set up | - Oxygen and high pressure gases | - Injury - Mechanical/ electrical damage - Incorrect gas mixes/volume supplied - Operator and persons in vicinity | H(D4) | - Adhere to checklists and correct operating procedures - Note: Status tags must be noticeable on all gas bottles and distribution panels. | L(A4) |

| ACTIVITY : Chamber System Checklists | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 47 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Valve operation | - Pressure release | - Decompression of chambers - Chamber occupants | H(D4) | - Follow operating check lists - Restrict access | L(A4) |
| Auto start of LSS unit | - Moving machinery parts | - Injury - Damage - Dive technician | M(C4) | - Local warning notices - Isolate starter | L(A4) |
| Soda sorb change | - Chemical dust | - Irritation of eyes and respiratory system - Operator | M(C4) | - Use suitable safety precautions | L(A4) |
| Chamber checks | - Unsafe environment |  |  |  |  |
|  | - Low O2 levels | - Asphyxia - Injury - Operator | H(D4) | - Analyse environment - Safe working practices | L(A4) |
|  | - High pressure gas systems | - Pressure release - Injury - Misdirection - Operator and people in vicinity | H(D4) | - Adhere to safe working practices and maintenance schedules | L(A4) |
| Chamber checks (cont) | - Electrical systems | - Electric shock - Fire - Operator and people in vicinity | H(D4) | - Adhere to safe working practices and maintenance schedules | L(A4) |
|  | - Obstructions | - Injury to person - Operator | H(D4) | - Be aware and vigilant | L(A4) |
|  | - Deck plates not secured | - Injury | M(C4) | - After cleaning ensure deck plates are fastened | L(A4) |
|  | - Chemical | - Injury - Burns | H(D4) | - Follow safe working practices | L(A4) |
|  | - Valve operation incorrect | - Wrong gas - No gas - Mechanical damage | H(D4) | - Follow checklist procedures | L(A4) |

Note:	Make people aware if  you are working on equipment by posting warning notices.

| ACTIVITY : Medical & Equipment Lock Operations |  | DATE : Reviewed by: |  |  |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- | --- | --- | --- |
| JSA# 48 |  | LOCATION : |  |  |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |  |  |  |
| Basic/ General Safety Requirements |  |  |  | Project Specific |  |  |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK |  |  | 9. SUPERVISION | 13. |  |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION |  |  | 10. CERTIFICATION | 14. |  |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION |  |  | 11. TOOL BOX TALKS | 15. |  |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS |  |  | 12. COMPETENT PERSON | 16. |  |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Venting lock | - Pressure release - Noise - Items retaining pressure | - Damage to contents - Injury - Operator and people in vicinity | H(D5) | - Follow correct lock operating procedures - Alert personnel of possible hazardous items | L(A4) |
| Pressurising lock | - Pressure increase - Lock not secure | - Damage to contents - Injury to chamber occupants - Operator and people in vicinity | H(D5) | - Follow correct lock operating procedures | M(E1) |
| Lock operations | - Wrong communications | - Incorrect operations - Pressure release - Injury - Operator and people in vicinity | H(C5) | - Follow correct lock operating procedures | M(E1) |
|  | - Combustible/ unauthorised items in lock | - Damage to contents - Fire - Injury - Operator and people in vicinity | M(C4) | - Follow correct lock operating procedures | L(A4) |

Note:	Medical lock use is restricted to personnel who are competent and trained.

Safety of lock operations could be further enhanced, by fabrication of an external pressure control line, thus giving full control of operation to the LST

| ACTIVITY : BIBS Operations | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 49 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Breathing on BIBS | - Incorrect mix | - Asphyxia - O2 toxicity - Chamber occupants | H(D5) | - Analysation - Correct routing - Isolation | L(A4) |
|  | - Incorrect valve operation | - No gas - Injury - Pressure release - Chamber occupants | H(D4) | - Correct valve labelling - Adhere to correct procedures | L(A4) |
|  | - Inadequate hygiene | - Cross infection - Chamber occupants | M(C4) | - Adhere to hygiene procedures when cleaning masks | L(A3) |

Note:	Before breathing on BIBS make sure you know what the mix is. Ask questions.

| ACTIVITY : Toilet & Bilge Drain Operations | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 50 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Valve operation internal | - Pressure release - Valve left open | - Loss of depth - Decompression sickness - Chamber occupants | M(C4) | - Valve identification - Follow correct procedures - Fit spring loaded valves | L(A4) |
| Toilet bowl discharge | - Introduction of foreign bodies | - Valve malfunction/ damage - Pipe blockage - Chamber integrity - Chamber occupants | M(C4) | - Avoid misuse of toilet - Follow correct procedures | L(A4) |
| Toilet hygiene | - Unsatisfactory hygiene standards | - Infection/cross infection - Bacteriological build up - Chamber occupants | M(C4) | - Use recommended disinfectant in toilet bowl after use - Wash hands after using toilet - Follow correct operating procedures | L(A4) |
| Valve operation external | - Pressure release | - Loss of depth - Decompression sickness | M(C4) | - Valve identification - Follow correct procedures | L(A4) |
| Bilge drain operation | - Pressure release - Introduction of foreign bodies - Unsatisfactory hygiene standards | - Loss of depth - Decompression sickness - Valve malfunction/ damage - Pipe blockage - Chamber integrity - Infection/cross infection - Bacteriological build up - Chamber occupants | M(C4) | - Valve identification - Avoid misuse of bilge drain - Use recommended disinfectant in bilge drain after use - Wash hands after operating bilge drain - Follow correct operating procedures - Note: Hygiene in the toilet area must be of a high standard. | L(A4) |
| Bilge cleaning | - Contact with chemicals | - Possible reaction to chemicals | M(C4) | - Use of gloves - Avoid skin contact - Follow correct cleaning procedures | L(A4) |

| ACTIVITY : Chamber Handover Procedure | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 51 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Shift handover | - Lack of information | - Incorrect gas mix/volume - Pressure/omitted decompression - Delay in operations - Medical treatment missed - Injury - Decompression sickness - Operator, chamber occupants and people in vicinity | H(C5) | - Follow safe working practices and correct handover procedures - Maintain up to date shift log - Stagger shift times of LST personnel | L(A4) |
|  | - Forget information | - As above | H(C5) | - Maintain up to date shift log - Stagger shift times of LST personnel | L(A4) |

Note:	Adequate time for shift handover must be allowed for.

| ACTIVITY : Shutdown Procedures Saturation Control | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 52 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Shut down LSS units | - Incorrect procedure - High pressure gas - Machinery | - Used soda sorb left in place - Water in units - Machinery left running - Operator and people in vicinity | M(C4) | - Follow correct shut down procedures | L(A3) |
| Shut down gas distribution panel | - Incorrect procedure - Venting gas | - High pressure gas in gas lines - Noise - Injury - Operator and people in vicinity | M(C4) | - Follow correct shut down procedures | M(C2) |
| Securing of chambers and doors | - Incorrect procedure - Swinging doors - Moving around in chambers - Rough weather | - Chamber left pressurised - Chamber atmosphere low in O2 - Mechanical damage (doors and hatches) - Dirty chambers - Injury - Operator and people in vicinity - Future occupants | M(C4) | - Follow correct shut down procedures | L(A3) |
| Electrical shut down | - Incorrect procedure - Electricity | - Damage to equipment - Operator | M(C2) | - Follow correct shut down procedures | L(A3) |
| Medical kit inventory | - Unqualified auditor | - Out of date supplies - Misappropriation of supplies - Inadequate supplies - Operator and people in vicinity | M(C2) | - Follow correct procedures - Only trained personnel to carry out inventories | L(A3) |
| Control panel shut down | - Incorrect procedures | - Pressure in gas lines and service lines | M(C2) | - Follow correct shut down procedure | L(A3) |

Note:	Chambers must be cleared of perishables when shutting down a saturation system.

| ACTIVITY : Chamber System Emergency Procedures | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 53 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| External chamber fire containment | - Fire/smoke | - Injury - Smoke inhalation - Mechanical and material damage - Explosion - Chamber occupants and all other crew members | H(D5) | - On site training/ evacuation routines - Maintenance of all safety equipment - Maintain and install a high level of fire prevention awareness | M(E1) |
| Internal chamber fire containment | - Fire/smoke | - Injury - Smoke inhalation - Mechanical and material damage - Explosion - Chamber occupants and all other crew members | H(D5) | - On site training/ evacuation routines - Maintenance of all safety equipment - Maintain and install a high level of fire prevention awareness - Restricted use of combustible materials in chambers – Fire retardent bedding - Hyperbaric fire extinguishers - DDC Deluge system installed - BIBS gas available - Low voltage in DDCs | L(A4) |
| Loss of pressure in chamber | - Pressure release | - Decompression sickness - Personal injury - Mechanical and material damage - Chamber occupants | H(D5) | - Routine maintenance - Correct working practices - Regular Emergency Drills - Automatic shut off valves – Flow fuses - Medical contingency arrangements - Possibility to evacuate to adjacent chamber | L(A4) |
| Service lock failure | - Pressure release | - Decompression sickness - Personal injury - Mechanical and material damage - Chamber occupants | H(D5) | - Routine maintenance - Correct working practices | M(C2) |
| Vessel collision | - Vessel sinking | - Drowning - Divers | H(D5) | - Routine HRL - Evacuation drills | M(C2) |
| Bacterial Infection | - Illness | - Personnel - Project Delay | H(D5) | - Strict hygiene regime - Regular swabbing and bacteriological analysis - Pre Sat Medical - Use of preventative drops – Treatment antibiotics | M (C2) |
| Chamber Atmosphere Contamination | - Illness - Injury – possibly fatal | - Personnel - Project Delay | M (C4) | - Strict control of items in DDC by LSS - Evaluation of any extraordinary materials used in complex - Analysers and Draeger tubes for detection | L (A4) |
| Anoxic/Hypoxic/Hyperoxic Gas Mix | - Illness - Injury – possibly fatal | - Personnel - Project Delay | H (D5) | - Gas analysed on receipt and prior to going on-line - Pre mix gases used - Minimum of 2% O2 in Heliox mixes- No pure Helium - Analysers calibrated every 12 hours - Pre mix air or nitrox mixes for surface supply diving | M (C2) |
| Loss of communication with chambers | - Lack of communications | - Misunderstanding - Mechanical and material damage - Pressure release - Injury - Fire - Chamber occupants and persons in vicinity | M(C4) | - Use recognised alternative method of communication | L(A4) |

Note:	Personnel must know what is required of them in an emergency.

| ACTIVITY : Hyperbaric Evacuation Phase I | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 54 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Chamber evacuation | - Lack of communication | - Misunderstanding - Misdirection - Catastrophe - Chamber occupants | H(D5) | - Good communication from relevant persons - Follow correct evacuation procedures | M(C2) |
| Transfer under pressure | - Depth changes | - Decompression sickness - Chamber occupants | H(D4) | - Adhere to safe working practices and parameters | M(C2) |
|  | - Environmental changes | - Extremes of temperatures - Chamber occupants | H(D4) | - Adhere to safe working practices and parameters | M(C2) |
|  | - O2 parameter changes | - Asphyxia - O2 toxicity - Chamber occupants | H(D5) | - Adhere to safe working practices and parameters | M(C2) |
|  | - Door/hatch opening/closing | - Personal injury - Chamber occupants | H(D4) | - Adhere to safe working practices | M(C2) |
|  | - Not being aware of divers recent activities | - Decompression sickness - Chamber occupants | H(D4) | - Adequate communication and record keeping | M(C2) |
| HRL launch | - HRL not in a state of readiness | - Inability to transfer/launch/start/occupy | H(D5) | - Carry out routine maintenance and check list schedules | M(C4) |
| Recovery of HRL | - Unfavourable conditions | - Inability to recover HRL - Mechanical/ material damage - Injury - HRL occupants and recovery team | H(D5) | - Personnel training and experience - Note: It is the duty of all personnel to know what is expected of them in an emergency. | M(C4) |

| ACTIVITY : Hyperbaric Evacuation Phase II | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 55 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| HRL crew general instruction | - Inexperienced crew | - Mechanical and material damage - Injury - Failure to effect successful hyperbaric evacuation - HRL occupants and crew | H(D5) | - Training for personnel responsible for HRL evacuation - Regular drills | M(C4) |
| Chamber occupants | - Leaving chamber complex | - Seasickness - Injury - Decompression sickness - Psychological damage - Chamber occupants | H(D5) | - Follow correct evacuation procedures | M(C4) |

Note:	It is the duty of all personnel to know what is expected of them in an emergency.

| ACTIVITY : HRL Checklists | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 56 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Pre saturation HRL chamber/ cockpit checklists | - Low oxygen in HRL chamber/cockpit - Obstructions in and around HRL - Procedures not followed correctly - Working with HP gas and oxygen | - HRL not ready to occupy - Injury - Mechanical/ electrical damage - Operator and people in vicinity - Pressure release - Explosion - fire - Mechanical and material damage | H(D4) | - Analyse oxygen content In HRL chamber/cockpit - General awareness - Follow correct checklists and ensure all areas are kept clean and free from grease/oil | L(A4) |
| HRL batteries on charge | - Explosive gases | - Explosion - fire - Mechanical and material damage - Injury - HRL occupants and persons in vicinity | H(D4) | - Adequate ventilation and warning notices - Follow correct checklists and procedures - No smoking | L(A4) |

Note:	Checklists and maintenance to be performed by competent personnel only.

| ACTIVITY : Dive System Fire Fighting Equipment | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 57 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Fire fighting equipment | - Inappropriate/ inadequate equipment | - Inability to extinguish fire - Injury - Mechanical/ material damage - All crew | H(D5) | - Correct maintenance and operating procedures - Safe working practices | M(E1) |

| ACTIVITY : Life Support Package (LSP) Procedures | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 58 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Connecting service umbilical | - Incompatible connection | - Inability to connect LSP. to HRL | H(D5) | - Ensure pre-mobilisation checks are carried out - Familiarisation of operating procedures and equipment | M(E1) |
|  | - Unserviceability of equipment | - Explosion - fire - Injury - Inability to use equipment | H(D5) | - Ensure pre-mobilisation checks are carried out - Familiarisation of operating procedures and equipment | M(E1) |
| Supplying services | - Unprotected umbilical (LSP) | - Decompression sickness - Pressure release - Explosion - fire - Mechanical and material damage - HRL occupants and people in vicinity | H(D5) | - Ensure pre-mobilisation checks are carried out - Familiarisation of operating procedures and equipment | M(E1) |
| Operation of LSP. | - Limited resources | - Fatigue - Depletion of supplies and gas stocks - HRL occupants | H(D5) | - Ensure pre-mobilisation checks are carried out - Familiarisation of operating procedures and equipment - Periodic checks by LSS at LSP location | M(E1) |

| ACTIVITY : Dry Docking Vessel with Personnel in Saturation | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 59 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Entering the dry dock | - Collision | - Damage to vessel. - Flooding at chamber level. | M(C4) | - Fendering. - Cross-winds. - Weather forecast. - Adequate crew on deck to man ropes. | L(A4) |
| Setting down on blocks | - Distortion of chamber complex | - Loss of pressure. - Leakage. | H(C5) | - During docking process all internal doors to remain closed - Check stability of vessel | M(C2) |
| Changing from ships services to shore services | - Continuity of services | - Loss of environmental control | L(A5) | - Situation to be stressed at pre docking meeting. | L(A2) |
| Day to day problems | - Additional fire hazard | - Hot work round gas quads / risk of fire | H(D5) | - Permit to work system enforced. - Restrictions on hot work permit on adjacent compartments to living area and around O2 storage. - Permanent fire watch | M(C2) |
| Day to day problems | - Loss of services | - As above (continuity of services). - Loss of water |  | - Clarification of primary power to be used (safe, reliable power supply) with ship’s generator as secondary supply (back-up). - Two independent water supplies and discussions to be held with local fire brigade |  |
| Hyperbaric Evacuation | - Inability to deploy HRL | - Divers exposed to uncontrolled dangers. - Fire. - Explosion. | H(D5) | - Pre rigging of fire hoses for boundary cooling of HRL. - Designated crane with designated transportation trolley. - Shoreside LSP with gas supplies (check compatibility of LSP with system). - Pre rig boat for crane deployment. - Adequate Life Support personnel for evacuation | M(C2) |
| Additional dock yard works | - Falling loads. - Damage to equipment. | - Loss of pressure to HRL. - O2 explosion / fire. | M(C4) | - Restrict overhead movement of loads above HRL and gas quads. - Ensure all lifting rigging is certified. | L(A4) |
| Security | - Sabotage. | - Danger to personnel in chamber - Loss of equipment | M(C4) | - Gangway watch / position camera on gangway | L(A4) |
| Power requirements for diving system in addition to vessel requirements 150 kW | - Inability to supply sufficient power to maintain life support through shore power connections | - Unable to maintain life support to divers in saturation. | H(E5) | - Upgrade shore supply connection box to accept 630 amp supply. - Ensure immediate availability and serviceability of onboard emergency generator. - Shoreside power requirements would be additional and above normal dockyard supplies require rental of portable generator 60Hz, 500KW. | M(C2) |
| Unfamiliar activity of decompression of divers whilst in dockyard hands. | - Unintentional hazardous activities by dockyard. | - Interruption with the LSP and / or danger of fire in life support machinery compartments. | H(D4) | - Dockyard personnel to cover Risk Assessment of this activity during their pre works induction meeting. | M(C2) |

| ACTIVITY : Working with High Pressure Gases | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 60 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Working with high pressure gasses | - Leaking - Explosion - Fire | - Cold/hot burns - Breathing problems - Eye injury | H(D5) | - Good ventilation - Clean working area - Checking for leakage - No open fire | L(A4) |
| Lifting helium gas quads | - Lifting strops - Caught round pillar valve | - Impact injury - Rapid gas discharge | H(C5) | - Check lay of strops before lifting | L(A3) |
| Lifting O2 gas quads | - Lifting strops - Caught round pillar valve | - Impact injury - Rapid gas discharge - Explosion | H(C5) | - Check lay of strops before lifting | L(A3) |
| Lifting gas quads with incorrect slings | - Slings parting | - Falling load | H(C5) | - Never remove slings from quads | L(A3) |
| Lifting quads | - Damage to manifold pipework | - Render quad unusable - Release of gas | M(C4) | - Take care when lifting - Use tag lines | L(A3) |

Note:	Be mindful of storing gas quads near fire hazards high activity areas.

| ACTIVITY : Gas Management Procedures | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 61 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  |  |  | Project Specific |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. APPROPRIATE PPE |  | 5. PERMIT TO WORK |  | 9. SUPERVISION |  |  | 13. |  |  |
| 2. TASK SPECIFIC PROCEDURE |  | 6. MECHANICAL ISOLATION |  | 10. CERTIFICATION |  |  | 14. |  |  |
| 3. SPECIFIC TRAINING |  | 7. ELECTRICAL ISOLATION |  | 11. TOOL BOX TALKS |  |  | 15. |  |  |
| 4. RESTRICT WORK AREA ACCESS |  | 8. POST BARRIERS / SIGNS |  | 12. COMPETENT PERSON |  |  | 16. |  |  |
| Step 1 Task/Activity Description | Step 2 Hazard |  | Step 3 Who or What Exposed |  | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures |  | Step 6 Residual Risk |  |
| Working with high pressure gas storage | - Massive gas release - Massive gas release (high O2 content) - Massive gas release (low O2 content) |  | - Over pressurisation of an area causing structural damage - Explosion - fire - Asphyxia - Operator and people in vicinity |  | H(C5) | - Storage area suitable for use - Adherence to routine maintenance schedules of all safety analytical equipment and machinery - Safe working practices |  | L(A4) |  |
| Gas transferring | - Unsuitable whips and fitting - Mis-routing of gas mixes - Damage to unprotected whips, pipes, flasks - Lack of maintenance and cleanliness of equipment - Personnel unaware of gas transferring operations - Unauthorised operations - Unauthorised operator/ personnel in area of gas transferring - Inadequate lighting - Noise |  | - Injury - Damage - Explosion - fire - Asphyxia - Contamination - Operator and people in vicinity |  | H(C5) | - Correct usage of materials and equipment - Identification and awareness of gas routes - Local area warning signs - Protection of vulnerable gas lines and equipment - Always secure gas whips using wire tie backs - Adherence to recommended maintenance schedules - Restricted access - Inform relevant personnel - Familiarisation and training - Adequate illumination - Wear ear/eye protection - Note: Competent personnel only to use gas transferring equipment. |  | L(A4) |  |

| ACTIVITY : Gas Storage, Banks and Distribution | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 62 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed |  | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures |  | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Opening high pressure gas flasks | - High pressure gas loss | - Injury - Mechanical damage - Operator and people in vicinity |  | H(D5) | - Visual and mechanical inspection of valves and fittings |  | L(A4) |
| Distribution of gas to required destination | - High pressure gas in transit | - Injury - Mechanical damage - Operator and people in vicinity |  | H(D5) | - Vigilance/awareness |  | L(A4) |
|  | - Incorrect routing | - Injury/death - Explosion - fire - Operator and people in vicinity |  | H(D5) | - Correct labelling and valve positioning |  | L(A4) |
| Below decks gas storage | - Over pressurisation of whips, pipes, flasks and gas bag | - Injury/death - Explosion - fire - Operator and people in vicinity |  | H(D4) | - Correct relief valve and sensor setting on compressor and gas bag |  | L(A4) |
|  | - Inadequate lighting - Poor access for leak detection on pipe work runs | - Injury - Incorrect mechanical operations - Gas loss - Operator and people in vicinity |  | M(C4) | - Adequate illumination - Ensure all pipe work fittings are accessible |  | L(A4) |
|  | - Noise | - Injury - Operator and people in vicinity | H(D4) |  |  | - Wear ear/eye protection | L(A4) |

Note:	Gas distribution panels, whips and hard plumbing should be clearly marked to avoid incorrect routing.

| ACTIVITY : Transferring Oxygen | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 63 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Storage of high pressure O2 flasks | - Damage to unprotected whips, pipes and flasks | - Injury/death - Explosion - fire - Operator and people in vicinity | H(D5) | - Local area warning signs - Designated open deck storage | M(C2) |
| High pressure O2 below decks | - Unregulated O2 | - As above | H(D5) | - Regulated O2 from deck quads | M(C2) |
| Charging O2 flasks | - Working with high pressure gas | - As above | H(D5) | - Familiarisation of equipment and safe working practices | M(C2) |
|  | - Unsuitable whips, fittings and compressor | - As above | H(D5) | - Correct usage of Haskel pump, materials and equipment - All to be O2 cleaned | M(C2) |
|  | - Personal unaware of O2 transferring operations | - Injury - Damage to equipment - Operator and people in vicinity | H(D5) | - Local area warning signs - Designated open deck storage | M(C2) |

Note:	Transferring O2 is an extremely hazardous operation and should only be performed by competent personnel.

| ACTIVITY : Transfer of Gas Using Compressors | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 64 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk |  | Step 5 Recommended Corrective Actions / Control Measures |  | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Gas transferring | - Unserviceable compressor - Poor maintenance - Blocked filters | - Injury - Explosion - fire - Operator and people in vicinity |  | H(D5) |  | - Confirm with dive tech. compressor is ready for operation - Follow correct maintenance procedures | M(E1) |
|  | - Unserviceable or inappropriate whips and fittings | - Injury - Explosion - fire - Operator and people in vicinity - Loss of gas - Mechanical damage |  | H(D5) |  | - Ensure whips and fittings are compatible and properly maintained - Use wire tie backs to secure gas whips | M(E1) |
|  | - Incorrect operating procedure | - Injury - Explosion - fire - Operator and people in vicinity |  | H(D5) |  | - Familiarisation and training - All tubes/valves to be correctly marked | M(E1) |
|  | - Pressure release | - Injury - Explosion - fire - Operator and people in vicinity |  | H(D5) |  | - Follow recommended operating and maintenance procedure | M(E1) |
| Gas transferring (cont) | - Moving mechanical parts | - Injury - Damage - Operator and people in vicinity |  | H(D5) |  | - Ensure all safety guards are in place | M(E1) |
|  | - Noise | - Injury - Operator and people in vicinity |  | H(D4) |  | - Wear ear/eye protection | L(A4) |
|  | - Incorrect routing of gas - Too high percentage of oxygen supplied to W & J compressor | - End user supplied with wrong gas or pressure - Injury - Explosion - fire - Mechanical damage - End user and people in vicinity |  | H(D5) |  | - Adhere to correct operating procedures - Familiarisation and training | M(E1) |
|  | - Relief valve/pressure control cut off malfunction | - Over pressure - Explosion - fire - Mechanical damage - Injury - Operator and people in vicinity |  | H(D5) |  | - Adhere to maintenance schedules | M(E1) |

Note:	Competent personnel only to start and stop compressors. If in doubt ask!

| ACTIVITY : Helium Recovery System | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 65 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Gas reclaiming | - Automatic compressor starting up without warning | - Injury - Damage - Operator and people in vicinity | H(C5) | - Display local warning signs - Restrict access to competent personnel | L(A4) |
|  | - Pressure release | - Injury - Explosion - fire - Operator and people in vicinity | H(D5) | - Follow recommended operating and maintenance procedures | L(A4) |
|  | - Possible asphyxia | - Injury/death - Operator and people in vicinity | H(D5) | - Restrict access - Warning alarms - O2 sensors | L(A4) |
|  | - Noise | - Injury - Operator and people in vicinity | H(D4) | - Wear ear protection | L(A4) |
|  | - Over pressurisation of gas bag | - Injury - Material damage - Asphyxia - Operator and people in vicinity | H(D4) | - Maintenance of relief valves and limit switches - Remote observation | L(A4) |
|  | - Inadequate lighting | - Injury - Incorrect mechanical operation - Operator and people in vicinity | H(D)4 | - Adequate lighting - Note:When using helium recovery system always ensure O2 valve is closed after use. | L(A4) |

| ACTIVITY : Gas Cylinder Charging | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 66 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Gas decanting | - Pressure release - Incorrect routing of gas | - Injury - Gas loss/wrong mix - Operator and people in vicinity | H(D5) | - Follow charging procedures and safe working practices | L(A4) |
|  | - Opening valves too quickly | - Injury - Overheating - Explosion - Operator and people in vicinity | H(D4) | - Follow charging procedures and safe working practices - Charge bailout cylinders in water if possible | L(A4) |
| Gas pumping | - Pressure release - All hazards associated with compressors | - Injury - Gas loss - Operator and people in vicinity | H(D4) | - Follow charging procedures and safe working practices | L(A4) |
|  | - Over pressure | - Injury - Explosion - Operator and people in vicinity | H(D4) | - Follow charging procedures and safe working practices - Maintenance schedules | L(A4) |
|  | - Incorrect routing of gas | - End user supplied with wrong gas - Injury - Mechanical damage - End user and people in vicinity | H(D5) | - Follow charging procedures and safe working practices | L(A4) |
|  | - Wrong mix supplied | - Injury - Decompression sickness - Explosion - fire - Mechanical damage - End user and people in vicinity | H(D5) | - Follow charging procedures and safe working practices | L(A4) |
|  | - Unauthorised access - Hazardous materials | - Injury - Explosion - fire | H(D5) | - Follow charging procedures and safe working practices - Restrict access | L(A4) |
|  | - Incorrect storage of cylinders | - Mechanical/ material damage | H(D5) | - Follow charging procedures and safe working practices - Local warning signs | L(A4) |

Note:	Always use safety whips to tie hose ends on in case of blow-out.

| ACTIVITY : Pressure test of the Bell | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 67 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Pressure test of bell | - High pressure gas explosion | - Personal injury | H(C5) | - Area cordoned off - Signs and tannoys - Electrical isolation | L(A3) |
|  | - Elevated partial pressure of oxygen | - Fire risk - Explosive mixture | H(D5) | - Maintain oxygen levels below that which sustains fire | L(A3) |
|  | - Excessive pressure | - System failure - Component failure | M(C4) | - Set regulators and system relief valves to maximum setting - 2 men minimum in dive control during test | L(A2) |
|  | - Excessive temperature | - Component failure | M(C2) | - Pressurise in a controlled manner to maintain temperature | L(A2) |
|  | - Reduced partial pressure of oxygen on venting | - Anoxia | H(C5) | - Vent bell with air on surfacing and analyse before entering | L(A2) |
|  | - Lowered temperature | - Freeing of valves and other component parts | M(C2) | - Pressurise and vent in a controlled manner | L(A2) |

| ACTIVITY : ROV Operations | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 68 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Power up 440 V AC electronic power on vehicle | - Fire - Earth leakage | - Damage to equipment - Personnel injury - Lost time | H(C5) | - Follow start up procedure - Vehicle earth strap to ship’s deck - Perform regular maintenance - Ensure good communications - 1, 2, 3, 4, 5, (6, 7), 10, 12 | L(A3) |
| Start up hydraulic power unit on vehicle | - Fire - Oil leak - Burst hoses - Pollution - Earth leakage | - Damage to equipment - Personnel injury - Lost time | M(C4) | - Follow start up procedure - Vehicle earth strap to ship’s deck - Perform regular maintenance - Ensure good communications - Wear correct PPE - 1, 2, 3, 4, 5, 10, 11, 12 | L(A4) |
| Equipment test | - Moving manipulators - Moving/ running thrusters - Oil leak - Burst hoses - Pollution - Movement of additional equipment | - Damage to equipment - Personnel injury - Lost time | M(C4) | - Follow start up procedure - Perform regular maintenance - Ensure good communications - Wear correct PPE - 1, 2, 3, 4, 5, 10, 11, 12 | L(A3) |

Note:	Personnel assisting in ROV deployment recovery should be made aware of any dangers that may occur

| ACTIVITY : ROV Umbilical Winches | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 69 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Starting electric hydraulic power pack | - Burst hoses - Fire - Oil leak - Pollution | - Damage to equipment - Lost time | M(C4) | - Perform regular maintenance and visual inspection - 1, 2, 3, 5, 10, 12 | L(A4) |
| Operating control valves | - Burst hoses - Oil leak - Pollution | - Destruction of moving parts - Damage on umbilical - Personnel injury | M(C2) | - Ensure good communication - Ensure all valves/handles are clearly marked/tagged - Ensure that immediate area is free for personnel not involved in launch/recovery operation - Ensure that yellow signs “Danger Remote Operated Winch” is present - 1, 2, 3, 5, 10, 12 | L(A3) |

Note:	Competent personnel only to operate umbilical winches.

| ACTIVITY : ROV Cranes | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 70 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Running up deployment system ready for operations | - Moving cylinders - Falling load - Burst hoses - Swinging boom | - Trapped limbs - Damage to ROV - Personnel injury - Lost time | H(D4) | - Follow start up and launch and recovery procedure - Ensure good line of sight - Wear correct PPE - Ensure good communications - Perform visual inspection and regular maintenance - 1, 2, 3, 5, 10, 12 | M(C2) |

Note:	Competent personnel only to operate ROV cranes.

| ACTIVITY : Simultaneous ROV/Diver Operations | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 71 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| ROV/diver working in close proximity | Electric shock to diver ROV hits diver ROV umbilical gets tangled in divers umbilical ROV hits bell ROV looses power ROV thrusters | Death/injury to diver Crush injury to diver Diver gets dragged off job/to surface Possible injury to bellman divers Entanglement/ knocks diver Damage to Divers umbilical Trapped fingers Diver entrapment | M(C2) | Test of earth circuit leakage breaker every 24 hrs. min. ROV maintains a safe distance from diver ROV maintains safe distance Good tether management ROV is equipped with sonar etc. in poor visibility ROV keeps down stream of diver in tides Make sure suitable guards are fitted over all thrusters Dive Supv has monitor with ROV pilot vision Open comms Dive / ROV control - 2, 3, 9, 10, 11, 12 | L(A3) |

Note:	ROV must adhere to Company safe operations procedure. 
ROV pilot must inform Dive Supervisor of all ROV movements on the seabed.

| ACTIVITY : Cutting of ROV Umbilical | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 72 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Cut ROV umbilical to allow transport of ROV & winch separately | Electric shock from umbilical or power supply | ROV Personnel Vessel Personnel | H4 | Isolate power supplies Lock power supplies Raise permit to work Isolation certificate Trained/competent personnel PPE Correct tools – 110V Prove supplies are dead Toolbox talk Place welders blanket beneath area where cut is occurring - 1, 2, 3, 4, 5, 7, 9, 11, 12 | L2 |

| ACTIVITY : Towfish Operations | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 73 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. LIFEJACKET |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Deploy Towfish using the stern A-frame and towcable winch Simultaneously | - Swinging Loads - Dropped objects - Overside work | - Personnel injury - Personnel falling overboard - Damage to Towfish | C2=M | - Secure the area - Use tag lines - Use Life vests - Consider Vessel movements - Consider sea and wind state - Experienced personnel - 1, 2, 3, 4, 5, 8, 9, 11, 12, 13 | C1=L |
| A-Frame deployed to outer position | - Moving parts | - Trapped limbs | C2=M | - Secure the area - Experienced personnel - 1, 3, 4, 5, 9, 12 | C1=L |
| Remote operation of towcable winch | - Moving machinery - Towcable under tension | - Moving machinery - Towcable under tension | C2=M | - Secure area by red/white plastic chain - Ensure that sign “Danger Remote” - 1, 3, 4, 5, 9, 12 | B2=L |

Note:	Towfish operator must adhere to Company safe operations procedure.

| ACTIVITY : Free Swimming. SCV. Launch/Recovery/Operations | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 74 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Launching Decent Recovery | - Swing vehicle - Vehicle overheat - Drop vehicle - Sea tidal condition - Thrusterwash - Launcher breakdown - Winch breakdown - Blackout - DP event | - Impact on vessel side - Impact with Thruster - Damaged vehicle - Lost vehicle - Fouled Thruster - Delay to Project | E3=H | - Good Tether Management - Familiarisation - Toolbox talks. - Dedicated - Continuous comm’s with bridge and survey - Surface lighting - Deselect closet Thruster during launch and recovery. - A-Frame wire to follow - 1, 2, 3, 4, 5, 9, 11, 12 | D2=M |
| Launching Decent | - Thruster wash - Dead vehicle - Foul Main Lift Tether - Beacon failure - Loss of power | - Impact on vessel side - Impact with Thruster - Damaged vehicle - Lost vehicle - Fouled Thruster - Delay to Project | E3=H | - Vehicle Launch - Pilots to monitor vehicle - Position continuously - 1, 2, 3, 4, 5, 9, 11, 12 | D2=M |
| Seabed Operations | - Vessel manoeuvring - Fouled Main Lift Thruster - Surface current - Fouling with Fast Winch Wire - Dead Vehicle - Float up - Tether drag - Turns on main lift cable - Beacon failure | - Impact on vessel side - Impact with Thruster - Damaged vehicle - Lost vehicle - Fouled Thruster - Delay to Project | E3=H | - Continuous comm’s with bridge, survey and winch. - Good umbilical management - Good lighting during dark hours - Recovery procedure to be posted on DP Desk - 2 beacons to be carried - Responder to be function test - A-Frame wire to follow vehicle - Vehicle to offset during launch - 1, 2, 3, 4, 5, 9, 11, 12 | D2=M |
| Transit | - Water column tidal current - Fouling on seabed / structure - Tether falling on ship Thruster - Speed of vehicle / ship - Communication failure - Dead vehicle | - Impact on vessel side - Impact with Thruster - Damaged vehicle - Lost vehicle - Fouled Thruster - Delay to Project | E3=H | - DP desk to select follow sub - Survey to display target & route - Continuous monitoring of position - Good Umbilical Management - Recovery procedure to be posted on DP desk. - 1, 2, 3, 4, 5, 9, 11, 12 | D2=M |
| Dead Vehicle | - Fouled ship Thruster - Float up - Lost vehicle - Loss of Nav | - Impact on vessel side - Impact with Thruster - Damaged vehicle - Lost vehicle - Fouled Thruster - Delay to Project | E3=H | - Ship to select port. Move at 0.1 ms - Continuous position monitoring - Continuous comm’s with bridge & survey - Good Tether Management - Good lighting during dark hours - Offman to be called / Captain - DP operators to check sea room for manoeuvring. - DP Operators to select 0.1 ms away from vehicle. (Not over tether). - Monitor vehicle position continuously - Continuous status reporting to Bridge / ROV and from Survey - 1, 2, 3, 4, 5, 9, 11, 12 | D2=M |
| DP event | - Fouled ship Thruster - Float up - Lost vehicle - Loss of Nav | - Impact on vessel side - Impact with Thruster - Damaged vehicle - Lost vehicle - Fouled Thruster - Delay to Project | E3=H | - Vehicle to keep lift cable under tension - Drive away from ship - Continuous Umbilical monitoring - Good Tether Management - Continuous position monitoring - Vehicle to sit down on seabed. - 1, 2, 3, 4, 5, 9, 11, 12 | D2=M |

| ACTIVITY : Transferring fuel to Include Bunkering and Deck Equipment | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 75 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Transfer fuel/bunkering in port | - Pollution - Leaking | - Environment - Eye injury - Injury from falling - Fire | E x 4 = H | - Watchkeeping - Stop welding and burning whilst bunkering - Scuppers plugged - SOPEP material available - 1, 2, 4, 5, 12 | L(A4) |
| Transfer fuel/bunkering at sea | - Collision of ships - Connecting fuel hose - Parting / failure of hose | - Fall overboard - Structural damage - Back strain - Grated fingers - Hose / ropes in thrusters - Environmental pollution | E x 5 = H | - Good lookouts - Use of fenders - Correct lifting techniques - Mooring arrangement if possible - Scuppers plugged - SOPEP material available - 1, 2, 4, 5, 12 | M(C4) |
| Transferring fuel on deck from engine room to deck equipment | - Deck spillage - Damaged transfer hoses and fittings - Slippery surfaces - Flammable liquid | - Environmental pollution - Fire / explosion - Slips, trips, falls - Skin irritation - Eye injury | B x 3 = M | - Deck crew to follow Engineers instructions. - Ensure hot work controlling measures are in place - Maintain watch during fuel transfer operations - Scuppers plugged - SOPEP material available - 1, 2, 4, 5, 12 | L(A4) |

Note:	A general PA should be made to advise people that bunkering is intended to take place.          All supervisors/personnel in charge to be informed directly.

Hot work permits to be suspended.

| ACTIVITY : Mooring | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 76 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Arrival / departure | - Caught in bights - Hit by slipping lines - Hit by parting lines - Tripping over lines / material | - Leg injury - Body/head injury | D x 4 = H | - Keep rope coils tidy - Watch moving lines - Watch load on the lines - Stay in correct positions | M(C4) |
|  | - Lifting - Handling mooring ropes | - Back strain | D x 4 = H | - Correct stance/lifting techniques - Sufficient deck lights - 1, 11, 12 | M(C4) |
| Working capstan | - Trapped fingers - Riding turn, rope jams on capstan - Rope jumps off or parts |  | D x 4 = H | - Correct hold on ropes - Keep safe distance from capstan - Feed rope properly onto capstan - Work area tidy and accessible - Mooring lines fit for purpose - 1, 3, 12 | M(C4) |

Note:	Personnel other than trained seamen must work under guidance when mooring operations are ongoing.

| ACTIVITY : Anchoring, Hanging off Anchor and Running Anchor Wires | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 77 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Anchoring operations | - Hit underwater structures - Flying rust and dirt particles - Crew member gets caught between chain and winch - Chain falls on AB - Inadvertent letting go of anchor | - Damage to structures - Eye injury - Time wasting - Injury to crew - Injury to AB | D x 4 = H | - Use safety glasses - Confirm anchor position - Do not let anchor go until Master has given order to do so - Keep clear of operating winches - Visual check of chain locker before operations - Minimum 2 persons required for hoisting anchor - Sufficient lights on deck and in chain locker | M(C4) |
| Hanging off anchor | - Fall overboard - Falling objects - Rigging failure | - Drowning - Loss of equipment - Impact - Personnel injuries | E2 | - PPE – Safety Harness, life jackets - Raise permit to work - Clear non essential personnel from location - Establish good communication | B1 |
| Running anchor wires | - Fall overboard from workboat - Falling objects - Wire and ropes in tension | - Drowning - Loss of equipment - Personnel injuries | E2 | - PPE – lifejackets - Establish good communication - Ensure correct positioning of wires/ropes | B1 |

Note:	Competent personnel only to be involved in anchor handling.

Anchors to be raised/lowered on instruction of Master only.

| ACTIVITY : Ballasting | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 78 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Ballasting load/ discharge | - Loss of stability - flooding | - Unstable ship - Damage to equipment | C2=M | - Consult responsible stability officer before starting ballast operation - Check manhole records | B1=L |
| Ballasting to port/stb. | - Ballast pumps malfunction - Equipment breaks loose | - Unstable ship delays | C2=M | - Regular maintenance function tests - Crew to be aware of ballasting | A1=L |

Note:	Ballasting operations to be performed on instruction from Master only.

| ACTIVITY : Transit Navigation | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 79 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Navigation | - Human error - Stress / fatigue - Heavy weather - Blackout - Loss of night vision | - Damage to vessel - Collision - Grounding | E x 5 = H | - Follow Safe Watching Procedures - Qualified personnel as per STCW95 | M(C4) |

Note:	Only essential marine crew to be on navigation bridge when vessel is in transit.

| ACTIVITY : Dynamic Positioning | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 80 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Dynamic positioning | - Operator error - Stress / fatigue - Black out | - Damage to equipment being installed - Damage to vessel - Damage to offshore installation - Injury to diver - Damage to subsea installation - Environmental pollution | E x 5 = H | - Good pre planning - Follow DP guideline - Qualified and experienced personnel | L(A4) |

Note:	DP status lights must be reported if malfunctioning.

| ACTIVITY : Transponder/Tautwire | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 81 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Deploying Recovering Repositioning equipment | - Material failure - Contact with diver(s) - Entanglement with other down line - Swinging weights - Contact with ROV - Entanglement with ROV | - Injury to diver(s) - Material damage - Damage to subsea structure, pipeline, etc - Damage to ROV | C x 4 = H | - Obtain confirmation from Bridge that transponder / lightweight taut wire are clear for surface - Good communications between DPO, Dive Control, Taut Wire/Transponder Operator, ROV Control - Bridge must inform Dive Control of all LTW movements - Bridge must inform ROV control of all LTW movements - Careful monitoring of transponder wire - Good pre-planning | L(A4) |

Note:	Recovery, deployment or repositioning of LTW must not take place until Dive Supervisor has been informed.

| ACTIVITY : Pilot Ladder | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 82 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Rigging ladder Use of ladder | - Fall from ladder - Improper rigging - Material failure - Poor lighting - Pilot gets trapped between boats | - Drowning / hypothermia - Slipping / tripping - Injury to pilot | E x 4 = H | - Pick correct heading and speed to achieve safe boarding - Ladder must be adequately illuminated - Deck personnel wear work vest - Comply with SOLAS, Chapter 5 – Reg. 17 - Ladder to be properly secured - Lifebuoy with line to be ready | L(A4) |

Note:	Pilot ladder will be overboarded at Master’s request only.

| ACTIVITY : Gangway | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 83 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Rigging Using gangway | - Fall overboard - Fall off - Gangway falls / wire parts - Trapping of fingers - Gangway falls off pier – vessel - Trapping of gangway - Rise and fall of tide - Movement due to cargo operations / passing traffic | - Drowning / hypothermia - Hit quay side - Personnel injury - Material damage | C x 3 = M | - Use safety harness - Rig safety net - Life buoy standby - Properly rigged - Use tag lines - Good communications to crane driver - Vessel must be moored correctly - Gangway must be of adequate length - Frequent monitoring of gangway - 1, 3, 12 | L(A3) |

Note:	Gangway should be under the direction of marine crew.

| ACTIVITY : Seafastening/Removal of Seafastening | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 84 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Seafastening activity | - Trapped limbs - Loose objects | - Limb injury - Damage to equipment | C x 2 = M | - Use correct equipment - Check before sailing - Frequently check at sea - Establish safe walkways in relevant areas | L(A2) |
|  | - As associated with welding, burning, etc. | - As associated with welding, burning, etc. | C x 2 = M | - As associated with welding, burning, etc - Comply with “Cargo Securing Manual” | L(A2) |
|  | - Lifting heavy rigging equipment | - Back injury - Muscular strain | C x 2 = M | - Use correct lifting techniques - Man handling procedures awareness | L(A2) |
|  | - Seafastening parts | - Injury to personnel - Damage to equipment | C x 2 = M | - Use rigging that is fit for purpose - Engineer welded seafastenings | L(A2) |

Note:	When seafastening take dynamic forces into account.

| ACTIVITY : Basket Transfer | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 85 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Transfer of personnel by basket using ship’s crane. | - Incorrect hooking up of basket - Falling down from basket - Swinging loads - Mechanical / hydraulic failure of crane - Material failure | - Injury - Drowning / hypothermia - Damage or lost equipment | E x 4 = H | - Safe lifting equipment - Certified Billy Pugh / basket / crane for man riding - Basket provided with taglines of adequate length - Wear life jacket and survival suit (cold water areas only) - Only if approved by Master - Unfamiliar personnel to be briefed as per Marine Operations / Vessel Operations. - Suitable weather conditions - Situation to be risk assessed | M(C4) |

| ACTIVITY : Bunkering at Sea | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 86 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Navigation at worksite. | - Grounding - Collision - Heavy weather - Fog - Instruments malfunction - Navigation lights malfunction - Total instrument failure - Normal lights on bridge whilst steaming - V.H.F. on wrong channel | - Damage to vessel - Personal injury - Off course - Late arrival - Difficult for other vessel to identify - Need to celestial navigate - Loose night vision - Miss mayday or pan signal - Delay to Project | E2=H | - Favourable conditions - Double check positions at regular intervals - Ensure who is in command - Keep sharp lookout and take action well before situation gets critical - Use “seaman’s eye” - Have adequate systems on line - Plan actions in good time - Use back up instruments - Check calibration of instruments - 1, 2, 3, 4, 5, 6, 7, 8, 9, 11, 12 | B2=L |
| Navigation at worksite (cont) | - Permission to enter zones - Mobile installations - Fix installations - Restricted manoeuvring - Drifting - In Field Rules |  | E2=H | - Regular check of lights - Nav. instruments on U.P.S. - Back up nav. Instruments - Red light status to be maintained at all times - V.H.F. checks to be done prior to departure | B2=L |
| Mooring ship alongside | - Pollution - Leaking - Pipe rupture - Explosion - Collision of ships - Connecting fuel hose - Prevailing condition - Offshore Installation | Both ships - All personnel - Delay to Project | H E-2 | - Fenders to be placed alongside. - Captains to evaluate conditions. - Approach of second ship to be in a slow controlled manner. - Only Captains to give final approval to proceed. - Marine checklist to be completed. - Briefing to be held prior. - Task plan to be produced | M E-1 |
|  |  | - See section 3.8 Ships Generic Risk Assessment |  | - 4, 12 - See section 3.8 Ships Generic Risk Assessment |  |
|  |  | - Ships - All Personnel - Delay to project | H E-2 | - Task Briefing / Toolbox Talk. - Comms. Check to be completed with all stations. - Single Point Control - 1, 4, 10, 12 | M D-2 |
|  |  | - Ships - All Personnel - Delay to project | H E-2 | - None essential personnel to remain clear. - MOB to be made ready. - All personnel to wear Life Jackets. - Single point control. - 1, 4, 10, 12 | M C-2 |
|  |  | - Ships equipment - Delay to Project | M C-2 | - Both ships to ensure lines in good condition. - Good seamanship. | L B-1 |
| Transfer fuel/bunkering | - Pollution - Leaking - Pipe rupture - Explosion - Collision of ships - Connecting fuel hose | - Environment - Eye injury - Injury from falling - Fire - Pipework - Fall overboard - Structural damage - Back strain - Grated fingers | E1=M | - Watch keeping - Close monitoring - Establish max pumping pressure - Stop welding and burning whilst bunkering - Display fuelling notices - Pollution equipment standing by - Good communications - No smoking - Good lookouts - Use of fenders - Correct lifting techniques - 1, 2, 3, 4, 5, 8, 9, 11, 12 | C1=L |

Note:	A general P.A. should be made to advise people that bunkering is intended to take place.

| ACTIVITY : Personnel transfer At Night Using Work Basket | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 87 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Deployment of Workboat | - Swing Load - Overhead Lifting - Drop Object - Crane breakdown | - Personal Injury - Damaged Equipment - Delay to project | C2 = M | - Captain to be on Bridge - Inspection prior deployment - 1, 3, 5, 9, 11, 12 | B2=L |
| Transfer of Personnel into boat | - Trip / Fall on deck. - Fall from ladder - Trip / Fall in boat - Lost equipment & personnel items. | - Personal Injury - Overside Work - Delay to Project | E2 = H | - Life Jackets to be worn - Pilot Ladder - Rescue Lines - Toolbox Talk - 1, 3, 5, 9, 11, 12 | C1=L |
| Operation of workboat at night | - Breakdown | - Adrift at night - Delay to project | B2=L | - Pre-use Checks. - Crewboat to cover operations of workboat for drift off. - Flash Lights to be carried in workboat - Crewboat to be notified - Dedicated Comm’s channel - Tool Box Talk - 1, 3, 5, 9, 11, 12 | A1=L |
| Transfer of personnel and luggage | - Falling overboard - Loosing equipment / luggage - Engine breakdown | - Injury - Drift off | D3+H | - Life Jackets - Trained personnel - MOB immediate readiness - Rescue Lines standby - 1, 3, 5, 9, 11, 12 | B2=L |
|  | - Weather conditions - Darkness | - Capsize / Swamp - Personal Injury - Adrift in darkness | D3=H | - Both Captains approval - Weather watch - Establish communication - Fix secondary light to life jackets - 1, 3, 5, 9, 11, 12 | B2=L |
|  |  | - Poor Lighting - Trip / Fall on deck - Trip / Fall in boat | D3=H | - Flash Light to be carried - Ships searchlight to be lit. - Good Deck Lighting - Crane to be available during transfer - 1, 3, 5, 9, 11, 12 | B2 |
| Simops | - Interference with transfer | - Delay - Slow recovery of personnel / boat | D3=H | - Crane to be available during transfer - 1, 3, 5, 9, 11, 12 | B2 |

| ACTIVITY : Launch Boat/Recovery of Work Boat | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 88 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Boat checklist | - Procedures were not correctly followed | - Boat not ready to occupy mechanical damage - Boat crew launch team material damage | C2=M | - General awareness - Follow correct checklist and ensure boat launch platform clear and free from grease and oil - Use long bow and stern ropes attached to vessel - 1, 3, 5, 9, 11, 12 | B2-L |
| Boat crew general instructions | - Inexperienced crew | - Mechanical and material damage - Injury to crew - Failure to launch recovery boat safely - Injury to passengers | D2=M | - Train personnel - Follow procedures - Good communications - Good brief - 1, 3, 5, 9, 11, 12 | B2-1 |
| Launching boat | - Boat not in state of readiness | - Inability to launch start occupy | CM=3 | - Carry out routing maintenance and safety checklist schedules. Good communication with Bridge by radio - 1, 3, 5, 9, 11, 12 | B2-L |
| Recovering boat | - Unfavourable conditions - Vessel crew not in state of readiness | - Inability to recover boat - Mechanical/material damage - Injury to crew - Injury to deck crew | C3=M | - Personnel training and experienced - Good communication - Long ropes for recovery vessel available - Weather watch - 1, 3, 5, 9, 11, 12 | B2-L |

| ACTIVITY : Ship to Ship Transfer of Materials | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 89 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Mooring ship alongside | - Weather conditions | - Both ships - All personnel - Delay to project | H E-2 | - Fenders to be placed alongside - Captains to evaluate conditions - Approach of second ship to be in a slow controlled manner - Only Captains to give final approval to proceed - Marine checklist to be completed - Briefing to be held prior - Task Plan to be produced - 1, 3, 9, 11, 12 | M E-1 |
|  | - Loss of DP | - See section 3.8 Ships Generic Risk Assessment | H E-2 | - See section 3.8 Ships Generic Risk Assessment | M E-1 |
|  | - Communication failure - Incorrect instructions - Misheard instructions | - Ships - All personnel - Delay to project | H E-2 | - Task briefing/toolbox talk - Comms. Check to be completed with all stations - Single point control - 1, 3, 9, 11, 12 | M D-2 |
| Mooring ship alongside (cont) | - Impact damage - MOV | - Ships - All personnel - Delay to project | H E-2 | - None essential personnel to remain clear - MOB to be made ready - All personnel to wear life jackets - Single point control - 1, 3, 9, 11, 12 | M C-2 |
|  | - Fouled mooring lines | - Ships equipment - Delay to project | M C-2 | - Both ships to ensure lines in good condition - Good seamanship | L B-1 |
| Crane transfer of deck cargo | - Vessel stability | - Both ships - Man overboard - Collision damage | H D-3 | - Tag lines - Captains to evaluate - Only Captains to allow work to proceed - MOB ready - Toolbox talk - Single point control - 1, 3, 9, 11, 12 | L C-1 |
|  | - Swinging load | - Ships - Ships equipment - Personnel - Load - Delay to project | H D-3 | - Monitor weather - Vessel stability - Tag lines - Good comms - A. Independence crane, speed, performance to be checked - 1, 3, 9, 11, 12 | L C-1 |
|  | - Impact with superstructure and deck equipment | - Ships - Ships equipment - Personnel - Load - Delay to project | H D-3 | - Monitor weather - Vessel stability - Tag lines - Good comms. - A. Independence crane, speed, performance to be checked - 1, 3, 9, 11, 12 | M D-2 |
| Landing Loads | - Swinging Load | - Ships - Ships equipment - Personnel - Load - Delay to Project | H D-3 | - Monitor load movement. - Use Tag Lines - Clear escape route - 1, 3, 9, 11, 12 | M D-2 |

| ACTIVITY : ROV Support Operations Alongside Rig, Platform or Drill Ship | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 90 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Communications | - Not following procedures correctly - Cross use of channels - Interference from external sources | - Misinformation - Misdirection - Human injury - Mechanical/material damage - ROV/Deck crew - Loss of vehicle | E5=H | - Good communications to be maintained between Bridge/Survey and ROV Control - Correct operating procedures and agreed operating channels - Alternative communication method confirmed - Use dedicated communications between ROV Control/Bridge/Survey - Single point communication control - 2, 3, 5, 9, 11, 12 | C1=L |
| Operation activities in vicinity of anchor pattern with ROV deployed | - Loss of DP - Subsea collision - Loss of ROV/TMS - Operating ROV with extended tether | - ROV/TMS/vessel contact with anchor wire - Damage to vessel - Loss of ROV - Damage to ROV/TMS - ROV entrapment | E5=H | - Confirm position and catenary of anchor wires with Survey/Bridge - Ensure at least 2 independent reference systems are on line - DP Officers on watch to be extra vigilant - Identify vessel escape routes in event of emergency - ROV to be deployed the maximum possible horizontal distance from anchor wire - Good tether management - Toolbox talks - 2, 3, 5, 9, 11, 12 | C1=L |
| Relocation of ship whilst ROV is deployed | - Loss of DP - Subsea collision/contact with anchor wire - Loss of power - Loss of reference system | - Loss of ship’s position - Damage to ROV/TMS - ROV entrapment - Loss of ROV | E5=H | - Ensure at least 2 independent reference systems are on line and a 3rd available on standby - DP Officers on watch to be extra vigilant - Good, clear, concise communications - 2, 3, 5, 9, 11, 12 | C1=L |
| ROV operating on extended tether | - Damage to ROV/TMS - Loss of ROV - Material damage - Damage to third party/subsea equipment | - Damage to vessel - Loss of ROV - Damage to ROV/TMS - ROV entrapment - Project loss | E5=H | - Good communications to be maintained between Bridge/Survey and ROV Control - Good tether management - Good vehicle HPR reference display - HPR on vehicle must be fully operational - Toolbox talks | D1=M |

| ACTIVITY : DP Operations Close to a Surface Installation DP Operations Close to a Subsea Structure | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 91 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Simultaneous operations Rig operations | - Lifting operations overside work - Flaring - Drilling - Communications failure - Mis-read comm’s - Incorrect understanding | - Dropped loads - Overhead lifting - Dropped objects - Noise - Heat - Interference - Minimum distances - Uncontrolled operation - Undesired operation | E3=H | - Cancel or Permit to Work - No crane operations outside drill rig ship’s side - Halt drilling activities - Flare N/A - Access to be unrestricted - Single point of control - Communications to be reported/acknowledged - DSV to control operations - 2, 3, 5, 9, 11, 12 | E1=M |
| ROV Operations | - Blackout - Fouled tether - Cut tether - Lost vehicle - Cutter failure - Short circuit - Degraded ROV ability | - Lost ROV - Damaged ROV - Uncontrollable ROV - Delay to project - Undesired operations - Uncontrolled operations | E3=H | - Rig vehicle crew to provide brief - Rig vehicle to be powered down - Single point control - Communications to be repeated/acknowledged - No overside rig operations - No Simops without ROV clearance - 2, 3, 5, 9, 11, 12 | E1=M |
| Recovery of stranded vehicle | - Fouled ROV - Fouled recovery wire - Damaged vehicle - Blacked out vehicle - Lost vehicle - Lost 2nd vehicle | - Cost of loss - Delay to project | E3=H | - Cutter to be fitted - Vehicle to stand-off 10m without cutter - Comm’s to be established - Single point control - DSV to control operations - 2, 3, 5, 9, 11, 12 | E1=M |
| Recover to deck | - Swinging load - Dropped load - A-frame breakdown - Winch breakdown - Winch operations - A-frame operations - Seabed | - Damage to ROV - Damage to lift wire - Damage to lifting equipment - Injury to personnel - Man overboard - Cost of loss - Delay to project | E3=H | - Certified rigging only - Restrict access to aft deck - Vessel to move off 50m prior to air grip lift - 2, 3, 5, 9, 11, 12 | E1=M |

| ACTIVITY : Lifeboat Drills | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 92 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. VIGILANCE |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Launching of lifeboat Recovery of lifeboat Driving lifeboat | - Malfunction of brakes - Excessive swinging of boat - Failure to clear all securing devices - Fall overboard - Slipping hooks - Trapped fingers - Human error - Failure of lifting gear (e.g. fall chains) | - Boat in free fall - Rectifying boat jammed in davits in dangerous position - Drowning / hypothermia - Faulty lifting of boat - Injury personnel - Damage to lifeboat / ship - Injury to personnel - No control of lifeboat | H(D4) | - Do not walk on top of boat - Personnel inside the boat seated and strapped in - Double check securing devices - Ensure correct engagement of hook securing device - Life vest to be worn - Only relevant personnel to be involved - Coxswain of lifeboat must be competent - Regular maintenance - Start engine prior to water borne - Engineer to be taken along - 1, 2, 3, 9, 12, 13 | L(B2) |

| ACTIVITY : Liferaft | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 93 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. VIGILANCE |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Change out | - Fall overboard - Lifting injury - Liferaft fall - Accidental inflation | - Drowning / hypothermia - Personnel injury - Damage to rafts | M(A4) | - Safety harness to be used for work - Use mechanical lifting equipment - Good manual handling practice - 1, 2, 3, 4, 5, 6, 8, 10, 11, 12, 13 | L(B2) |

| ACTIVITY : MOB Operations | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 94 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. VIGILANCE |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Launch Recovery Sailing | - Weather condition - Trapped fingers - Excessive recovery of boat | - Damage to boat/crew - Injury to hands | H(D4) | - Judge weather condition - Permit to work - Sufficient slack on lifting wire, suitable hooks - 1, 2, 3, 5, 10, 11, 12, 13 | M(C3) |
| Launching mob boat | - Rigging failure - Weather condition | - Damage to equipment | H(D4) | - Check validation of rigging - Competent personnel to launch - 1, 2, 3, 5, 10, 11, 12, 13 | M(C3) |
| Sailing | - Engine failure - Weather condition | - Drifting into danger - Loss of control | H(D4) | - Extra fuel to be carried - Crew should have knowledge of boat - 1, 2, 3, 5, 10, 11, 12, 13 | M(C3) |

Note:	Mob boat to be launched only on authority of master.

Mob crew to be competent personnel.

| ACTIVITY : Helicopter Operations | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 95 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Helicopter handling | - Moving rotor blades - Strong wind - Flying objects - Tripping hazards - Excessive noise - Loose cargo - Cargo handling - Crash - Fire | - Impact damage to personnel or equipment - Blown off deck - Ear damage - Back strain - Instability for aircraft - Damage to luggage/cargo - Fire - Impact injuries | H(E4) | - Enter or leave helideck in safe zones - Follow instructions - Close survival suits properly - Consider abandoning the operation - Ensure no loose objects on helideck - Ensure correct stowing and securing of cargo / luggage - Strict control of helideck by HLO - Careful monitoring of vessel motion prior to helicopter arrival - Firefighting/crash team standby - First aid assistance - Careful control of baggage weights - Passengers adequately briefed - 1, 2, 3, 4, 9, 10, 12 | M(E1) |

Note:	Follow advice of helicopter landing crew.

| ACTIVITY : Launch/Recovery HRL |  | DATE : Reviewed by: |  |  |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- | --- | --- | --- |
| JSA# 96 |  | LOCATION : |  |  |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |  |  |  |
| Basic/ General Safety Requirements |  |  |  | Project Specific |  |  |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK |  |  | 9. SUPERVISION | 13. VIGILANCE |  |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION |  |  | 10. CERTIFICATION | 14. LIFEJACKETS |  |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION |  |  | 11. TOOL BOX TALKS | 15. |  |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS |  |  | 12. COMPETENT PERSON | 16. |  |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Launch | - Fouling of prop / nozzle | - Damage to davit prop / nozzle | H(E3) | - When stowed and during launch steering gear to be amidships - 1, 3, 11, 12 | L(B2) |
|  | - Moving loads - Restricted areas with protruding objects | - Impact injury - Damage to equipment | H(E3) | - Follow procedure - Ensure personnel are wearing seat belts - Minimum personnel - 1, 3, 11, 12 | L(B2) |
| Removal of clamp and supplies to HRL | - Release of gas pressure - Sharp objects | - Grazed hands - Hearing | H(E3) | - Follow procedure - 1, 3, 11, 12 | L(B2) |
| Water born underway | - Violent movement | - Damage to boat / falls - Impact injury | H(E3) | - Ensure personnel wear seat belts - Care to be taken to avoid falls - Sea sick pills to be taken - Remain inboard with hatches closed - 1, 3, 11, 13, 14 | L(B2) |
| Removal of fouled falls | - Violent movement - Moving falls - Impact | - Trapped hands - Impact injury - Damage to vessel | H(E3) | - Extreme care and awareness to be taken - Minimise personnel exposed - Beware fwd falls have restricted access - 1, 3, 11, 13, 14 | L(B2) |
| Recovery | - Moving falls - Impact | - Trap hands - Impact injury - Damage to vessel | H(E3) | - Extreme care and awareness to be taken - Minimise personnel exposed - 1, 3, 11, 13, 14 | L(B2) |

| ACTIVITY : Changing Lifeboat Suspension Chains | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 97 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. OVERSIDE |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Changing lifeboat fall suspension chains | - Falling overboard - Trapped fingers | - Drowning - Injury to personnel - Damage to lifeboat | E2 | - PPE – safety harness and lifevests to be worn by boat crew - Lifeboat to be well secured - Favourable weather conditions - 1, 2, 3, 4, 5, 6, 9, 10, 11, 12 | B2 |

| ACTIVITY : Domestic Cleaning | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 98 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  |  |  | Project Specific |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. APPROPRIATE PPE |  | 5. PERMIT TO WORK |  | 9. SUPERVISION |  |  | 13. |  |  |
| 2. TASK SPECIFIC PROCEDURE |  | 6. MECHANICAL ISOLATION |  | 10. CERTIFICATION |  |  | 14. |  |  |
| 3. SPECIFIC TRAINING |  | 7. ELECTRICAL ISOLATION |  | 11. TOOL BOX TALKS |  |  | 15. |  |  |
| 4. RESTRICT WORK AREA ACCESS |  | 8. POST BARRIERS / SIGNS |  | 12. COMPETENT PERSON |  |  | 16. |  |  |
| Step 1 Task/Activity Description | Step 2 Hazard |  | Step 3 Who or What Exposed |  | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures |  | Step 6 Residual Risk |  |
| Cleaning with chemicals/ detergents | - Solvent exposure - Spillage |  | - Respiratory and skin damage - Eye injury - Skin burns - Pollution |  | M(C4) | - Consult relevant data sheets and follow instructions - Safety glasses to be worn - Rubber gloves to be worn - Control residuals - 1, 2, 3, 4, 5, 8, 12 |  | L(A3) |  |
| High pressure cleaning | - Malfunction of equipment - High pressure fluid escape |  | - Penetration of skin - Eye injury |  | H(C5) | - Secure area - Check equipment - Safety glasses to be worn - 1, 2, 3, 4, 5, 8, 12 |  | L(A4) |  |
| Seawater cleaning | - Water damage through objects not being secured |  | - Electrical equipment damage - Flooded interior |  | M(C4) | - Secure all openings - 1, 2, 3, 4, 5, 8, 12 |  | L(A3) |  |
| Cleaning cabins, galley, corridors, messroom, offices and recreation room | - Wrong body position - Chemical injury |  | - Back, neck, arm injury - Skin injury - Fall injury - Slippery floors |  | M(C4) | - Ensure correct body position - Use chemical protection - Knowledge of chemicals - Take extra care working on wet surface (wear non slip shoes) - 1, 2, 3, 4, 5, 8, 12 |  | L(A3) |  |
| Making top bunks in bad weather | - Falling |  | - Fall injury |  | M(C4) | - Small foot ladders - Two stewards to be present - 1, 2, 3, 4, 5, 8, 12 |  | L(A3) |  |

Note:  Be aware of C.O.S.H.H. assessments.

| ACTIVITY : Galley Operations | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 99 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Carrying provisions, garbage and laundry | - Wrong body position - Cuts - Falls on stairs | - Back, neck & arm injury - Cuts to hands, arms, etc. - Head, back, arm, leg injury | M(C4) | - Correct body position - Check garbage bags before carrying them - use hand protection - Take extra care carrying laundry/provisions/garbage on the stairs - Training/Manual Handling/Videos - 3, 12 | L(A3) |
| Cleaning machinery | - Cuts - Electric shock | - Hand and arm injury - Heart attack | H(D4) | - Ensure machinery not connected to electricity supply - Knowledge of the machine - 3, 12 | L(A4) |
| Cutting with knives | - Cuts | - Hand and arm injury | H(D5) | - Knowledge of using knives - Pay extra attention when working with knives - 3, 12 | L(A4) |
| Cutting on machine | - Cuts | - Hand and arm injury | M(C4) | - Knowledge of the machine - Use protection as required - details on the machine - PPE’s - 3, 12 | L(A4) |
| Dishwashing | - Cuts - Burns/Scalds | - Hand, arm and face injury | M(C4) | - Take care with glass and china - Be careful with hot water/steam - Lifting top of dishwasher - Correct PPE’s/Rubber Gloves - 3, 12 | L(A3) |
| Making coffee | - Burns/Scalds | - Hand, arm and leg injury | M(C4) | - Pay extra attention when making coffee or dealing with hot water - Use handcloths - 3, 12 | L(A3) |
| Boiling/frying with deep fat fryer | - Burns from warm oil - Fire - Oil boiling over | - Burns/Scalds to all body parts - Damage to vessel | M(C4) | - Knowledge of process - Hand protection - Ensure temperature is not too high - 3, 12 | M(E1) |
| Working with heat | - Burns | - Burns to all body parts | H(C5) | - Knowledge of process - Use hand protection - Pay extra attention working with heat - 3, 12 | M(E1) |
| Carrying garbage on deck | - Falls on wet surfaces - Overhead loads - Slips, Trips and Falls | - Leg, arm injury - Head injury | M(C4) | - Pay extra attention - use correct protective equipment - Be observant when out on deck - 3, 12 | L(A4) |
| Working with non cutting machinery | - Breakage of bones - Bruising | - Arms, hands injury | M(C4) | - Knowledge of machinery - Never put hands or arms in machine when connected to electricity supply - 3, 12 | L(A3) |
| Cutting on meat cutter | - Cuts | - Hand and arm injury | M(C4) | - Knowledge of the machine - Pay extra attention when working on the machine - 3, 12 | L(A4) |
| Working in deep freezer | - Hypothermia - Cold Burns - Slips, Trips and Falls | - Loss of body parts | H(C5) | - Ensure that another person knows that you are working in the deep freezer - Limit time spent in the freezer - Use extra protective clothing - 3, 12 | L(A3) |
| Operating steam oven | - Burns/Scalds | - Face, arms injury | M(C4) | - Pay extra attention when operating steam oven - Knowledge of equipment - Use hand protection - Open door slowly - Stand behind the door - 3, 12 | L(A3) |
| Spillage’s of hot oil, stock, etc | - Carrying heavy loads, hot pans | - Burns - Injury | M(C4) | - Correct body position - Look at route - Oven gloves - 3, 12 | L(A3) |
| Meat cannon | - Cuts | - Hands | M(C)4 | - Put cannon in the correct position - Butchers gloves - Use closed hands, not spread fingers | L(A2) |

| ACTIVITY : Garbage Compactor | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 100 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Garbage/rubbish compactor | - Shattering glass - Airborne fragments/ liquids - Burst air supply hose - Fracturing fittings - Trapped hands - Possible risk to passing personnel - Foul odour | - Eye injury - Body injury - Injury to hands and fingers - Odour could bring on nausea | M(C4) | - PPE, including safety glasses - Operator to be familiar with equipment - Close off and open supply valve before and after use - Requires to be emptied at regular intervals - Ensure non essential personnel are clear of machine prior to operation - 1, 3, 12 | L(A3) |

| ACTIVITY : Mobilisation And Demobilisation | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 101 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description |  | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- | --- |
| Access to ship via gangway |  | - Dropped objects - Unsecured attachment - Tidal motion - Inclined/slippery gangway | - Personal injury - Man Overboard - Trip/slip/fall - All personnel | H(E5) | - Strict implementation of alcohol and drug abuse policy for ALL personnel - Familiarisation to be completed | M(E1) |
| Mobilisation of Company and subcontract personnel to vessel including temporary visitors |  | - Lack of familiarisation - Lack of knowledge - Inexperience | - All personnel who fall into this category are exposed to significant risk particularly personal injury | H(E5) | - All such persons to report to radio room - Safety induction to be given at the appropriate level - Subcontractor personnel carrying out work onboard the vessel are to be fully briefed by their employers prior to arrival onboard - Company familiarisation program - Shift Supv & Deck Foreman to be particularly vigilant around deck area regarding “new” personnel - Restricted access - Familiarisation to be completed - 1, 3, 5, 9, 11, 12 | M(E1) |
| General deck work to include manual handling, hot work, fabrication, modifications etc |  | - Hot work of all kinds - Grinding sparks - Unsecured objects - Congested area - Activity interphase | - Fire, explosion - Eye injury - Burns - Personal injury - All personnel on deck | H(E5) | - Correct management of change - Familiarisation to be completed - 1, 3, 5, 9, 11, 12 | M(C4) |
| Lifting/moving equipment from the ship onto the quayside. |  | - Shifting loads - Insecure objects falling off lifts - Overhead lifting operations - Dropped loads/objects | - Impact - Equipment damage - Damage to ship’s deck, hull & equipment. - Exceeding load limits - Injury to personnel on deck and the quayside | H(E5) | - Use correct/certified rigging - Inspect rigging before use - Good communication between crane op, banksman & Deck Foreman. - Secure the area - Use tag lines - Confirm status of equipment to be lifted - Brief subcontract crane driver - Do not transfer the lift over working personnel or sensitive areas - 1, 3, 5, 9, 11, 12 | L(A4) |
| Lifting/moving equipment from the quayside onto the ship. | - Shifting loads - Insecure objects falling off lifts - Overhead lifting operations - Dropped loads/objects - Man overboard |  | - Impact - Equipment damage - Damage to ship’s deck, hull & equipment. - Exceeding load limits - Injury to personnel on deck and the quayside - Drowning | H(E5) | - Use correct/certified rigging - Inspect rigging before use - Good communication between crane op, banksman & Deck Foreman. - Secure the area - Use tag lines - Confirm status of equipment to be lifted - Check wind speed is within limits for lift - Do not transfer the lift over working personnel or sensitive areas - Leave sufficient access between deck cargo for emergency access - 1, 3, 5, 9, 11, 12 | L(A4) |
| Rigging and slinging | - Rigging failure - Dropped loads - Dropped objects |  | - Damage to equipment - Injury to personnel - All Personnel | H(E5) | - Wear correct PPE - Use correct/certified rigging - Perform visual inspection of rigging - Remove all loose objects - Be extra vigilant - 1, 3, 5, 9, 11, 12 | M(E1) |
| Installation of seafastenings | - Hot work of all kinds - Grinding sparks - Welding Operations - Unsecured objects - Congested area - Damaged rigging equipment |  | - Fire, explosion - Eye injury - Burns - Personal injury - All personnel on deck | H(D5) | - Refer to generic and project level 1 risk assessments - 1, 3, 5, 9, 11, 12 | M(C4) |

| ACTIVITY : Dynamically Positioned Diving Operations | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 102 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Diving Operations utilising Dynamic positioning | - DP reference system limitations - Confused communications - Erratic vessel move - Third party shipping - Rapid change in current speed and direction - Rapid change/deterioration in environmental conditions. - Close proximity of DP reference systems to the worksite | - Reduced DP reference system operational limits - Loss of HPR signal - Hull interference to Taut wire - Degradation of DP status - Collision - Possible unplanned vessel movement - Injury to diver - Increased power - Excessive thruster wash - Increased subsea noise - Interference to divers | H(D5) | - Operate vessel in accordance with regulatory and statutory requirements - Utilise available system redundancy - Monitor weather and maintain lookout - Maintain good radar watch - Confirm all vessel moves - D.P.O. must not deploy reposition or recover taught wire without informing Dive Supv - Work within limits of HPR & Taut wire - Monitor DGPS system if online - Monitor artemis function if online - Pre plan location of HPR transponders and taut wires - Monitor fishing and third party vessel activity - 2, 3, 5, 9, 10, 11, 12 | M(C4) |
| Relocation of ship whilst diving bell is deployed | Failure of ships equipment Loss of power Loss of reference system | Loss of ship’s position DP reference system exceeding operational limits Diver entrapment Injury to Divers | H(D5) | Ensure sufficient independent reference systems are on line one of which should be a surface reference system DP officers on watch to be extra vigilant Good clear concise communications Inform Dive Supv of all vessel movements ie. heading and position 2, 3, 5, 9, 10, 11, 12 | M(E1) |

| ACTIVITY : Shallow Water DP Diving Operations | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 103 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Shallow water diving operations | - Ship’s Thrusters - Exceeding diver excursion limits - Tidal rise and fall - Excessive subsea current - Reduced visibility | - Diver injury - Umbilical entrapment - Diver entrapment - Barotrauma - Reduced DP reference system operational limits | H(E5) | - Umbilicals secured in bell at maximum permissible length - Bellman to inform Supervisor of umbilical length deployed from bell - Monitor divers depth and PPO2 Bellman/divers/ supervisors are familiar with umbilical markings | L(A3) |
|  | - DP reference system limitations - Confused communications - Erratic vessel movements - Close proximity of DP reference systems to the worksite | - Loss of HPR signal - Hull interference to Taut wire - Degradation of DP status - Possible unplanned vessel movement - Injury to diver - Increased power consumption - Excessive thruster wash - Increased subsea noise - Interference to divers | H(E5) | Good management of diver umbilical Ensure umbilicals are negatively buoyant. - Ensure bailouts are charged to capacity - Generate accurate tidal graph - Monitor current speed and rise and fall - Diver only to lock in/out during periods when current speed is workable - Work within limits of HPR & Taut wire - Pre plan location of HPR transponders and taut wires - At least one surface reference system online at all times | L(A3) |

| ACTIVITY : DP Diving Operations Within An Anchor Pattern | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 104 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Operational activities in vicinity of anchor pattern with diving bell deployed | Failure of ships equipment Subsea cables/chains Loss of reference system | Loss of ship’s position Contact with anchor wire Loss of diving bell Damage to diving bell Loss of diving bell life support services Diver entrapment Loss of breathing gas Loss of divers hot water Divers | H(D5) | Where possible lower nearest anchor wire until below diving bell. Horizontal separation of 50M between bell and nearest catenary maintained Confirm position and catenary of anchor wires utilising WOROV and plotted Weather limitations pre defined Weather modelling and drift off predictions on Bridge Vessel to operate in pre-planned escape corridor Ensure at least three independent reference systems are on line and a fourth available on standby DP officers on watch to be extra vigilant Perform individual Risk Assessment 2, 3, 5, 9, 10, 11, 12 | M(E1) |
| Operational activities in vicinity of anchor pattern with diving bell deployed (cont) | Failure of ships equipment Subsea cables/chains Loss of reference system | Loss of ship’s position Contact with anchor wire Loss of diving bell Damage to diving bell Loss of diving bell life support services Diver entrapment Loss of breathing gas Loss of divers hot water Divers | H(D5) | Identify vessel escape routes in event of emergency Diving bell to be deployed the maximum possible/practicable horizontal distance from anchor wire Diving bell to be stored at upward extent of ascending excursion Diver excursion umbilical to be properly managed at all times On bottom bell turn arounds are prohibited during this operation Weather to be monitored closely Anchored structure requirements to be implemented and adhered to 2, 3, 5, 9, 10, 11, 12 | M(E1) |

| ACTIVITY : Diving Operations Alongside A Platform Or Structure | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 105 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Diving Operations utilising Dynamic positioning Alongside a Platform or structure | - DP reference system conflict - Trapped, snagged taut wire - Loss of DP - Loss of HPR signal - Loss of Artemis signal - Confused communications - Dropped objects | - Damage to vessel - Taught wire weight hits diver - Taught weight wire parts - Degradation of DP status - Damaged or broken crane wire or hook - Collision | H(D5) | - Obtain permission to enter 500 metre zone - Identify platform based external activities - Use full system redundancy - Monitor weather/tide/current. - Confirm all vessel moves - Regular checks of taught wire rigging - D.P.O. must not deploy reposition or recover taut wire without informing Dive Supv - Evaluate project specific and location specific requirements - 2, 3, 5, 9, 10, 11, 12 | M(C4) |
| Diving Operations utilising Dynamic positioning Alongside a Platform or structure (cont) | - Dumping of mud/cuttings - Simultaneous operations - Strong surface currents - Platform based emissions - Foreign objects entangling with thrusters | - Possible unplanned vessel movement - Bell collides with structure - Reduced visibility - Injury to diver | H(D5) | - Work within limits of reference systems - Ensure Artemis fixed station integrity remains intact - Monitor DGPS system - Identify vessel escape routes - Confirmation of HPR transponders in use and taut wire locations - Maintain good clear communications - Evaluate project specific and location specific requirements - 2, 3, 5, 9, 10, 11, 12 | M(C4) |

| ACTIVITY : Diving Operations In The Vicinity Of A Subsea Structure | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 106 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Diving Operations in the vicinity of a fixed subsea structure. | - Erratic vessel move - DP reference system conflict - Trapped, snagged crane wire - Loss of HPR signal - Confused communications - Deployment of subsea equipment | - Loss of ship’s position - Damage to equipment being installed - Bell collides with structure - Injury to diver - Taut wire weight entanglement - Degradation of DP status - Damaged or broken crane hook - Dropped objects - Collision | H(D5) | - Use full system redundancy - Monitor weather - Confirm all vessel moves - Regular checks of taut wire rigging - D.P.O. must not deploy, recover or reposition taut wire without informing the Dive Supv. - Work within the limits of the HPR - Monitor DGPS system - Identify vessel escape routes - Confirmation of HPR transponders in use and taut wire locations - Locate bell and clump weight above the structure - Evaluate project specific and location specific requirements - 2, 3, 5, 9, 10, 11, 12 | M(C4) |

| ACTIVITY : Diving Operations In The Vicinity Of Drilling Mud, Cuttings, Etc | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 107 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Diver Working in and around large quantities of drill cuttings | - Poor Visibility - Caustic substance - Transfer of mud into bell or chambers | - Skin irritation - Chemical burns - Nausea - Contamination of bell and chambers - Deck crew - Divers - Life support personnel | H(D4) | - Control movement through mud - Wear protective undersuits and boots - Apply barrier creams - Wash thoroughly personnel and equipment - Clean off excess mud and remove suits before entering bell - Evaluate project specific and location specific requirements - 2, 3, 5, 9, 10, 11, 12 | M(C2) |

| ACTIVITY : Overboarding & Deploying Equipment, Concrete Mattresses, Support Piers & Pipeline Spoolpieces From Vessel Deck to Subsea Worksite | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 108 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Removal and relocation of seafastenings | - Insecure load - Dropped object - Swinging Loads - Incomplete removal of seafastenings | - Personnel injury - Damage to equipment - Deck Crew - Crane Operator | H(D5) | - Ensure seafastenings are correctly removed and properly slung - Use certified rigging which has been visually checked - Ensure all loose items have been removed or are secured - Use of tag lines - Suitable weather and forecast - Minimum personnel on deck | L(A4) |
| Lifting and moving equipment on ships deck | - Swinging loads - Manual handling - Dropped objects - Insecure loads - Hydraulic pressure release at crane | - Personnel Injury - Damaged equipment - Deck Crew - Vessel damage - Trips, slips and falls | H(D5) | - Rigging certified and fit for purpose - Monitor environmental conditions - Consider ship movement - Use of tag lines - No loads to be lifted over personnel or sensitive areas - Not lifting excessive load by hand - Correct lifting procedure - Wear safety equipment - Correct and clear communication - Good communications with crane - Minimum personnel on deck | L(A4) |
| Overboard Equipment | - Swinging loads - Excessive loading - Hydraulic pressure release - Confined space - Dropped object - Proximity to thruster | - Damage to subsea equipment - Injury to divers - Injury to deck crew - Thruster damage - Vessel damage - Slips, trips and falls | H(D5) | - Suitable weather conditions and forecast - Rigging certified and fit for purpose - Good communications - Weighted tag lines - Minimum personnel on deck - Monitor clearance to vessel deck - Use crane line out meter - calibrate on first lift | M(E1) |
| Deploying spoolpiece from the ship’s deck down to the seabed | - Shifting loads - Insecure objects falling off lifts - Uncontrolled movement - Pressure differential if not allowed to equalise to ambient | - Impact - Equipment damage - Damage to Flange Faces - Exceeding load limits - Deck Crew - Crane Operator - Divers | H(E5) | - Only certified lifting equipment - Check equipment before use - Monitor environmental conditions - Monitor vessel movement - Flange Protectors Fitted with Vent holes to allow free-flooding - Good communication - Secure the area - Use tag lines - Confirm status of spools to be lifted - Ensure crane alarm limits are set - Lifelines - Workvests - Handrails secured | L(A4) |
| Overboarding mattresses and concrete piers at sea | - Shifting loads - Objects falling off lifts - Break up of mattress - Insecure lift points | - Dropped Objects - Impact - Equipment damage - Exceeding load limits - Deck crew | H(D5) | - Only certified lifting equipment - Check/inspect equipment before use - Personnel not to be located in confined areas - Secure the area - Use tag lines - Confirm status of cargo to be lifted - Ensure crane operating alarm limits are set - Safety hooks to be used on lift frame and crane pennants - Attach lightsticks or subsea strobes to loads - Toolbox talk to be held prior to operations | L(A4) |
| Deploying mattresses and concrete piers subsea | - Uncontrolled lowering of load - Break up of mattress - Confused communications | - Objects falling off lifts - Injury to divers - Diver entrapment - Diver umbilical entrapment - Damage to diving bell - Damage to Pipeline - Damage to subsea equipment | H(E5) | - Good communication to be maintained working channels to be agreed with redundancy available - Use tag lines - Consider sea state and wind - Consider subsea visibility - Consider vessel movements - Good management of diver excursion umbilicals - Locate divers in position of safety during load deployment - Lower load a safe distance away from pipeline until 10 metres above seabed then relocate into position - Ensure divers reserve gas supplies are full prior to diving operations - Use of steel cargo net to recover previously laid mats or mats without known history | M(E1) |
| Lift Frame Recovery | - Heaving or extended footprint of load - Objects falling off lift. - Uncontrolled movement | - Equipment damage - Deck crew - Crane operator | H(D5) | - Assess weather conditions only attempt in stable conditions. - Clear and secure the landing area | L(A4) |
| Wet Storing Equipment | - Unstable seabed - Debris on seabed - Congested laydown area - Loss of equipment | - Damage to equipment - Damage to pipeline - Trapped umbilicals - Equipment sinking into seabed - Separation of equipment | H(D5) | - Wet store area to be identified - HPR beacon attached to equipment - Seamap showing area - ROV pre-survey of wetstore area - Good communications - Umbilical management - Suitable weather conditions and forecast | L(A3) |
| Deploying heavy loads from the ship’s side down to the seabed | - Heavy Lift Crane Operations - Shifting loads - Insecure objects falling off lifts - Uncontrolled movement - Dropped Object | - Impact - Ship’s equipment damage - Damage to Pipeline/subsea equipment - Exceeding load limits - Change in ship’s stability - Deck Crew - Crane Operator - Divers | H(D5) | - Only certified lifting equipment - Check/inspect equipment before use - Secure the area - Use tag lines - Confirm status of load to be lifted - Ensure crane alarm limits are set - Lifelines - Workvests - Handrails removed temporarily - Tank vents removed temporarily - Monitor weather conditions and forecast - Perform detailed toolbox talk - Personnel not to be located in confined areas - Pre heavy lift meeting to be held prior to crane operations and lift plan - Good communication to be maintained working channels to be agreed with redundancy available - Consider sea state and wind - Consider subsea visibility - Consider vessel movements - Attach lightsticks or subsea strobes to loads - Good management of diver excursion umbilicals - Locate divers in position of safety during load deployment - Lower load a safe distance away from pipeline/subsea structure until 10metres above seabed then relocate under divers instruction - Ensure divers reserve gas supplies are full prior to diving operations - Position vessel taking into consideration all relevant factors | L(A4) |

| ACTIVITY : Pipeline Flooding Activities | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 109 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Flooding Operations | - Confined space - Subsea hoses - Wire rope downlines - Pressure differentials | - Entanglement of hoses in thrusters - Damage to equipment - Explosion/fire | H(D4) | - Material COSHH sheets reviewed with respect to first aid, firefighting, spillage and handling and storage. - Confirm integrity and certification of all hoses and connections prior to subsea deployment - 1, 2, 3, 4, 5, 8, 9, 10, 11, 12 | L(A4) |
|  | - Moving/rotating machinery - Confused communications | - Pressure related injury - Pollution - Damage to flange face - Slips, trips and falls - Burst Hose - Dropped object - Injury to divers - Injury to deck personnel | H(D4) | - Attach downlines with weak links - Good management of diver excursion umbilicals - Maintain good clear communications - Protect surface hose runs - Secure area around pressurised hoses and equipment - Refer to generic risk assessments for transfer of fuel oil - Good communications during start/ stop of equipment - 1, 2, 3, 4, 5, 8, 9, 10, 11, 12 | L(A4) |

| ACTIVITY : Air Diving Operations Utilising Ship’s Crane To Assist In Accessing Worksite | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 110 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. VIGILANCE |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Preparation of Diving System | - Contaminated Divers breathing air supply. - Incorrect recording of dive profile | - Carbon monoxide poisoning - Carbon dioxide poisoning - Incorrect dive profile - Decompression sickness | H(E5) | - Ensure all air supplies and stored air has been sampled for impurities. - Ensure dive recording computer system is operating correctly if used - Analyse gas - 1, 2, 3, 8, 9, 10, 11, 12 | M(C4) |
| Preparation of Diving System (cont) | - Faulty equipment | - Breakdown of equipment whilst in service - Schedule delays - Divers | H(E5) | - Ensure all communications CCTV and recording systems are fully operational. - Ensure all equipment has been properly maintained, checked with sufficient quantities of therapeutic and emergency oxygen/air available. - Ensure air intakes are located away from exhaust or similar contaminant sources - Ensure HP air supply is available to DDC for rapid pressurisation - Ensure Divers basket reserve air cylinders are fully charged with first stage valves checked and operational. - 1, 2, 3, 8, 9, 10, 11, 12 | M(C4) |
| Deployment to worksite | - Excessive movement of dive basket - Uncontrolled lowering of dive basket - Environmental conditions - Ships thrusters - Absence of handrails - Rotating machinery - Wires under tension - L P air hoses under pressure | - Exceeding planned depth of dive - Divers falling out of basket - Entanglement of divers umbilicals - Man overboard - Deck crew - Divers | H(E5) | - Confirm maximum diver excursion umbilical length and fit mechanical stops - Divers umbilicals to be routed through “doughnuts” on dive basket - Each diver to have separate tender - Ensure guard chains are fitted after deployment - Confirm integrity of HP & LP air hoses and supply, back up supply available - Surface tenders and crane operator to be fully briefed on their duties. - 1, 2, 3, 8, 9, 10, 11, 12 | M(C2) |
| Cross hauling of divers basket | - Unplanned changes in basket depth - Spinning of dive basket - Confused communications | - Reduction in working time - Entanglement of divers umbilicals - Disorientation - Nausea | H(D5) | - Confirm cross haul device is certified man riding and is fully operational - Install hold back ropes to basket. - Good management of divers umbilicals - Maintain constant depth of dive basket - Confirm communications with back up - Cross hauling operations only to be performed with the basket submerged. - Risk assessment to be performed on each case | M(C2) |
| Diving operations on/around a platform or structure including splash zone operations. | - Dropped objects - Platform based SIMOPS - Dumping of water, mud etc - Salt water inlets/outlets located subsea - Fishing nets, lines, hooks etc. - Swell and sea motion. | - Barotrauma - Pnuemothorax - Decompression sickness - Diver entrapment - Damage to divers equipment - Injury to divers | H(E5) | - Locate ship to provide optimum lee - Maintain good communications with platform/structure - MOB and field standby boats to be notified and available for intervention. - Divers to avoid “breath holding” in splash zone - Scaffolding, overside and similar platform based operations strictly prohibited - Fishing activities strictly prohibited - Dive Supv to be made aware of seawater inlets/dumps and status of same - Dumping of muds, cuttings etc to be strictly controlled. - Ensure divers bailout is fully charged - Monitor environmental conditions - Monitor vessel movements - Be aware of possible changes in direction of wind, waves and current - 1, 2, 3, 8, 9, 10, 11, 12, 13 | M(E1) |

| ACTIVITY : Transfer of tubular Lifts From Supply Boat To a Drilling Rig or Platform Whilst Performing Diving Operations | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 111 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Lifting / moving tubulars at sea in parallel with saturation diving operations | - Trapped/snagged loads - Falling/ swinging loads - Shifting loads - Objects falling off lifts | - Dropped tubulars - Impact damage - Equipment damage - Exceeding load limits - Injury to personnel - Supply boat crew - Divers | H(E5) | - Only certified and fit for purpose lifting equipment and rigging to be used - Inspect rigging equipment before use - Experienced personnel to be employed - Maintain good communication - Use tag lines - Consider sea and wind state - Injury to personnel - Supply boat crew - Divers - 1, 2, 3, 4, 5, 8, 9, 10, 11, 12 | M(E1) |
|  | - Objects falling off lifts | - Injury to personnel - Supply boat crew - Divers | H(E5) | - Injury to personnel - Supply boat crew - Divers | M(E1) |

| ACTIVITY : Saturation Diving Operations Utilising Extended Diver Excursion Umbilicals To Access The Worksite | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 112 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Shallow water diving operations | - Ship’s Thrusters - Exceeding diver excursion limits - Tidal rise and fall - Excessive subsea current - Reduced visibility - DP reference system limitations | - Diver injury - Umbilical entrapment - Diver entrapment - Barotrauma - Reduced DP reference system operational limits - Loss of HPR signal - Hull interference to Taut wire | H(E5) | - Umbilicals secured in bell at maximum permissible length - Bellman to inform Supervisor of umbilical length deployed from bell - Monitor divers depth and PPO2 Bellman/divers/ supervisors are familiar with umbilical markings Good management of diver umbilical - 3, 5, 9, 11, 12 | L(A3) |
| Shallow water diving operations (cont) | - Confused communications - Erratic vessel movements - Close proximity of DP reference systems to the worksite | - Degradation of DP status - Possible unplanned vessel movement - Injury to diver - Increased power consumption - Excessive thruster wash - Increased subsea noise - Interference to divers | H(E5) | Ensure umbilicals are negatively buoyant. - Ensure bailouts are charged to capacity - Generate accurate tidal graph - Monitor current speed and rise and fall - Diver only to lock in/out during periods when current speed is workable - Work within limits of HPR & Taut wire - Pre plan location of HPR transponders and taut wires - At least one surface reference system online at all times - Adhere to requirements of specific extended umbilical procedure - 3, 5, 9, 11, 12 | L(A3) |
| Slewing of divers basket and deployment to worksite | - Excessive movement of dive basket - Uncontrolled lowering or raising of dive basket - Environmental conditions - Ships thrusters - Wires under tension - Unplanned changes in basket depth - Spinning of dive basket - Confused communications | - Exceeding diver upward excursion limits - Entanglement of divers umbilicals - Barotrauma - Injury to Divers - Entanglement of divers umbilicals - Disorientation - Nausea | H(E5) | - Confirm diver maximum upward excursion limit - Confirm maximum diver excursion umbilical length and fit mechanical stops - Divers umbilicals to be routed through dive basket - Confirm cross haul device is certified and is fully operational - Good management of divers umbilicals - Maintain constant depth of dive basket - Confirm communications with back up - Slewing operations only to be performed under Diver direction. - Ensure Crane Operator and Bridge Watch Officers are fully familiarised with all aspects of the operation. - 3, 5, 9, 11, 12 | M(C2) |

| ACTIVITY : Water Jets | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 113 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  |  |  |  |  | Project Specific |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. APPROPRIATE PPE |  |  | 5. PERMIT TO WORK |  |  | 9. SUPERVISION |  |  |  |  | 13. |  |  |
| 2. TASK SPECIFIC PROCEDURE |  |  | 6. MECHANICAL ISOLATION |  |  | 10. CERTIFICATION |  |  |  |  | 14. |  |  |
| 3. SPECIFIC TRAINING |  |  | 7. ELECTRICAL ISOLATION |  |  | 11. TOOL BOX TALKS |  |  |  |  | 15. |  |  |
| 4. RESTRICT WORK AREA ACCESS |  |  | 8. POST BARRIERS / SIGNS |  |  | 12. COMPETENT PERSON |  |  |  |  | 16. |  |  |
| Step 1 Task/Activity Description |  | Step 2 Hazard |  |  | Step 3 Who or What Exposed |  |  | Step 4 Quantify Risk |  | Step 5 Recommended Corrective Actions / Control Measures |  |  | Step 6 Residual Risk |
| Use of LP water jet | - Increased noise levels - LP water - Diesel driven equipment - Moving/rotating machinery - Transfer of fuel oil - Unrestrained subsea hoses |  |  | - Pressure related injury - Injury to diver - Injury to deck crew - Entanglement of hoses in thrusters - Reduced operational capability - Explosion/fire - Spillages |  |  | E3 = H |  | - Confirm integrity of all hoses and connections prior to subsea deployment. - Secure supply hose on down line - Good management of diver excursion umbilicals - Protect surface hose runs - Secure area around pressurised hoses and equipment - Maintain communication between divers - Subsea valve within reach - Emergency pack for spillages - Fuel bung system on deck - Notify Bridge - 2, 3, 9, 11, 12 |  |  | D1 = M |  |
| Use of HP water jet | - Use of HP water jet - Increased noise levels - HP water - Diesel driven equipment - Moving/rotating machinery - Transfer of fuel oil - Unrestrained subsea hoses |  |  | - Pressure related injury - Injury to diver - Injury to deck crew - Entanglement of hoses in thrusters - Reduced operational capability - Explosion/fire |  |  | E3 = H |  | - Confirm integrity of all hoses and connections prior to subsea deployment. - Secure supply hose on down line - Good management of diver excursion umbilicals - Protect surface hose runs - Secure area around pressurised hoses and equipment - Maintain communication between divers - Ensure “dead mans” handle is operational - Emergency stop in dive control - Notify Bridge - 2, 3, 9, 11, 12 |  |  | D1 = M |  |

| ACTIVITY : Air Lifting |  | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- | --- |
| JSA# 114 |  | LOCATION : |  | REF PROCEDURES: |
|  | SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description |  | Step 2 Hazard |  | Step 3 Who or What Exposed |  | Step 4 Quantify Risk |  | Step 5 Recommended Corrective Actions / Control Measures |  | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dredging Operations utilising LP Airlift | - Subsea hoses - Wire rope downlines - Pressure differentials - Reduced visibility - Exhaust air - Diesel driven equipment - Moving/rotating machinery - High volume LP air - Heavy subsea equipment - Transfer of fuel oil - Confused communications |  | - Entanglement of hoses in thrusters - Damage to equipment - Air ingress into diving bell - Air ingress into ship’s salt water intakes - Interference with HPR - Explosion/fire - Trapped diver umbilical - Pressure related injury - Injury to divers - Injury to deck personnel - Excessive removal of seabed |  | D3 = H |  | - Confirm integrity and certification of all hoses and connections prior to subsea deployment. - Secure dredge hog hose on down line - Attach air lift to crane and secure air hose to crane wire - Attach downlines with weak links - Install subsea “dead man” handle and surface cut off on air lift - Good management of diver excursion umbilicals - Maintain good clear communications - Locate ship and diving bell with consideration to exhaust air stream - Notify Bridge - 2, 3, 9, 11, 12 |  | B2 = L |  |

| ACTIVITY : Lifting Bags | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 115 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description |  | Step 2 Hazard |  | Step 3 Who or What Exposed |  | Step 4 Quantify Risk |  | Step 5 Recommended Corrective Actions / Control Measures |  | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Working with Lifting Bags | - Uncontrolled ascent - Entrapment |  | - Vessel damage - Diver injury - Diver/umbilical - Project loss |  | D3= H |  | - Certified rigging - Good communication - Refer to IMCA guidance DO16 Rev 3 2007 - 2, 3, 9, 11, 12 |  | B2= L |  |
| Inflating lifting Bags | - Too much lift for load - Uncontrolled load - Diver entrapment in rigging - Dropping load |  | - Vessel damage - Diver injury - Equipment - Project loss |  | D3= H |  | - Tie backs always to be used - Dump line to reach bottom of bag - Diver to have direct control of fill - Extension nozzle to be fitted to inflation hose - 2, 3, 9, 11, 12 |  | B2 = L |  |
| Deflating Lifting Bags | Uncontrolled load Diver entrapment Effect to DP reference |  | - Vessel damage - Diver injury - Equipment - Project loss |  | D3 = H |  | Umbilical management Diver awareness Inform DP operator if large amounts of gas released on deflating bags Notify Bellman - 2, 3, 9, 11, 12 |  | B2= L |  |
| Lift bag operations | - Un-planned rise of over buoyant object - Impact due to bursting or leaking bags after incorrect operation - Air in bell when venting - Drifting of lifted object in current |  | - Damage to equipment - Injury to personnel |  |  |  | - Use certified rigging of correct SWL - Use certified lift bags - Calculate expected lift bag loads - Confirm weight of object to which air bag is attached - Use tripping and holdback lines. - Clearly identify extended dump lines - Maintain bell at safe distance - 2, 3, 9, 11, 12 |  |  |  |

Refer to IMCA D 016 Rev 3 June 2007

| ACTIVITY : Dredge Hog Unit |  | DATE : Reviewed by; |  |  |  | PROJECT : |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| JSA# 116 |  | LOCATION : |  |  |  | REF PROCEDURES: |  |
| SAFETY – FIRST PRIORITY |  |  |  |  |  |  |  |
| Basic/ General Safety Requirements |  |  |  | Project Specific |  |  |  |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK |  |  | 9. SUPERVISION | 13. |  |  |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION |  |  | 10. CERTIFICATION | 14. |  |  |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION |  |  | 11. TOOL BOX TALKS | 15. |  |  |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS |  |  | 12. COMPETENT PERSON | 16. |  |  |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Use Of Subsea “Dredge Hogs” | - Subsea hoses - Wire rope downlines - Pressure differentials - Reduced visibility - Seabed material discharge - Diesel driven equipment - Moving/rotating machinery | - Entanglement of hoses in thrusters - Damage to equipment - Interference with HPR - Explosion/fire - Trapped diver umbilical - Pressure related injury | D3 = H | - Confirm integrity and certification of all hoses and connections prior to subsea deployment. - Secure dredge hog hose on down line - Attach downlines with weak links - Good management of diver excursion umbilicals - Maintain good clear communications - Locate diving bell with consideration to material discharge stream - 2, 3, 9, 11, 12 | B1 = L |
|  | - Heavy subsea equipment - Transfer of fuel oil - Confused communications - Environment - wind, waves, tide - Emptying of strainer box - Leaving equipment working unmanned | - Injury to divers - Injury to deck personnel - Excessive removal of seabed - Unstable spoolpiece - Spillages - Abandonment of equipment - Lost/buried equipment | D3 = H | - Protect surface hose runs - Secure area around pressurised hoses and diesel driven pumping equipment - Refer to generic risk assessments for transfer of fuel oil - Good communications during start/ stop of equipment - Emergency pack for spillages - Fuel bung system on deck - Divers to ensure that they and their equipment are kept clear of Dredge Hog suction inlet - Safety cage over Dredge Hog inlet - Discharge hose at max. extension and out of current flow - 2, 3, 9, 11, 12 | B1 = L |
| Use of Subsea “Dredge Hogs” (cont) |  |  | D3 = H | - Diver to remain above spoolpiece - Check trench level, dredge area evenly not locally - Keep trench side shallow - Check riser/spoolpiece connection is secure. - Dredge Hog familiarisation with deck divers. - Ensure dredge hog is isolated prior to emptying strainer box. - Ensure strainer box is resealed - Unmanned equipment to be tied off to spoolpiece and location marked | B1 = L |

| ACTIVITY : Hydraulic Tools | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 117 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Use of Hydraulic Power tools | - High pressure hydraulics - Rotating cutting and drilling equipment | - Environmental pollution - Pressure injury - Entanglement with rotating equipment - Injury to diver - Injury to deck crew - Entanglement of hoses in thrusters - Reduced operational capability - Oil ingress into diving bell | D3 = H | - Confirm integrity of all hoses and connections prior to subsea deployment. - Secure umbilical on down line - Remove loose items from divers dress - Good management of diver excursion umbilicals - Protect surface hose runs - Physical stop on drills and cutters - Emergency stop in dive control - 3, 9, 11, 12 | B1 = L |

| ACTIVITY : Hydratight Equipment | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 118 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Use of hydraulic bolt tensioning equipment | - High pressure fluids - Excessive oil pressure - Leaking oil - Moving equipment - Dropped objects | - Oil contamination - High pressure fluid injury - Ruptured hoses - Equipment failure - Trapped fingers, limbs - Deck crew - Divers | B3 = M | - Pressure and function test equipment before use - Check items have lanyards. - Control bolt tensioning pressures - Secure area around high pressure pumps - Ensure correct operation of equipment - Do not “over stroke” jacks. - Keep to side of jacks and hoses if possible. - Ensure all whips are connected properly. - 2, 3,8, 9, 11, 12 | B1 = L |
| De-rig Equipment and Recover Work Basket | - Entanglement of diver’s umbilical. Dropping equipment Impact Injury. Damage to equipment | Personnel Damage |  | - Ensure hot line is at zero pressure. - Secure all equipment in basket. - Ensure care is taken when loading basket. - Do not throw jacks back into the basket. - Slow and easy crane movements. - Keep away from under load. - Firmly tend diver umbilical. h) Good communications. |  |

| ACTIVITY : Concrete Mattress Installation | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 119 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Overboarding mattresses and concrete piers at sea | - Shifting loads - Objects falling off lifts - Break up of mattress - Insecure lift points | - Dropped Objects - Impact - Equipment damage - Exceeding load limits - Deck crew | C3 = M | - Only certified lifting equipment - Refer to IMCA guidance DO42 Rev 1 Sep 11 and Mattress Checklists - Check/inspect equipment before use - Personnel not to be located in confined areas - Secure the area - Use tag lines - Confirm status of cargo to be lifted - Ensure crane operating alarm limits are set - Safety hooks to be used on lift frame and crane pennants - Attach lightsticks or subsea strobes to loads - Toolbox talk to be held prior to operations - 1, 2, 3, 4, 5, 9, 10, 11, 12 | B2 = L |
| Deploying mattresses and concrete piers subsea | - Uncontrolled lowering of load - Break up of mattress - Confused communications | - Objects falling off lifts - Injury to divers - Diver entrapment - Diver umbilical entrapment - Damage to diving bell - Damage to Pipeline - Damage to subsea equipment | C3 = M | - Good communication to be maintained working channels to be agreed with redundancy available - Use tag lines - Consider sea state and wind - Consider subsea visibility - Consider vessel movements - Good management of diver excursion umbilicals - Locate divers in position of safety during load deployment - Lower load a safe distance away from pipeline until 10 metres above seabed then relocate into position - Ensure divers reserve gas supplies are full prior to diving operations - Steel cargo nets to be used for removal of previously laid mats or mats without known history - 1, 2, 3, 5, 9, 10, 11, 12 | B2 = L |
| Lift Frame Recovery | - Heaving or extended footprint of load - Objects falling off lift. - Uncontrolled movement | - Equipment damage - Deck crew - Crane operator | E2 = H | - Assess weather conditions only attempt in stable conditions. - Clear and secure the landing area - 1, 2, 3, 4, 5, 9, 10, 11, 12 | C2 = M |

| ACTIVITY : Oxy / Arc Cutting | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 120 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  | SAFETY CRITICAL |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Use of subsea oxy - arc cutting equipment | - Electricity underwater - Build up of oxygen - High pressure gas - 100% oxygen - Leaking oxygen - Moving equipment - Dropped objects - Cutting in the wrong place | - Subsea explosion - Surface explosion - Electric shock - Increased PPo2 in diving bell - Hypoxia - Trapped fingers, limbs - Severe Injury - Deck crew - Divers - Project delay - Asset Damage | E3 = H | - See IMCA D 003 Rev 1 Sep 11 and “Surface Preparation” and “Subsea Operation” Checklists. - PMS of the equipment in use. - Good burning technique and training. - Properly earthed and insulated equipment Ensure correct collets and washers are being used. Check flash arrestor is clean. Ensure rods are compatible. Check polarity is Negative. Regulator O2 clean and certified High flow high pressure regulator - Divers to wear rubber gloves and burning visor - Use separate earth cable attached to work site - Do not locate between electrode and earth - Electrical ELCB to be fitted - Regulated oxygen supply - Well ventilated storage area - Exhaust oxygen to be routed away from diving bell. Use pneumo to assess. - Clear instructions given by Supervisor - Check and confirm cut position. 2nd Diver or ROV to validate. - Positively identify position of any vent holes required. 2nd Diver or ROV to validate. - Supervisor to ensure a ventilation hole is burnt (or cold cut) to prevent O2 build-up. - Exhaust oxygen to be vented clear from cutting location. Use pneumo to assess. - Regular checks to confirm no build up of oxygen sub sea - Support object being cut before final cut - Good escape route - 2, 3, 9, 10, 11, 12 | E1 = M |

| ACTIVITY : Diamond Cutter | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 121 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Training in use of machine. | a) Not understanding the workings or procedures. b) Making mistakes Accidents occurring. | Personnel Damage | H (E3) | - Ensure all personnel understand the operation. - Rig up test piece to practice cut. - 1,2,4,5,6,7,8,9,11,12 | L (C1) |
| Connect crane to cutter. | - Vessel movement in swell or from passing vessels. - Crush or entrapment from movement. - Use correct rigging/slings and shackles. | Personnel | H (E3) | - Secure tag lines to cutter. - Use correct rigging and slings. - Good communication to crane driver. - 1,2,4,5,6,7,11,12 | L (C1) |
| Swing cutter over side. | - Crush or entrapment from swinging load. | Personnel | H (E4) | - Move personnel away from arc of swing. - Good communications to crane driver. - Use tag lines. Refer also to JSA# 108 Overboarding and Deploying equipment. - 1,2,4,5,6,7,11,12 | L (C2) |
| Lower cutter to diver. | - Entrapment or Crush Injury whilst lowering cutter. - Movement of cutter by passing vessels or sea state. | Personnel Damage | H (E4) | - Have diver move away from under load whilst lowering. - Be aware of moving vessel and sea state. - Slow and easy movements with crane. - Night aids in dark or poor visibility. - 1,2,4,5,6,7,11,12 | L (C2) |
| Position diamond cutter on pipe line. | 1. Low Visibility 2. Trapped umbilical-pinched hands. | Personnel Damage | H (E4) | - Have diver move from under load. - Keep divers umbilical firmly tended. - Keep away from pinch points. - Good communications to diver/crane operator/deck personnel. - 1,2,4,5,6,7,11,12 | L (C2) |
| Close clamping device to pipeline. | - Crush injury with clamps. - Umbilical entrapment. c) Injury to diver. d) Loss of gas supplies to diver. | Personnel | H (E4) | - Good communications with diver. - Good communications with crane operator. - Tend diver’s umbilical firmly. - Keep body parts away from clamps. - Slow and easy movements with crane. - 1,2,4,5,6,7,11,12 | L (C2) |
| Lowering blade for measurement. | 3. Cutter starts operating inadvertently. | Personnel Damage | H (E4) | 3a. Good communications with surface. Check with operator that machine is in secure position. 3b. Approach machine from cutting face side only. 3c. Be aware of umbilical position. 3d. Do not approach machine until directed by surface. - 1,2,4,5,6,7,11,12 | L (C1) |
| Start machine | 4. Danger to hands fingers. 5. Diamond cutter blade breaks / pulley wheel severed. | Personnel Damage | H (E4) | 4a. Return to bell before starting cut. ROV to monitor cut. 5a. Move back from cutter and return to bell. ROV monitor cut. - 1,2,4,5,6,7,11,12 | L (C1) |

THE ONLY TIME A DIVER WILL BE PERMITTED TO WORK ON THE MACHINE OR SETTING THE CUTTER ON THE PIPELINE WILL BE WHEN ALL HYDRAULICS ARE COLD AND MAIN POWER PACK IS OFF. ALL CONTROLS WILL BE TAGGED OUT AND LOCKED.

THE DIVER WILL BE ON SURFACE (OR IN BELL) BEFORE ANY HYDRAULICS ARE TURNED ON AND TAG OUT UNLOCKED.

| ACTIVITY : Blind Flange Removal / Installation | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 122 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Lower flange to diver | Flange strikes Diver. Slings part | Personnel | H (C3) | - Maintain vigilance and good communications with surface. Supervision, follow procedures, maintain control, tag lines and communication - Always use correct SWL and rigging with current certification. Check weight, use correct slings. - Refer also to JSA# 108 Overboarding and Deploying equipment. - 1,3,4,5,8,9,11,12 | L (C2) |
| Identify the blind flange to be removed / installed | Removal of wrong flange. | Project Schedule | H (C4) | Follow Procedure - check idents before removing blind. Confirm location and pressure differential status with Client. - 1,3,4,5,8,9,11,12 | L (C2) |
| Open vent valve. | Pressure differential across flange. Severe suction. Negative pressure. Positive pressure. | Personnel | H ( E4) | Fit diffuser onto flooding valves to prevent suction injury. Supervisor to ensure all necessary steps have been taken to remove pressure prior to dive. Open any pressure relief plug or valve slowly to ensure no pressure differential exists Stay to one side of blind flange Good umbilical management No loose items or body parts near diffuser After valve is opened, no more work to be performed near diffuser until riser / spool is flooded - 1,3,4,5,8,9,11,12 | M (C3) |
| Back off nuts until flange is apart. | Striking hand with hammer. Hand injury. | Personnel | M (C2) | Flanges pre-loosened to prevent such an injury. - 1,3,4,5,8,9,11,12 | L (C2) |
| Remove blind flange and lower to seabed. | Dropping blind flange. Foot injury. | Personnel | H (C3) | Ensure lift bag/rigging has the weight before releasing the last bolt. Ensure lift bag is firmly secured with hold back. - 1,3,4,5,8,9,11,12 | L (C2) |
| Stow blind flange in basket for later recovery. | Nipping trauma. Finger injury. | Personnel | M (C2) | Ensure the blind is placed into the basket in a controlled manner. - 1,3,4,5,8,9,11,12 | L (C2) |
| Recover blind flange to surface. | Flange gets suction in mud. Flange becomes fouled Blind strikes diver. Uncontrollable load over deck | Personnel | H (C3) | Assist lifting of flange using tools to break suction. Good instructions to topside to guide flange out or away from structure and diver. Diver to keep clear of lift and rigging. Fit tag lines to load before leaving bottom - 1,3,4,5,8,9,11,12 | L (C2) |

| ACTIVITY : Riser Clamp Installation | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 123 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Diver/Deck crew briefing. | Unsafe acts/conditions during crane ops that may contribute to an incident. Diver/deck crew injury. | Personnel Damage Project Schedule | H (C3) | - Brief all personnel on rigging procedures. - Be aware of vessel movements. - Procedures issues. - Good communications. Tag lines used. Refer also to JSA# 108 Overboarding and Deploying equipment. - 1,2,4,5,9,11,12 | L (C2) |
| Check load rigging. | - Rigging failure. - Unstable load. Fouled rigging. | Personnel Damage Project Schedule | H (C3) | - Only certified rigging to be used. - Inspect rigging/load before lift. Perform test lift before overboarding. - 1,2,4,5,9,10,11,12 | L (C2) |
| Position clamp for diver. | - Poor visibility for diver. - Diver entanglement. Impact by load. | Personnel Damage | M (C2) | - ROV to locate clamp if possible. - Diver sights clamp. - Clamp positioned under diver’s direction. - Use tag lines on clamp. Use visual aids/light sticks on clamp. No topside rigging or lifts while diver is in water. Utilise hogging gear or make work platform to make job easier. - 1,2,4,5,9,11,12 | L (C2) |
| Alignment of clamp on member/alignment of clamp with platform batter. | - Diver entanglement. - Movement of clamp. Entrapment of umbilical | Personnel Damage | H (C3) | - Tag lines to be used. - Good communications to deck crew /crane driver. Tender to monitor diver umbilical. Diver not to move his body between clamp and member whilst installing it. . Try to transfer load to platform tugger. - 1,2,4,5,9,11,12 | L (C2) |
| Closing clamp ½ shell | - Entrapment of diver’s umbilical. - Entrapment of diver. - Rigging failure | Personnel Project Schedule | M (C3) | - Firmly tend divers’ umbilical. - Good communications. Correct rigging procedures. Be aware of snatching rigging especially when closing gates. - 1,2,4,5,9,11,12 | L (C2) |
| Installing and tightening bolts. | Trap fingers during use of hydraulic impact wrench. | Personnel | M (C3) | - Correct use of impact wrench Good communications with diver. Hands clear of moving tools. - 1,2,4,5,9,11,12 | L (B1) |

| ACTIVITY : Riser Installation | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 124 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| During Installation. | - Crush Injury. - Impact Injury. Umbilical entrapment | Personnel Damage | H (E5) | - Do not go under load or between riser. - Good communications. - Firmly tend divers’ umbilical. - Be aware of riser movements. Move riser on diver directions. Use of ROV to monitor riser. Refer also to JSA# 108 Overboarding and Deploying equipment. - 1,3,5,9,11,12 | M (C3) |
| Installing Riser into clamps. | - Crush Injury. - Impact Injury. - Umbilical entanglement. | Personnel | H (E4) | - Good communications. - Keep away from pinch points. - Keep umbilical firmly tended. - Diver to keep vigilant view of umbilical. - Slow and easy crane movements. Keep hands and body away between riser and clamp. - 1,3,5,9,11,12 | L (C2) |
| Removing Knee brace. | - Spring or Crush Injury. Umbilical entanglement (can be difficult to maneuver due to shape and half shells, watch out for whip action when removing) | Personnel | H (E5) | - Good communications. - Undo bottom knee brace clamp first. - Leave two bolts in top clamp. - Take up weight on crane to remove last two bolts. - Slow and easy crane movements. Swing crane away from riser and diver. Be aware when undoing clamp bolts that brace may slip. - 1,3,5,9,11,12 | L (C2) |
| Removing rigging and Lifting Clamps | Umbilical entrapment. | Personnel | M (C3) | - Good communications. - Diver to give all instructions for crane or tugger movements. - Keep hand away from pinch points. - Firmly tend divers’ umbilical. - Easy controlled crane movements. Be aware of sea swell movements. Watch out for whip action when sling or clamp comes free. - 1,3,5,9,11,12 | L (C2) |

| ACTIVITY : Spool Installation | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 125 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Lower spool into position. | - Crush Injury. - Impact Injury. - Damage to flanges. | Personnel Damage | H (E5) | - Rig up spool with correct slings with SWL. - Use correct shackles with SWL. - Good communications with deck crew and crane operator. - Be aware of swell and sea movements. - Do not stand under the load. - Cover and protect flanges. Ensure protection on flanges will not cause a vacuum – Vent holes for free-flooding. Beacons at each end & ROV to monitor. Beacons tied behind flanges for protection. Refer also to JSA# 108 Overboarding and Deploying equipment. Keep spool away from any lift bags, existing pipelines and subsea structures. - 1,3,5,9,10,11,12 | L (C2) |
| Alignment of Spool | - Crush Injury. - Impact Injury. Umbilical entanglement | Personnel Damage | H (E4) | - Good communications to deck and crane operator. - Firmly tend diver’s umbilical. - Use night aids at night or in bad visibility. - Be aware of sea movement or swell. - Diver to be aware of his umbilical position. Supervisor to enforce good umbilical management techniques. Do not go under the load. Use sufficient rigging ( lacing wires, flange pullers, ‘A’ Frames, come-a-longs ) to maintain control of spool. - 1,3,5,9,10,11,12 | L (C2) |
| Tightening Flange Bolts | - Crush Injury. Umbilical entanglement. | Personnel | M (C2) | - Refer to JSA# 118 Hydratighting. - Refer to JSA# 117 Hydraulic Tools. - Caution when using tuggers. - Be aware of barge/vessel movements. Use aids for pulling flanges together. - 1,3,5,9,10,11,12 | L (C1) |
| Derigging Spool. | - Fouled umbilical. - Pulling diver off bottom. Damage to spool. | Personnel Damage | H (E4) | - Good communications to deck crew and crane operator. - Firmly tend diver’s umbilical. - Easy controlled crane movements. - Crane directions to be directed by the diver. Diver to watch slings whilst being recovered. - 1,3,5,9,10,11,12 | L (C1) |

- NOTE: - If a plywood protection cap is used on the flanges whilst lowering into position, ensure that holes are drilled in each end large enough to flood the spool. If not, a serious accident or fatality may occur when the diver removes the protection.

| ACTIVITY : Metrology | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 126 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Set Up | Disorientation. Fouled Umbilical | Personnel Damage | M (C3) | Use beacon and or compass to get to both ends of job site. Be well briefed before going in water as to where you are supposed to be going. Good tending, Avoid going around seabed obstructions that may trap umbilical. - 1,2,5,9,11,12 | L (B1) |
| Lower Table | Dropped Table | Personnel Damage | H (E4) | Refer also to JSA# 108 Overboarding and Deploying equipment. Deck crew to ensure suitable method and rigging used to get table to diver. Good comms between all concerned. - 1,2,5,9,11,12 | L (C2) |
| Take Metrology | Fouling, Disorientation, Cuts/Abrasions. | Personnel Project Schedule | H (E4) | Good tending, clear path of travel. Use beacon if no swimline available. Wear good boots and gloves, be aware of seabed obstructions. Take time to travel carefully in low viz. Report any cuts/abrasions to Supv. Avoid marine life if possible. All measurements to be confirmed by both divers independently. - 1,2,5,9,11,12 | L (C2) |
| Derig Table and Recover | Fouling. Dropped Table. | Personnel Damage | H (E4) | - Good communications to deck crew and crane operator. - Firmly tend diver’s umbilical. - Easy controlled crane movements. - Crane directions to be directed by the diver. Diver to watch slings whilst being recovered and monitor umbilical position. - 1,2,5,9,11,12 | L (C2) |

| ACTIVITY : Grout Bag Installation and Filling | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 127 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Lower Grout Bags | - Lowering bags too fast. - Entanglement with diver’s umbilical. | Personnel | H (E4) | - Have diver move away from down line when lowering. - Use night aids at night or in bad visibility. - Good communications to diver and deck crew. - Use tag lines from bags to downline. Be aware of sea conditions. Refer also to JSA# 108 Overboarding and Deploying equipment. - 1,3,4,5,9,10,11,12 | L (C2) |
| Installing Grout Bags | Entrapment. | Personnel | M (C3) | - Good communications with diver. - Firmly tend diver’s umbilical. Diver constantly monitors his umbilical and location of load. Diver to wear stout gloves during handling of bags. Diver to be requested to show umbilical clear at regular intervals by diving Supervisor. Diving supervisor to constantly monitor assembly/placement of bags to ensure bags are placed as planned. Maintain a 45º angle on stacked bags. - 1,3,4,5,9,10,11,12 | L (C2) |
| Installing pre formed bag under pipeline | Entrapment. Diver’s umbilical fouled. Unable to recover diver | Personnel | M (C3) | - Good communications with diver. - Firmly tend diver’s umbilical. Diver constantly monitors his umbilical. - 1,3,4,5,9,10,11,12 | L (C2) |
| Lowering grout hose | Entanglement with diver’s umbilical | Personnel Damage | H (E4) | - Have diver move away from down line when lowering. - Use night aids at night or in bad visibility. Good communications with diver and deck crew. - 1,3,4,5,9,10,11,12 | L (C2) |
| Pumping Grout | Sudden movement or jerking of hose. Entrapment of umbilical Chemical burns | Personnel | H (E4) | Give diver plenty of warning when grout is on its way. Diver to make sure that nozzle fits into receptacle correctly. Tie nozzle into bag or keep hold of nozzle to ensure it does not come out. Final check that his umbilical is not under the grout bag. Pump grout at the speed the diver can handle. Do not overfill bag. Do not allow grout to get in between diving suit and skin and lodge under arms or groin. Diver to keep back to tidal flow to retain some viz while doing job. - 1,3,4,5,9,10,11,12 | L (C2) |
| Recover Grout Hose | Loss of visibility Entanglement of grout hose with divers umbilical | Personnel Damage | M (C3) | Try to get all excess grout out of hose before recovery to surface. Point nozzle downstream if possible. When clear water comes then recover. - 1,3,4,5,9,10,11,12 | L (C2) |

| ACTIVITY : Move (Sweep) Pipeline | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 128 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Send Davits Down. | Entrapment of diver. Entrapment of diving bell. - Injury to diver. - Equipment damage. - Crush Injury | Personnel Damage | H (E4) | - Lower Davits before diving operation to within 10 ft of seabed. - If bell is in the water, ensure it is well out of the way. - Diver and bellman to be alert. - Use night aids at night or in bad visibility. Good communications. If using Crane – beacon on crane wire and diver. Good comms between diver / supv. / deck foreman / crane operator / survey. Crane driver to monitor load cell - 1,3,5,8,9,11,12 | L (C2) |
| Connecting up Davits. | - Umbilical Entanglement - Umbilical caught in davit block. - Pipe movement - Injury to hands or body parts | Personnel Damage | H (E4) | - Keep divers umbilical firmly tended. - Good communications. Diver to be aware of his umbilical at all times Connect to pull head whenever possible. - 1,3,5,8,9,11,12 | L (C2) |
| Moving Pipeline | - Umbilical caught in davit block. - Entrapment of bell. - Moving pipeline too fast. Davit down too far and on seabed with excess wire Diver losing contact with pipeline. | Personnel Damage | H (E5) | - Good communications. - Diver to give precise movements of pipe. - Slow and easy movements. - Bellman to be aware of all movements. - Firmly tend divers’ umbilical. Diver to ensure he is in good position and holding on before move is initiated. Make slow movements, have the diver at the point where the pipeline touches natural bottom, with his hands on the pipe. Follow movement and report what is happening. - 1,3,5,8,9,11,12 | M (C3) |
| Lay Pipeline Down | Umbilical under pipeline. Diver under pipeline - Crush Injury. - Injury to diver. Loss of divers gas supplies | Personnel | H (E5) | - Good communications. - Firmly tend divers’ umbilical. - Slow vessel or barge movements. Deck crew to monitor davit depths. - 1,3,5,8,9,11,12 | M (C3) |
| Disconnect Davits. | Unplanned movements. Davit wires parting. Pipe movement. Crush/Impact Injury. Diver being pulled off pipeline by davit block. Fingers/Body parts being caught. Umbilical being caught. | Personnel Damage | H (E4) | - Good communications. - Diver to let deck crew know what davit he is working on. - Slow and easy davit movements. - Diver to watch wire sling being pulled through. - Diver to be aware of his umbilical. - Be aware of vessel or barge movements. - Diving Supervisor not to give long and rambling instructions whilst davit movement in progress. - Especially in Black water or bad visibility, diver not to give instructions too quickly ‘Clear to Surface’ Deck crew to watch weight indicators for each davit when picking up. - 1,3,5,8,9,11,12 | L (C2) |

| ACTIVITY : Remove (or Install) Start-Up / Lay-Down Head | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 129 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Lowering or Recovering. | - Rigging not rated for job. - Diver getting fouled. - Losing the load. - Injury to diver or personnel | Personnel Damage | H (E5) | Use correctly rated slings and shackles with SWL for job. Good communications. Tend divers umbilical firmly. Use night aids at night or in poor visibility. Keep clear from under load when lowering or recovering. Refer also to JSA# 108 Overboarding and Deploying Equipment. - 1,2,5,9,11,12 | M (C3) |
| Removing/Installing. | Crush or Impact hazard. Hand, fingers or body parts entrapment Possibility of water still flowing into head valves. Head falls on diver if unsupported | Personnel Damage Project Schedule | H (E5) | Loosen all bolts enough to open up o-ring groove to see if pressure is present. Be aware that line could be dry, so possible suction. Check for suction by waving a two foot long piece of rope near flange. Be careful to keep hands clear. Use good tending techniques to keep divers hose from getting fouled. Use good hose management techniques to keep hose clear. Good communications with deck crew and crane operator. Slow and easy movements with crane. Be aware of vessel/barge movement. Check no flow with ropes over open valve inlets. Do not commence work unless all valves are secured/blinded or a hose or diffuser fitted. Ensure head is either supported on sandbags or by crane/liftbag prior to removing last bolt. NB. Ensure last bolt is in 12-o-clock position.. Supervision and awareness - 1,2,5,9,11,12 | M (C3) |
| Removing Flange Bolts | Possibility of diver striking hand with flogging hammer or striking face plate Possibility of strain when using heavy impact wrench | Personnel Damage | H (E4) | Exercise caution when using tools. Use floatation device to lessen weight of wrench. Reduce weight of tool. Follow manual handling procedures - 1,2,5,9,11,12 | L (C2) |
| Recovering to surface | - Impact or crush hazard. - Umbilical entrapment. | Personnel Damage | H (E4) | Have diver move away from load before recovering. Check pigs are secured and tied in. Slow and easy crane movements. Tend diver’s umbilical firmly. - 1,2,5,9,11,12 | M (C3) |

| ACTIVITY : Pigging Pipeline | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 130 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Lower Pigging Hoses to Diver | Dropped loads, freefalling crane, inadequate rigging. | Personnel Damage | H (E4) | Refer JSA# 108 on Lowering Equipment To Diver. Good comms between diver, Supv, D/F and crane op. Rig hoses for good angle to facilitate easy handling by diver. - 1,2,5,8,9,11,12 | L (C2) |
| Attach pigging hose to Cam-Lock connectors or other connector and valve operation | Disorientation, Diver being sucked into valve. Entanglement of umbilical Strain injuries. Pinch/crush injuries. | Personnel | H (E5) | - Use survey beacon to assist in being guided to job if low viz. Diver to avoid walking umbilical around obstructions that may foul umbilical. - Thoroughly under stand what has to be done. - Good communications. - Tend divers’ umbilical firmly. - Ensure the valve is closed; it may be capped off or plugged. - Remove all handles from valves before connecting hoses. - When using Cam-Loc fittings, ensure properly moused. - When pigging hose connected, tie off hose to frame or launcher. Only open valve on diving supervisor’s instructions Use diffusers on 2” flood valves. Ensure no loose gloves or dive gear can get sucked into diffuser. - 1,2,5,8,9,11,12 | M (C3) |
| Cycle Valves | Noise. Hose Movement. Hose Burst. Hose Collapse | Personnel Damage | H (E4) | It will be noisy when pumps are running. Understand what you are expected to do next and only do that. Move along pipe to area where hearing is better to receive next instruction if necessary. Be aware of hoses ‘kicking’ when pressure up by pumps. Ensure they are tied to job with good rope. Be aware of possible hose burst, hoses to be checked on surface prior to use. Hoses may collapse on opening pigging head valve. Keep body parts and umbilical clear of hose. - 1,2,5,8,9,11,12 | L (C2) |
| Derig Hoses | Suction/Pressure Pinch/Crush Strain. Fouled Umbilical, Pulled Off Bottom. | Personnel | H (E4) | Ensure all valves closed to hoses before loosening fittings. As above for connecting hoses. Ensure umbilical is all clear of hoses. If in sat, ensure hoses well clear of bell. - 1,2,5,8,9,11,12 | L (C2) |

Note: - Not all pipelines will be wet during pigging operations. On dry pipelines, extreme caution must be taken when connecting hoses and valves cannot be opened by mistake or accidentally.

| ACTIVITY : Hydrotesting | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 131 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Test spread assembly. | Rigging failure. Dropped object. Chemical hazard. | Personnel Damage | M (C3) | Use only certified rigging. Pre-task briefing to be held. PPE to be worn. Appropriate PTW issued. Remote siting of monitoring equipment - test gauges, chart recorder etc. - 1,2,4,5,6,8,9,10,11,12 | L (C1) |
| Test spread operational check | Equipment failure. Hose failure. | Personnel Damage | M (C3) | - PPE to be worn. - Warning sign to be displayed. - Barriers to be erected. - Equipment guards to be in place. - Area to be well ventilated. - RCD's to be used where appropriate. - Hose checks fitted Restrict access to test equipment area. Notify the Bridge. - 1,2,4,5,6,8,9,10,11,12 | L (C1) |
| Pressurising. | Equipment failure. Chemical hazard. HP substance hazard | Personnel Damage | H (E4) | - PPE to be worn. - Warning signs erected. - Barriers erected. - Machinery guards in place. - Increase pressure slowly. - Hose checks fitted. Warning broadcasts made. - 1,2,4,5,6,8,9,10,11,12 | L (C2) |
| Monitoring | Dangers associated with hoses and fittings under pressure | Personnel Damage | H (E4) | - PPE to be worn. - Warning signs erected. - Barriers / shields erected. - Machinery guards in place. - Hose checks fitted. - Warning broadcasts made. - No tightening of fittings under pressure - 1,2,4,5,6,8,9,10,11,12 | L (C2) |
| Depressurising. | High-pressure substance impact danger. Equipment failure whilst under pressure | Personnel Damage | H (E4) | - PPE to be worn. - Hoses vented in a safe direction. - Hoses to be secured during venting. Isolation to be established before venting Ensure no personnel are in the path of the water being released. - 1,2,4,5,6,8,9,10,11,12 | L (C2) |
| Dismantling equipment. | Dangers associated with breaking out equipment whilst under pressure. Impact injury. Slip, trips or falls. | Personnel Damage | M (C3) | - PPE to be worn. - In date rigging to be used. - Pre-task briefing to be held. System flushed with fresh water before dismantling. - 1,2,4,5,6,8,9,10,11,12 | L (C1) |

| ACTIVITY : Cutting and Removing Dummy Spools | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 132 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Burning vent hole at 12 o/c on dummy spool | Pipeline not flooded or still under pressure | Personnel | H (E5) | Refer to JSA# 120 – Oxy Arc Cutting. No work on pipeline to go ahead until it has confirmed to the Dive Superintendent that the Pipeline has been flooded, & de pressurised. When the first hole is cut the diver is to keep as far as practical away, & if there is any pressure differential to stop work & allow the pressure to equalise. - 1,2,5,9,11,12 | M (C3) |
| Cutting the dummy spool using Broco oxy arc | Blow back of entrapped oxygen, pipeline springing apart due to tension in line | Personnel Damage | H (E4) | The area to be cut to be completely cleared of mud/ sand so that there is a minimum 6 inches of clearance around the area. A large vent window to be cut at 12 o/c to allow the oxygen to escape. After the vent hole is cut the next area to be cut will be the bottom of the pipe, two areas from 8 o/c to 10 o/c & 4 o/c to 2 o/c will not be cut until last. When the last two areas are cut the diver will stand to one side of the pipeline so that if one piece moves he is clear. - 1,2,5,9,11,12 | L (C2) |
| Rig the crane to the cut sections | Incorrectly rigged leaving unbalanced load. | Personnel Damage | M (C3) | Pre determined lifting points to be marked out & lifting strops attached at marked, measured locations. These will be checked with divers beacon. - 1,2,5,9,11,12 | L (C2) |
| Lift the load off the seabed | Load fouling on jagged cut | Personnel Project Schedule | M (C3) | At the first cut location a band will be removed of approximated 6 inches to prevent the first piece of the dummy spool from hanging up - 1,2,5,9,11,12 | L (C1) |
| Lift the load up to the side of the vessel | Load slipping/falling out & hitting bell. | Personnel Damage | M (C3) | Refer to JSA# 108 – Overboarding and Deploying Equipment. Bell to be recovered to surface prior to recovery of the cut section. - 1,2,5,9,11,12 | L (C1) |

| ACTIVITY : TIDES AND CURRENTS | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 133 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Diving Operations | - Possible entrapment of diver resulting in inability to return to bell or stage - Standby diver unable to reach diver | - Personnel | H (D4) | - Careful dive planning - Don’t leave it too late to recover the diver - Use of swim lines - Awareness of sheltering effects of subsea structures that may mask tidal run - Use of tidal gauge - Increased vigilance of all involved - Incorporate precautions in project procedures for known tidal areas 1,2,5,9,11,12 | M C2) |

| ACTIVITY : POOR SUBSEA VISIBILITY | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 134 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Diving Operations | - Bad subsea visibility posing hazard to diver - Entanglement or entrapment of divers or umbilical or of being struck by crane load etc. | - Personnel - Assets | H (D4) | - Good dive planning - Experienced dive team - Vigilant umbilical tending - Use of swim lines - Use of strobes/beacons on crane lines - Minimise downlines and overboard hoses - Recover unwanted equipment ASAP from the seabed - Consider HPR beacon on diver - Postpone operation if necessaary 1,2,5,9,11,12 | M ( C2) |

| ACTIVITY : FLARE BOOM RADIATION | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 135 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Diving Operations | - Excessive heat from flare boom causing hazard to personnel or equipment - Injury to deck personnel - Material damage by radiated heat. | - Personnel - Assets | H (D4) | - Position vessel with due regard to radiation from flare - Use fire hoses to cool O2 quads and decks - Keep downlines to a minimum - Shutdown flare - Move vessel to position away or upwind from flare - Precautions to be incorporated in project procedures 1,2,4,5,8,9,11,12 | M ( C2) |

| ACTIVITY : CONCRETE COATING REMOVAL | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 136 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Removal of concrete weight coat from a pipeline | - Injury to diver - Possible damage to pipeline | - Personnel - Assets | H (D4) | - Double check setting on guards - Ensure pre-sets on saws - Deadmans handle - Training on equipment 1,2,,5,,9,11,12 | M ( C2) |

| ACTIVITY : STINGER CHECKS | DATE : Reviewed by: |  | PROJECT : Vessel Generic |
| --- | --- | --- | --- |
| JSA# 137 | LOCATION : |  | REF PROCEDURES: |
| SAFETY – FIRST PRIORITY |  |  |  |

| Basic/ General Safety Requirements |  | Project Specific |  |
| --- | --- | --- | --- |
| 1. APPROPRIATE PPE | 5. PERMIT TO WORK | 9. SUPERVISION | 13. |
| 2. TASK SPECIFIC PROCEDURE | 6. MECHANICAL ISOLATION | 10. CERTIFICATION | 14. |
| 3. SPECIFIC TRAINING | 7. ELECTRICAL ISOLATION | 11. TOOL BOX TALKS | 15. |
| 4. RESTRICT WORK AREA ACCESS | 8. POST BARRIERS / SIGNS | 12. COMPETENT PERSON | 16. |

| Step 1 Task/Activity Description | Step 2 Hazard | Step 3 Who or What Exposed | Step 4 Quantify Risk | Step 5 Recommended Corrective Actions / Control Measures | Step 6 Residual Risk |
| --- | --- | --- | --- | --- | --- |
| Communications Check to Tower, Dive Supv, Barge Foreman and Stinger Control | - Communications breakdown. - Injury to personnel. Equipment damage | Personnel Project Downtime | H (E4) | - Hard wire telephone system to Tower, Diving Supervisor, Stinger Control - Emergency procedures in place. - Stand by diver fully dressed. - Post this JSA in a conspicuous place for all relevant personnel to refer to. 1,2,3,4,5,6,9,11,12 | L (B1) |
| Diver checks stinger. | - Umbilical caught in rollers. - Umbilical caught in hitch area. - Umbilical caught in stinger connections at sections. - Umbilical caught on pipeline. - Fingers caught in pinch points on stinger sections. - Swimming under and over stinger. - Current and sea conditions Suction on flood valves. | - Entrapment of diver. - Entrapment of diver. - Crushed umbilical and loss of gas. - Diver being dragged with pipeline. - Injury to diver. - May result in fouled umbilical. - Diver being swept away or injury in splash zone. Divers fingers / equipment being caught | H (E4) | - Tower must be informed at all times when divers enter the water & back on surface. For a stinger check or dive whilst on pipe laying operations. - Tower will not move pipe line in any circumstances during the dive, unless instructed to do so by only dive supervisor - Stay outside of stinger at all times. - Use hand held guide wire on the stinger. - Good communications. - Good tender management and positioning. - Diver to be informed if pipe is going to be pulled. Diffusers to be fitted to 2” flood valves before operating. - Emergency procedures in place. 1,2,3,4,5,6,9,11,12 | M (B3) |

Note: It is normal practice to plan the dive to coincide between pipe pulling operations. The supervisor informs tower of diver in water and the barge will not move under any circumstances during the diver in water operation, unless instructed to do so by only the dive supervisor.