Global Divers & Contractors

Global Industries, Ltd.

METROLOGY PROCEDURES

GP 00003

| Revision | Description | Date | Prepared | Approved |
| --- | --- | --- | --- | --- |
| Draft | Issued for comment | 8/17/98 | LB | MS |

CONTENTS

1.0 INTRODUCTION

2.0 SUMMARY OF EQUIPMENT

2.1 Protractors

3.0 PIPELINE PROTRACTOR PROCEDURE, DUAL ANGLE

3.1 Mounting Flanges

3.2 Installation Procedures

4.0 PIPELINE PROCEDURE, SINGLE ANGLE

4.1 General	7

4.2 Installation Procedures	7

5.0 SPOOL PIECE FABRICATION	9

5.1 General	9
5.2 Fabrication Procedures	9

# 1.0	Introduction

This procedure has been written to summarize the correct usage of the two (2) protractors which Global Divers uses for obtaining information for the fabrication of underwater tie-in spools.  Protractors are designed for single angle (horizontal) and dual angle (horizontal and vertical) measurements.

# 2.0	Summary of Equipment

## 2.1	Protractors

Global Divers has two (2) Protractors that have been designed for the measurement of pipelines with  flanges ranging from 4” to 36” and pressure ratings of 600 and 900 #’s.  The pair of protractors is the most diverse in its ability to supply the degree of offset in the horizontal and vertical plane and spool length to obtain the most accurate measurement data. Operation of the protractor will be discussed in detail later in this procedure.

The second type of protractor has two(2) jigs with saddles which can be strapped on to pipelines from 2” to 24” in diameter.  A measurement tape is on one and a hook to capture the tape on the second.  This tool was primarily designed to determine the misalignment of pipelines with single dog leg bends at the same vertical elevation.  Detailed instructions of the method by which it is used will be discussed later in this procedure.

# 3.0 Pipeline Protractor Procedure, Dual Angle

## 3.1	Mounting Flanges

Prior to the start of any measurement activity, it is first necessary to know the size of the flange and its pressure rating.  There are specific adapter plates that have been designed to fit the various sizes and pressure ratings of flanges.  Make sure you have the appropriate flange adapter for the size and rating of the flanges you want to measure.

## 3.2	Installation Procedures

1.	Mount a flange adapter to each pipeline flange.   Position the adapter with the set screw on the right (three o’clock position) when facing the flange.

2.	Thread each tool onto a flange adapter and rotate until it bottoms out.

Back off until tool is level (use bubble level).  The winch mounting holes

will be on the left side of the tool.

3.	Lock  the set screws.

4.	Set bolt hole locators in-line with uppermost bolt or bolt hole on flanges and lock knob in place.

5.	Mount winch assembly to tool at Station 1.

6.	Spool out cable and thread through eye bolt and cursor.  Capture cable in cursor with cable keeper plate.

7.	Loosen protractor locking knobs on cursor on both tools.

8.	Extend cable to tool at Station 2 and thread through cursor.  Capture cable in cursor with cable keeper plate.

9.	Attach loop in end of cable to hook.

10.	Tighten cable.

11.	Tighten protractor locking knobs on cursor of both tools and report reading topside, if possible.

12.	Position cable marker (brass clip) on cable flush with top of cursor on both tools.  Crimp in place with pliers.

13.	Disassemble equipment from flanges and return to surface.  Use caution not to upset protractor settings.

14.	Record vertical (small ring) and horizontal (large ring) angle readings for both Stations No. 1 and No. 2.

15.	Mount the winch to a stationary object; i.e., a structure on the deck of the

vessel.  Extend the cable to expose both crimps and attach the loop in

the end of the cable to a hook, bolt, or peg, on the stationary object.

16.	Tighten the cable.

17.	Measure the distance between the crimps on  the cable and record the

measurement.

18.	If assembly must be two holed, record bolt hole angles for Stations No. 1 and No. 2.  Indicate if angle is clockwise or counterclockwise from “0” (top dead center).

# 4.0	Pipeline Protractor Procedure, Single Angle

## 4.1	General

This procedure shall be used in the measurement of a spool piece with a single bend or elbow and a run of pipe on each side of the bend. This tool was primarily designed to determine the misalignment of pipelines with single dog leg bends at the same vertical elevation. These instructions are used for providing a tie-in spool containing two straight sections and one elbow.

## 4.2	Installation Procedures

1.	The metrology jig is designed for horizontal pipelines laying on natural bottom.  If one or more of the pipelines is buried then the jigs will need to be modified to bring tables to same elevation.  A “T” bar of proper/desired  height tack welded to a line up clamp works well.  Care must be taken to confirm the “T” bar in perfectly in line with pipeline.

2.	Install jigs on pipeline checking level indicator to confirm verticality.  All measurements are based on protractor center point therefore place as close as possible to desired measuring point. (ie flange face)  If the jig is installed on “T” bars an plumb bob from the center point should be used for line up with flange.  If the jig will not fit with center point at desired location then the offset distance must be measured and later subtracted from spool length.

3.	Pull the tape measure from jig 1 and attach to jig 2.  Tighten tape measure, and check tape for hangs.  The Jigs may now be read.  When reading the diver should be behind the jig facing down the tape measure to the other jig and read the degrees  LEFT or RIGHT of center.  The jigs should always read opposite each other, if not there is an orientation problem and pipeline may require moving.  If one protractor reads near zero there may not be enough room for the bend, also requiring the pipeline to be moved.   The tape measure shall be read center point to center point.  When in no visibility water, clips may be attached and jig retrieved.

4.	Using graph paper lay out pipelines and flanges to confirm 	desired orientation.

5.	Calculate spool distances from WP of bend to center point of jigs.  	Subtract offsets as necessary and fabricate.  Jig may be used 	during fabrication to confirm lengths and angles.

# Spool Piece Fabrication

## 5.1      General

Materials must be addressed prior to mobilization.  Generally a 90 degree bend is supplied to be utilized as the required bend angle.  Therefore this bend must be “segmentable” i.e. it must have the required ovality throughout the segment to be able to be welded to a round pipe.  If 5D bend is required bending contractor must assure ovality with in required tolerance to aleviate any high-low during fit up.  In some cases a 10D bend may used to hold these tolerances.  In cases where ovality is a problem a “Dearman” clamp may be used during fit up.

  - Fabrication Procedure

- Complete calculations as shown in drawing.

- Lay straight edge on bends and mark pipe at tangent.  (The point where pipe bends away from straight edge.

- Measure pipe along out side of bend between tangents.  Divide measurement by bend angle, (usually 90 degrees) and multiply answer by required angle.  Measure and mark bend accordingly.   Repeat process measuring along inside of bend.

- Rough cut bend leaving approximately ½” to 1” of excess.

- Tack weld straight pipe on to bend.  This will provide a jig to mount beveling machine to produce a straight cut at required angle.

- Set beveling machine and cut pipe bend.

- Lay bend flat on deck.  Set straight edges on top of pipe square to cut and measure to Working Point.  (WP, intersection of straight edges)

- Cut spools with flanges to required lengths to complete each leg.

- Tack weld spool together.  Install saddle type Metrology jig to check fabrication.

- Complete welding of spool.