Table 1-10 STANDARD AIR WITH NITROX OR AIR DECOMPRESSION. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 100; 90; 80; 70; 60; 50; 40; 30; 20; 10. 200; 0:00; 0; 0:40; *. 210; 0:30; 2; 2:40; N. 230; 0:30; 7; 7:40; N. 250; 0:30; 11; 11:40; O. 270; 0:30; 15; 15:40; O. 300; 0:30; 19; 19:40; Z. SUPERIOR OPERATIONAL DEPTH / TIME LIMITS. 360; 0:30; 23; 23:40. 480; 0:30; 41; 41:40. 720; 0:30; 69; 69:40. DIVE PROCEDURES – TABLE 1-10 Modified in Water Decompression Tables with Nitrox or Air in Water Stops. IN WATER PHASE; * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours. * Ascent procedures; - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - 60 feet per min to the indicated stop or surface if no stops. - Decompression time as specified begins on arrival at stop depth; - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Do not include ascent time as part of stop times; - Refer to Table 1-12 for new RGD at the end of the surface interval. - Delays at earlier stops are not to be compensated for at later water stops; - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT). *50/50 Nitrox Switch; LOSS OF NITROX. - Switch to 50/50 Nitrox at 100 feet during ascent; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive. - Umbilical flush is to be formally logged. 40 FEET. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-10 STANDARD AIR WITH NITROX OR AIR DECOMPRESSION. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 100; 90; 80; 70; 60; 50; 40; 30; 20; 10. 100; 0:00; 0; 0:50; *. 110; 0:40; 3; 3:50; L. 120; 0:40; 5; 5:50; M. 140; 0:40; 10; 10:50; M. 160; 0:40; 21; 21:50; N. 180; 0:40; 29; 29:50; O. 200; 0:40; 35; 35:50; O. 220; 0:40; 40; 40:50; Z. SUPERIOR OPERATIONAL DEPTH / TIME LIMITS. 240; 0:40; 47; 47:50. DIVE PROCEDURES – TABLE 1-10 Modified in Water Decompression Tables with Nitrox or Air in Water Stops. IN WATER PHASE; * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours. * Ascent procedures; - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - 60 feet per min to the indicated stop or surface if no stops. - Decompression time as specified begins on arrival at stop depth; - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Do not include ascent time as part of stop times; - Refer to Table 1-12 for new RGD at the end of the surface interval. - Delays at earlier stops are not to be compensated for at later water stops; - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT). *50/50 Nitrox Switch; LOSS OF NITROX. - Switch to 50/50 Nitrox at 100 feet during ascent; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive. - Umbilical flush is to be formally logged. 50 FEET. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-10 STANDARD AIR WITH NITROX OR AIR DECOMPRESSION. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 100; 90; 80; 70; 60; 50; 40; 30; 20; 10. 60; 0:00; 0; 1:00; *. 70; 0:50; 2; 3:00; K. 80; 0:50; 7; 8:00; L. 100; 0:50; 14; 15:00; M. 120; 0:50; 26; 27:00; N. 140; 0:50; 39; 40:00; O. 160; 0:50; 48; 49:00; Z. SUPERIOR OPERATIONAL DEPTH / TIME LIMITS. 180; 0:50; 56; 57:00. 200; 0:40; 1; 69; 71:00. 240; 0:40; 2; 79; 82:00. 360; 0:40; 20; 119; 140:00. 480; 0:40; 44; 148; 193:00. 720; 0:40; 78; 187; 266:00. DIVE PROCEDURES – TABLE 1-10 Modified in Water Decompression Tables with Nitrox or Air in Water Stops. IN WATER PHASE; * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours. * Ascent procedures; - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - 60 feet per min to the indicated stop or surface if no stops. - Decompression time as specified begins on arrival at stop depth; - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Do not include ascent time as part of stop times; - Refer to Table 1-12 for new RGD at the end of the surface interval. - Delays at earlier stops are not to be compensated for at later water stops; - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT). *50/50 Nitrox Switch; LOSS OF NITROX. - Switch to 50/50 Nitrox at 100 feet during ascent; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; 60 FEET. - Umbilical flush is to be formally logged. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-10 STANDARD AIR WITH NITROX OR AIR DECOMPRESSION. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 100; 90; 80; 70; 60; 50; 40; 30; 20; 10. 50; 0:00; 0; 1:10; *. 60; 1:00; 8; 9:10; K. 70; 1:00; 14; 15:10; L. 80; 1:00; 18; 19:10; M. 90; 1:00; 23; 24:10; N. 100; 1:00; 33; 34:10; N. 110; 0:50; 2; 41; 44:10; O. 120; 0:50; 4; 47; 52:10; O. 130; 0:50; 6; 52; 59:10; O. 140; 0:50; 8; 56; 65:10; Z. SUPERIOR OPERATIONAL DEPTH / TIME LIMITS. 150; 0:50; 9; 61; 71:10. 160; 0:50; 13; 72; 86:10. 170; 0:50; 19; 79; 99:10. DIVE PROCEDURES – TABLE 1-10 Modified in Water Decompression Tables with Nitrox or Air in Water Stops. IN WATER PHASE; * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours. * Ascent procedures; - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - 60 feet per min to the indicated stop or surface if no stops. - Decompression time as specified begins on arrival at stop depth; - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Do not include ascent time as part of stop times; - Refer to Table 1-12 for new RGD at the end of the surface interval. - Delays at earlier stops are not to be compensated for at later water stops; - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT). *50/50 Nitrox Switch; LOSS OF NITROX. - Switch to 50/50 Nitrox at 100 feet during ascent; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; 70 FEET. - Umbilical flush is to be formally logged. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-10 STANDARD AIR WITH NITROX OR AIR DECOMPRESSION. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 100; 90; 80; 70; 60; 50; 40; 30; 20; 10. 40; 0:00; 0; 1:20; *. 50; 1:10; 10; 11:20; K. 60; 1:10; 17; 18:20; L. 70; 1:00; 23; 24:20; M. 80; 1:00; 2; 31; 34:20; N. 90; 1:00; 7; 39; 47:20; N. 100; 1:00; 11; 46; 58:20; O. 110; 1:00; 13; 53; 67:20; O. 120; 1:00; 17; 56; 74:20; Z. SUPERIOR OPERATIONAL DEPTH / TIME LIMITS. 130; 1:00; 19; 63; 83:20. 140; 1:00; 26; 69; 96:20. 150; 1:00; 32; 77; 110:20. 180; 0:50; 35; 85; 121:20. 240; 0:50; 5; 52; 120; 179:20. 360; 0:50; 29; 90; 160; 280:20. 480; 0:50; 59; 107; 187; 354:20. 720; 0:40; 17; 108; 142; 187; 455:20. DIVE PROCEDURES – TABLE 1-10 Modified in Water Decompression Tables with Nitrox or Air in Water Stops. IN WATER PHASE; * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours. * Ascent procedures; - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - 60 feet per min to the indicated stop or surface if no stops. - Decompression time as specified begins on arrival at stop depth; - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Do not include ascent time as part of stop times; - Refer to Table 1-12 for new RGD at the end of the surface interval. - Delays at earlier stops are not to be compensated for at later water stops; - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT). *50/50 Nitrox Switch; LOSS OF NITROX. - Switch to 50/50 Nitrox at 100 feet during ascent; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; 80 FEET. - Umbilical flush is to be formally logged. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-10 STANDARD AIR WITH NITROX OR AIR DECOMPRESSION. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 100; 90; 80; 70; 60; 50; 40; 30; 20; 10. 30; 0:00; 0; 1:30; I. 40; 1:20; 7; 8:30; J. 50; 1:20; 18; 19:30; K. 60; 1:20; 25; 26:30; L. 70; 1:10; 7; 30; 38:30; M. 80; 1:10; 13; 40; 54:30; N. 90; 1:10; 18; 48; 67:30; N. 100; 1:10; 21; 54; 76:30; O. SUPERIOR OPERATIONAL DEPTH / TIME LIMITS. 110; 1:10; 24; 61; 86:30; Z. 120; 1:10; 32; 68; 101:30. 130; 1:10; 5; 36; 74; 116:30. DIVE PROCEDURES – TABLE 1-10 Modified in Water Decompression Tables with Nitrox or Air in Water Stops. IN WATER PHASE; * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours. * Ascent procedures; - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - 60 feet per min to the indicated stop or surface if no stops. - Decompression time as specified begins on arrival at stop depth; - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Do not include ascent time as part of stop times; - Refer to Table 1-12 for new RGD at the end of the surface interval. - Delays at earlier stops are not to be compensated for at later water stops; - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT). *50/50 Nitrox Switch; LOSS OF NITROX. - Switch to 50/50 Nitrox at 100 feet during ascent; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; 90 FEET. - Umbilical flush is to be formally logged. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-10 STANDARD AIR WITH NITROX OR AIR DECOMPRESSION. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 100; 90; 80; 70; 60; 50; 40; 30; 20; 10. 25; 0:00; 0; 1:40; *. 30; 1:30; 3; 4:60; I. 40; 1:30; 15; 16:40; K. 50; 1:20; 2; 24; 27:40; L. 60; 1:20; 9; 28; 38:40; N. 70; 1:20; 17; 39; 57:40; O. 80; 1:20; 23; 48; 72:00; O. 90; 1:10; 3; 23; 57; 84:40; Z. SUPERIOR OPERATIONAL DEPTH / TIME LIMITS. 100; 1:10; 7; 23; 66; 97:40. 110; 1:10; 10; 34; 72; 117:40. 120; 1:10; 12; 41; 78; 132:40. 180; 1:00; 1; 29; 53; 118; 202:40. 240; 1:00; 14; 42; 84; 142; 283:40. 360; 0:50; 2; 42; 73; 111; 187; 416:40. 480; 0:50; 21; 61; 91; 142; 187; 503:40. 720; 0:50; 55; 106; 122; 142; 187; 613:40. DIVE PROCEDURES – TABLE 1-10 Modified in Water Decompression Tables with Nitrox or Air in Water Stops. IN WATER PHASE; * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours. * Ascent procedures; - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - 60 feet per min to the indicated stop or surface if no stops. - Decompression time as specified begins on arrival at stop depth; - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Do not include ascent time as part of stop times; - Refer to Table 1-12 for new RGD at the end of the surface interval. - Delays at earlier stops are not to be compensated for at later water stops; - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT). *50/50 Nitrox Switch; LOSS OF NITROX. - Switch to 50/50 Nitrox at 100 feet during ascent; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; 100 FEET. - Umbilical flush is to be formally logged. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-10 STANDARD AIR WITH NITROX OR AIR DECOMPRESSION. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 100; 90; 80; 70; 60; 50; 40; 30; 20; 10. 20; 0:00; 0; 1:50; *. 25; 1:40; 3; 4:50; H. 30; 1:30; 7; 8:50; J. 40; 1:30; 2; 21; 24:50; L. 50; 1:30; 8; 26; 35:50; M. 60; 1:30; 18; 36; 55:50; N. 70; 1:20; 1; 23; 48; 73:50; O. 80; 1:20; 7; 23; 57; 88:50; Z. SUPERIOR OPERATIONAL DEPTH / TIME LIMITS. 90; 1:20; 12; 30; 64; 107:50. 100; 1:20; 15; 37; 72; 125:50. DIVE PROCEDURES – TABLE 1-10 Modified in Water Decompression Tables with Nitrox or Air in Water Stops. IN WATER PHASE; * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours. * Ascent procedures; - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - 60 feet per min to the indicated stop or surface if no stops. - Decompression time as specified begins on arrival at stop depth; - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Do not include ascent time as part of stop times; - Refer to Table 1-12 for new RGD at the end of the surface interval. - Delays at earlier stops are not to be compensated for at later water stops; - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT). *50/50 Nitrox Switch; LOSS OF NITROX. - Switch to 50/50 Nitrox at 100 feet during ascent; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; 110 FEET. - Umbilical flush is to be formally logged. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-10 STANDARD AIR WITH NITROX OR AIR DECOMPRESSION. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 100; 90; 80; 70; 60; 50; 40; 30; 20; 10. 15; 0:00; 0; 2:00; *. 20; 1:50; 2; 4:00; H. 25; 1:50; 6; 8:00; I. 30; 1:50; 14; 16:00; J. 40; 1:40; 5; 25; 32:00; L. 50; 1:40; 15; 31; 48:00; N. 60; 1:30; 2; 22; 45; 71:00; O. 70; 1:30; 9; 23; 55; 89:00; O. 80; 1:30; 15; 27; 63; 107:00; Z. SUPERIOR OPERATIONAL DEPTH / TIME LIMITS. 90; 1:30; 19; 37; 74; 132:00. 100; 1:30; 23; 45; 80; 150:00. 120; 1:20; 10; 19; 47; 98; 176:00. 180; 1:10; 5; 27; 37; 76; 137; 284:00. 240; 1:10; 23; 35; 60; 97; 179; 396:00. DIVE PROCEDURES – TABLE 1-10 Modified in Water Decompression Tables with Nitrox or Air in Water Stops. IN WATER PHASE; * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours. * Ascent procedures; - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - 60 feet per min to the indicated stop or surface if no stops. - Decompression time as specified begins on arrival at stop depth; - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Do not include ascent time as part of stop times; - Refer to Table 1-12 for new RGD at the end of the surface interval. - Delays at earlier stops are not to be compensated for at later water stops; - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT). *50/50 Nitrox Switch; LOSS OF NITROX. - Switch to 50/50 Nitrox at 100 feet during ascent; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; 120 FEET. - Umbilical flush is to be formally logged. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-10 STANDARD AIR WITH NITROX OR AIR DECOMPRESSION. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 100; 90; 80; 70; 60; 50; 40; 30; 20; 10. 10; 0:00; 0; 2:10; *. 15; 2:00; 1; 3:10; F. 20; 2:00; 4; 6:10; H. 25; 2:00; 10; 12:10; J. 30; 1:50; 3; 18; 23:19; M. 40; 1:50; 10; 25; 37:10; N. 50; 1:40; 3; 21; 37; 63:10; O. 60; 1:40; 9; 23; 52; 86:10; Z. SUPERIOR OPERATIONAL DEPTH / TIME LIMITS. 70; 1:40; 16; 24; 61; 103:10. 80; 1:30; 3; 19; 35; 72; 131:10. 90; 1:30; 8; 19; 45; 80; 154:10. DIVE PROCEDURES – TABLE 1-10 Modified in Water Decompression Tables with Nitrox or Air in Water Stops. IN WATER PHASE; * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours. * Ascent procedures; - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - 60 feet per min to the indicated stop or surface if no stops. - Decompression time as specified begins on arrival at stop depth; - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Do not include ascent time as part of stop times; - Refer to Table 1-12 for new RGD at the end of the surface interval. - Delays at earlier stops are not to be compensated for at later water stops; - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT). *50/50 Nitrox Switch; LOSS OF NITROX. - Switch to 50/50 Nitrox at 100 feet during ascent; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; 130 FEET. - Umbilical flush is to be formally logged. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-10 STANDARD AIR WITH NITROX OR AIR DECOMPRESSION. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 100; 90; 80; 70; 60; 50; 40; 30; 20; 10. 10; 0:00; 0; 2:20; *. 15; 2:10; 2; 4:20; G. 20; 2:10; 6; 8:20; I. 25; 2:00; 2; 14; 18:20; J. 30; 2:00; 5; 21; 28:20; K. 40; 1:50; 2; 16; 26; 46:20; N. 50; 1:50; 6; 24; 44; 76:20; O. 60; 1:50; 16; 23; 56; 97:20; Z. SUPERIOR OPERATIONAL DEPTH / TIME LIMITS. 70; 1:40; 4; 19; 32; 68; 125:20. 80; 1:40; 10; 23; 41; 79; 155:20. 90; 1:30; 2; 14; 18; 42; 88; 166:20. 120; 1:30; 12; 14; 36; 56; 120; 240:20. 180; 1:20; 10; 26; 32; 54; 94; 168; 386:20. DIVE PROCEDURES – TABLE 1-10 Modified in Water Decompression Tables with Nitrox or Air in Water Stops. IN WATER PHASE; * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours. * Ascent procedures; - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - 60 feet per min to the indicated stop or surface if no stops. - Decompression time as specified begins on arrival at stop depth; - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Do not include ascent time as part of stop times; - Refer to Table 1-12 for new RGD at the end of the surface interval. - Delays at earlier stops are not to be compensated for at later water stops; - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT). *50/50 Nitrox Switch; LOSS OF NITROX. - Switch to 50/50 Nitrox at 100 feet during ascent; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; 140 FEET. - Umbilical flush is to be formally logged. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-10 STANDARD AIR WITH NITROX OR AIR DECOMPRESSION. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 100; 90; 80; 70; 60; 50; 40; 30; 20; 10. 5; 0:00; 0; 2:30; C. 10; 2:20; 1; 3:30; E. 15; 2:20; 3; 5:30; G. 20; 2:10; 2; 7; 11:30; H. 25; 2:10; 4; 17; 23:30; K. 30; 2:10; 8; 24; 34:30; L. 40; 2:00; 5; 19; 33; 59:30; N. 50; 2:00; 12; 23; 51; 88:30; O. 60; 1:50; 3; 19; 26; 62; 112:30; Z. SUPERIOR OPERATIONAL DEPTH / TIME LIMITS. 70; 1:50; 11; 19; 39; 75; 146:30. 80; 1:40; 1; 17; 19; 50; 84; 173:30. DIVE PROCEDURES – TABLE 1-10 Modified in Water Decompression Tables with Nitrox or Air in Water Stops. IN WATER PHASE; * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours. * Ascent procedures; - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - 60 feet per min to the indicated stop or surface if no stops. - Decompression time as specified begins on arrival at stop depth; - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Do not include ascent time as part of stop times; - Refer to Table 1-12 for new RGD at the end of the surface interval. - Delays at earlier stops are not to be compensated for at later water stops; - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT). *50/50 Nitrox Switch; LOSS OF NITROX. - Switch to 50/50 Nitrox at 100 feet during ascent; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; 150 FEET. - Umbilical flush is to be formally logged. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-10 STANDARD AIR WITH NITROX OR AIR DECOMPRESSION. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 100; 90; 80; 70; 60; 50; 40; 30; 20; 10. 5; 0:00; 0; 2:40; D. 10; 2:30; 1; 3:40; F. 15; 2:20; 1; 4; 7:40; H. 20; 2:20; 3; 11; 16:40; J. 25; 2:20; 7; 20; 29:40; K. 30; 2:10; 2; 11; 25; 40:40; M. 40; 2:10; 7; 23; 39; 71:40; N. 50; 2:00; 2; 16; 23; 55; 98:40; Z. SUPERIOR OPERATIONAL DEPTH / TIME LIMITS. 60; 2:00; 9; 19; 33; 69; 132:40. 70; 1:50; 1; 17; 22; 44; 80; 166:40. DIVE PROCEDURES – TABLE 1-10 Modified in Water Decompression Tables with Nitrox or Air in Water Stops. IN WATER PHASE; * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours. * Ascent procedures; - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - 60 feet per min to the indicated stop or surface if no stops. - Decompression time as specified begins on arrival at stop depth; - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Do not include ascent time as part of stop times; - Refer to Table 1-12 for new RGD at the end of the surface interval. - Delays at earlier stops are not to be compensated for at later water stops; - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT). *50/50 Nitrox Switch; LOSS OF NITROX. - Switch to 50/50 Nitrox at 100 feet during ascent; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; 160 FEET. - Umbilical flush is to be formally logged. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-10 STANDARD AIR WITH NITROX OR AIR DECOMPRESSION. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 100; 90; 80; 70; 60; 50; 40; 30; 20; 10. 5; 0:00; 0; 2:50; D. 10; 2:40; 2; 4:50; F. 15; 2:30; 2; 5; 9:50; H. 20; 2:30; 4; 15; 21:50; J. 25; 2:20; 2; 7; 23; 34:50; L. 30; 2:20; 4; 13; 26; 45:50; M. 40; 2:10; 1; 10; 23; 45; 81:50; O. 50; 2:10; 5; 18; 23; 61; 109:50; Z. SUPERIOR OPERATIONAL DEPTH / TIME LIMITS. 60; 2:00; 2; 15; 22; 37; 74; 152:50. 70; 2:00; 8; 17; 19; 51; 86; 183:50. 90; 1:50; 12; 12; 14; 34; 52; 120; 246:50. 120; 1:30; 2; 10; 12; 18; 32; 42; 82; 156; 356:50. 180; 1:20; 4; 10; 22; 28; 34; 50; 78; 120; 187; 535:50. DIVE PROCEDURES – TABLE 1-10 Modified in Water Decompression Tables with Nitrox or Air in Water Stops. IN WATER PHASE; * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours. * Ascent procedures; - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - 60 feet per min to the indicated stop or surface if no stops. - Decompression time as specified begins on arrival at stop depth; - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Do not include ascent time as part of stop times; - Refer to Table 1-12 for new RGD at the end of the surface interval. - Delays at earlier stops are not to be compensated for at later water stops; - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT). *50/50 Nitrox Switch; LOSS OF NITROX. - Switch to 50/50 Nitrox at 100 feet during ascent; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; 170 FEET. - Umbilical flush is to be formally logged. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-10 STANDARD AIR WITH NITROX OR AIR DECOMPRESSION. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 100; 90; 80; 70; 60; 50; 40; 30; 20; 10. 5; 0:00; 0; 3:00; D. 10; 2:50; 3; 6:00; F. 15; 2:40; 3; 6; 12:00; I. 20; 2:30; 1; 5; 17; 26:00; J. 25; 2:30; 3; 10; 24; 40:00; L. 30; 2:20; 6; 17; 27; 53:00; N. 40; 2:10; 3; 14; 23; 50; 93:00; O. 50; 2; 9; 19; 30; 65; 128:00; Z. SUPERIOR OPERATIONAL DEPTH / TIME LIMITS. 60; 2:10; 5; 16; 19; 44; 81; 168:00. DIVE PROCEDURES – TABLE 1-10 Modified in Water Decompression Tables with Nitrox or Air in Water Stops. IN WATER PHASE; * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours. * Ascent procedures; - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - 60 feet per min to the indicated stop or surface if no stops. - Decompression time as specified begins on arrival at stop depth; - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Do not include ascent time as part of stop times; - Refer to Table 1-12 for new RGD at the end of the surface interval. - Delays at earlier stops are not to be compensated for at later water stops; - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT). *50/50 Nitrox Switch; LOSS OF NITROX. - Switch to 50/50 Nitrox at 100 feet during ascent; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; 180 FEET. - Umbilical flush is to be formally logged. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-10 STANDARD AIR WITH NITROX OR AIR DECOMPRESSION. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 100; 90; 80; 70; 60; 50; 40; 30; 20; 10. 5; 0:00; 0; 3:10; D. 10; 2:50; 1; 3; 7:10; G. 15; 2:50; 6; 7; 14:10; I. 20; 2:40; 2; 6; 20; 31:10; K. 25; 2:40; 5; 11; 25; 44:10; M. 30; 2:30; 1; 8; 19; 32; 63:10; N. SUPERIOR OPERATIONAL DEPTH / TIME LIMITS. 40; 2:30; 8; 14; 23; 55; 103:10. 50; 2:20; 4; 13; 22; 33; 72; 147:10. 60; 2:20; 10; 17; 19; 50; 84; 183:10. DIVE PROCEDURES – TABLE 1-10 Modified in Water Decompression Tables with Nitrox or Air in Water Stops. IN WATER PHASE; * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours. * Ascent procedures; - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - 60 feet per min to the indicated stop or surface if no stops. - Decompression time as specified begins on arrival at stop depth; - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Do not include ascent time as part of stop times; - Refer to Table 1-12 for new RGD at the end of the surface interval. - Delays at earlier stops are not to be compensated for at later water stops; - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT). *50/50 Nitrox Switch; LOSS OF NITROX. - Switch to 50/50 Nitrox at 100 feet during ascent; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; 190 FEET. - Umbilical flush is to be formally logged. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-10 STANDARD AIR WITH NITROX OR AIR DECOMPRESSION. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 100; 90; 80; 70; 60; 50; 40; 30; 20; 10. 5; 3:10; 1; 4:20; -. 10; 3:00; 1; 4; 8:20; -. 15; 2:50; 1; 4; 10; 18:20; -. 20; 2:50; 3; 7; 27; 40:20; -. 25; 2:50; 7; 14; 25; 49:20; -. 30; 2:40; 2; 9; 22; 37; 73:20; -. SUPERIOR OPERATIONAL DEPTH / TIME LIMITS. 40; 2:30; 2; 8; 17; 23; 59; 112:20; -. 50; 2:30; 6; 16; 22; 39; 75; 161:20; -. 60; 2:20; 2; 13; 17; 24; 51; 89; 199:20; -. 90; 1:50; 1; 10; 10; 12; 12; 30; 38; 74; 134; 324:20; -. 120; 1:40; 6; 10; 10; 10; 24; 28; 40; 64; 98; 180; 473:20; -. DIVE PROCEDURES – TABLE 1-10 Modified in Water Decompression Tables with Nitrox or Air in Water Stops. IN WATER PHASE; * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours. * Ascent procedures; - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - 60 feet per min to the indicated stop or surface if no stops. - Decompression time as specified begins on arrival at stop depth; - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Do not include ascent time as part of stop times; - Refer to Table 1-12 for new RGD at the end of the surface interval. - Delays at earlier stops are not to be compensated for at later water stops; - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT). *50/50 Nitrox Switch; LOSS OF NITROX. - Switch to 50/50 Nitrox at 100 feet during ascent; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; 200 FEET. - Umbilical flush is to be formally logged. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-10 STANDARD AIR WITH NITROX OR AIR DECOMPRESSION. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 100; 90; 80; 70; 60; 50; 40; 30; 20; 10. 5; 3:20; 1; 4:30; -. 10; 3:10; 2; 4; 9:30; -. 15; 3:00; 1; 5; 13; 22:30; -. 20; 3:00; 4; 10; 23; 40:30; -. 25; 2:50; 2; 7; 17; 27; 56:30; -. 30; 2:50; 4; 9; 24; 41; 81:30; -. SUPERIOR OPERATIONAL DEPTH / TIME LIMITS. 40; 2:40; 4; 9; 19; 26; 63; 124:30; -. 50; 2:30; 1; 9; 17; 19; 45; 80; 174:30; -. DIVE PROCEDURES – TABLE 1-10 Modified in Water Decompression Tables with Nitrox or Air in Water Stops. IN WATER PHASE; * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours. * Ascent procedures; - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - 60 feet per min to the indicated stop or surface if no stops. - Decompression time as specified begins on arrival at stop depth; - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Do not include ascent time as part of stop times; - Refer to Table 1-12 for new RGD at the end of the surface interval. - Delays at earlier stops are not to be compensated for at later water stops; - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT). *50/50 Nitrox Switch; LOSS OF NITROX. - Switch to 50/50 Nitrox at 100 feet during ascent; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; 210 FEET. - Umbilical flush is to be formally logged. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-10 STANDARD AIR WITH NITROX OR AIR DECOMPRESSION. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 100; 90; 80; 70; 60; 50; 40; 30; 20; 10. 5; 3:30; 2; 5:40; -. 10; 3:20; 2; 5; 10:40; -. 15; 3:10; 2; 5; 16; 26:40; -. 20; 3:00; 1; 3; 11; 24; 42:40; -. 25; 3:00; 3; 8; 19; 33; 66:40; -. 30; 2:50; 1; 7; 10; 23; 47; 91:40; -. SUPERIOR OPERATIONAL DEPTH / TIME LIMITS. 40; 2:50; 6; 12; 22; 29; 68; 140:40; -. 50; 2:40; 3; 12; 17; 18; 51; 86; 190:40; -. DIVE PROCEDURES – TABLE 1-10 Modified in Water Decompression Tables with Nitrox or Air in Water Stops. IN WATER PHASE; * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours. * Ascent procedures; - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - 60 feet per min to the indicated stop or surface if no stops. - Decompression time as specified begins on arrival at stop depth; - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Do not include ascent time as part of stop times; - Refer to Table 1-12 for new RGD at the end of the surface interval. - Delays at earlier stops are not to be compensated for at later water stops; - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT). *50/50 Nitrox Switch; LOSS OF NITROX. - Switch to 50/50 Nitrox at 100 feet during ascent; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; 220 FEET. - Umbilical flush is to be formally logged. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..