Table 1-26 SURFACE DECOMPRESSION USING OXYGEN WITH NITROX OR AIR IN WATER STOPS. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); SURFACE INTERVAL *; IN CHAMBER ON OXYGEN; NO OF AIR BREAKS; ASCENT; DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 80; 70; 60; 50; 40; 30; 50; 40. 52; 1:36; 3; NOT TO EXCEED 5 MINUTES; 10 MINUTES O2 **; 5:36; K. 90; 1:36; 3; 10; 15; 1; 50:36; N. 120; 1:36; 3; 10; 23; 2; 63:36; 0. 150; 1:36; 3; 10; 31; 2; 71:36; Z. SUPERIOR DIVING OPERATIONAL DEPTH/TIME LIMIT. 180; 1:36; 3; 10; 39; 2; 79:36; Z. DIVE PROCEDURES – TABLE 1-26 Surface Decompression Using Oxygen with Nitrox or Air in Water Stops. IN WATER PHASE; CHAMBER PHASE. * Ascent procedures; - Diver is recompressed to 50 feet in 30 seconds and is to be breathing oxygen within 3 minutes of arriving at 50 feet. - 25 feet per minute to the first stop. - Ascent time between stops & from 30 feet to surface is ONE MINUTE; - Remain at 50 feet for 10 minutes then ascend to 40 feet in 1 minute. - Decompression time as specified begins on arrival at the stop depth; - On arrival at 40 feet the first air break is given prior to starting Oxygen time at this depth. - Do not include ascent time as part of stop times; - Breath Oxygen for the time listed broken into cycles of 20 minutes interspersed with 5 minute air breaks. - Delays at earlier stops are not to be compensated for at later stops. *50/50 Nitrox Switch; ASCENT FROM 40 FEET. - Switch to 50/50 Nitrox at 100 feet during ascent; - ** On completion of total Oxygen time at 40 feet surface the diver at the rate 4 feet per minute (10 minutes). - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; - Diver to breath Oxygen during the entire ascent.. - Umbilical flush is to be formally logged. LOSS OF NITROX. * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Refer to Table 1-12 for new RGD at the end of the surface interval. - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT); 70 FEET. * Surface Interval. - Upon surfacing the diver has 3 and a half minutes maximum to start recompression in the chamber. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-26 SURFACE DECOMPRESSION USING OXYGEN WITH NITROX OR AIR IN WATER STOPS. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); SURFACE INTERVAL *; IN CHAMBER ON OXYGEN; NO OF AIR BREAKS; ASCENT; DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 80; 70; 60; 50; 40; 30; 50; 40. 40; 2:00; 3; NOT TO EXCEED 5 MINUTES; 10 MINUTES O2 **; 6:00; *. 70; 2:00; 3; 10; 14; 1; 50:00; M. 85; 2:00; 3; 10; 20; 2; 61:00; N. 100; 2:00; 3; 10; 26; 2; 67:00; O. 115; 2:00; 3; 10; 31; 2; 72:00; Z. SUPERIOR DIVING OPERATIONAL DEPTH/TIME LIMIT. 130; 2:00; 3; 10; 37; 2; 78:00. 150; 2:00; 3; 10; 44; 3; 90:00. DIVE PROCEDURES – TABLE 1-26 Surface Decompression Using Oxygen with Nitrox or Air in Water Stops. IN WATER PHASE; CHAMBER PHASE. * Ascent procedures; - Diver is recompressed to 50 feet in 30 seconds and is to be breathing oxygen within 3 minutes of arriving at 50 feet. - 25 feet per minute to the first stop. - Ascent time between stops & from 30 feet to surface is ONE MINUTE; - Remain at 50 feet for 10 minutes then ascend to 40 feet in 1 minute. - Decompression time as specified begins on arrival at the stop depth; - On arrival at 40 feet the first air break is given prior to starting Oxygen time at this depth. - Do not include ascent time as part of stop times; - Breath Oxygen for the time listed broken into cycles of 20 minutes interspersed with 5 minute air breaks. - Delays at earlier stops are not to be compensated for at later stops. *50/50 Nitrox Switch; ASCENT FROM 40 FEET. - Switch to 50/50 Nitrox at 100 feet during ascent; - ** On completion of total Oxygen time at 40 feet surface the diver at the rate 4 feet per minute (10 minutes). - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; - Diver to breath Oxygen during the entire ascent.. - Umbilical flush is to be formally logged. LOSS OF NITROX. * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Refer to Table 1-12 for new RGD at the end of the surface interval. - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT); 80 FEET. * Surface Interval. - Upon surfacing the diver has 3 and a half minutes maximum to start recompression in the chamber. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-26 SURFACE DECOMPRESSION USING OXYGEN WITH NITROX OR AIR IN WATER STOPS. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); SURFACE INTERVAL *; IN CHAMBER ON OXYGEN; NO OF AIR BREAKS; ASCENT; DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 80; 70; 60; 50; 40; 30; 50; 40. 32; 2:24; 3; NOT TO EXCEED 5 MINUTES; 10 MINUTES O2 **; 6:24; J. 60; 2:24; 3; 10; 14; 1; 50:24; M. 70; 2:24; 3; 10; 20; 2; 61:24; N. 80; 2:24; 3; 10; 25; 2; 66:24; N. 90; 2:24; 3; 10; 30; 2; 71:24; O. 100; 2:24; 3; 10; 34; 2; 75:24; Z. SUPERIOR DIVING OPERATIONAL DEPTH/TIME LIMIT. 110; 2:24; 3; 10; 39; 2; 80:24. 120; 2:24; 3; 10; 43; 3; 89:24. 130; 2:24; 3; 10; 48; 3; 94:24. DIVE PROCEDURES – TABLE 1-26 Surface Decompression Using Oxygen with Nitrox or Air in Water Stops. IN WATER PHASE; CHAMBER PHASE. * Ascent procedures; - Diver is recompressed to 50 feet in 30 seconds and is to be breathing oxygen within 3 minutes of arriving at 50 feet. - 25 feet per minute to the first stop. - Ascent time between stops & from 30 feet to surface is ONE MINUTE; - Remain at 50 feet for 10 minutes then ascend to 40 feet in 1 minute. - Decompression time as specified begins on arrival at the stop depth; - On arrival at 40 feet the first air break is given prior to starting Oxygen time at this depth. - Do not include ascent time as part of stop times; - Breath Oxygen for the time listed broken into cycles of 20 minutes interspersed with 5 minute air breaks. - Delays at earlier stops are not to be compensated for at later stops. *50/50 Nitrox Switch; ASCENT FROM 40 FEET. - Switch to 50/50 Nitrox at 100 feet during ascent; - ** On completion of total Oxygen time at 40 feet surface the diver at the rate 4 feet per minute (10 minutes). - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; - Diver to breath Oxygen during the entire ascent.. - Umbilical flush is to be formally logged. LOSS OF NITROX. * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Refer to Table 1-12 for new RGD at the end of the surface interval. - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT); 90 FEET. * Surface Interval. - Upon surfacing the diver has 3 and a half minutes maximum to start recompression in the chamber. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-26 SURFACE DECOMPRESSION USING OXYGEN WITH NITROX OR AIR IN WATER STOPS. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); SURFACE INTERVAL *; IN CHAMBER ON OXYGEN; NO OF AIR BREAKS; ASCENT; DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 80; 70; 60; 50; 40; 30; 50; 40. 26; 2:48; 3; NOT TO EXCEED 5 MINUTES; 10 MINUTES O2 **; 6:48; I. 50; 2:48; 3; 10; 14; 1; 50:48; L. 60; 2:48; 3; 10; 20; 2; 61:48; N. 70; 2:48; 3; 10; 26; 2; 67:48; O. 80; 2:48; 3; 10; 32; 2; 73:48; O. 90; 2:48; 3; 10; 38; 2; 79:48; Z. SUPERIOR DIVING OPERATIONAL DEPTH/TIME LIMIT. 100; 2:48; 3; 10; 44; 3; 90:48. 110; 2:48; 3; 10; 49; 3; 95:48. 120; 2:24; 2; 4; 10; 53; 3; 103:24. DIVE PROCEDURES – TABLE 1-26 Surface Decompression Using Oxygen with Nitrox or Air in Water Stops. IN WATER PHASE; CHAMBER PHASE. * Ascent procedures; - Diver is recompressed to 50 feet in 30 seconds and is to be breathing oxygen within 3 minutes of arriving at 50 feet. - 25 feet per minute to the first stop. - Ascent time between stops & from 30 feet to surface is ONE MINUTE; - Remain at 50 feet for 10 minutes then ascend to 40 feet in 1 minute. - Decompression time as specified begins on arrival at the stop depth; - On arrival at 40 feet the first air break is given prior to starting Oxygen time at this depth. - Do not include ascent time as part of stop times; - Breath Oxygen for the time listed broken into cycles of 20 minutes interspersed with 5 minute air breaks. - Delays at earlier stops are not to be compensated for at later stops. *50/50 Nitrox Switch; ASCENT FROM 40 FEET. - Switch to 50/50 Nitrox at 100 feet during ascent; - ** On completion of total Oxygen time at 40 feet surface the diver at the rate 4 feet per minute (10 minutes). - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; - Diver to breath Oxygen during the entire ascent.. - Umbilical flush is to be formally logged. LOSS OF NITROX. * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Refer to Table 1-12 for new RGD at the end of the surface interval. - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT); 100 FEET. * Surface Interval. - Upon surfacing the diver has 3 and a half minutes maximum to start recompression in the chamber. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-26 SURFACE DECOMPRESSION USING OXYGEN WITH NITROX OR AIR IN WATER STOPS. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); SURFACE INTERVAL *; IN CHAMBER ON OXYGEN; NO OF AIR BREAKS; ASCENT; DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 80; 70; 60; 50; 40; 30; 50; 40. 22; 3:12; 3; NOT TO EXCEED 5 MINUTES; 10 MINUTES O2 **; 7:12; H. 40; 3:12; 3; 10; 12; 1; 49:12; L. 50; 3:12; 3; 10; 19; 1; 56:12; M. 60; 3:12; 3; 10; 26; 2; 68:12; N. 70; 3:12; 3; 10; 33; 2; 75:12; O. 80; 2:48; 2; 3; 10; 40; 3; 89:48; Z. SUPERIOR DIVING OPERATIONAL DEPTH/TIME LIMIT. 90; 2:48; 2; 4; 10; 46; 3; 96:48. 100; 2:48; 2; 5; 10; 51; 3; 102:48. 110; 2:48; 2; 12; 10; 54; 3; 112:48. DIVE PROCEDURES – TABLE 1-26 Surface Decompression Using Oxygen with Nitrox or Air in Water Stops. IN WATER PHASE; CHAMBER PHASE. * Ascent procedures; - Diver is recompressed to 50 feet in 30 seconds and is to be breathing oxygen within 3 minutes of arriving at 50 feet. - 25 feet per minute to the first stop. - Ascent time between stops & from 30 feet to surface is ONE MINUTE; - Remain at 50 feet for 10 minutes then ascend to 40 feet in 1 minute. - Decompression time as specified begins on arrival at the stop depth; - On arrival at 40 feet the first air break is given prior to starting Oxygen time at this depth. - Do not include ascent time as part of stop times; - Breath Oxygen for the time listed broken into cycles of 20 minutes interspersed with 5 minute air breaks. - Delays at earlier stops are not to be compensated for at later stops. *50/50 Nitrox Switch; ASCENT FROM 40 FEET. - Switch to 50/50 Nitrox at 100 feet during ascent; - ** On completion of total Oxygen time at 40 feet surface the diver at the rate 4 feet per minute (10 minutes). - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; - Diver to breath Oxygen during the entire ascent.. - Umbilical flush is to be formally logged. LOSS OF NITROX. * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Refer to Table 1-12 for new RGD at the end of the surface interval. - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT); 110 FEET. * Surface Interval. - Upon surfacing the diver has 3 and a half minutes maximum to start recompression in the chamber. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-26 SURFACE DECOMPRESSION USING OXYGEN WITH NITROX OR AIR IN WATER STOPS. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); SURFACE INTERVAL *; IN CHAMBER ON OXYGEN; NO OF AIR BREAKS; ASCENT; DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 80; 70; 60; 50; 40; 30; 50; 40. 18; 3:36; 3; NOT TO EXCEED 5 MINUTES; 10 MINUTES O2 **; 7:36; H. 30; 3:36; 3; 10; 9; 1; 46:36; J. 40; 3:36; 3; 10; 16; 1; 53:36; L. 50; 3:36; 3; 10; 24; 2; 66:36; N. 60; 3:12; 2; 3; 10; 32; 2; 77:12; O. 70; 3:12; 2; 4; 10; 39; 2; 85:12; O. 80; 3:12; 2; 5; 10; 46; 3; 98:12; Z. SUPERIOR DIVING OPERATIONAL DEPTH/TIME LIMIT. 90; 3:12; 4; 8; 10; 51; 3; 108:12. DIVE PROCEDURES – TABLE 1-26 Surface Decompression Using Oxygen with Nitrox or Air in Water Stops. IN WATER PHASE; CHAMBER PHASE. * Ascent procedures; - Diver is recompressed to 50 feet in 30 seconds and is to be breathing oxygen within 3 minutes of arriving at 50 feet. - 25 feet per minute to the first stop. - Ascent time between stops & from 30 feet to surface is ONE MINUTE; - Remain at 50 feet for 10 minutes then ascend to 40 feet in 1 minute. - Decompression time as specified begins on arrival at the stop depth; - On arrival at 40 feet the first air break is given prior to starting Oxygen time at this depth. - Do not include ascent time as part of stop times; - Breath Oxygen for the time listed broken into cycles of 20 minutes interspersed with 5 minute air breaks. - Delays at earlier stops are not to be compensated for at later stops. *50/50 Nitrox Switch; ASCENT FROM 40 FEET. - Switch to 50/50 Nitrox at 100 feet during ascent; - ** On completion of total Oxygen time at 40 feet surface the diver at the rate 4 feet per minute (10 minutes). - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; - Diver to breath Oxygen during the entire ascent.. - Umbilical flush is to be formally logged. LOSS OF NITROX. * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Refer to Table 1-12 for new RGD at the end of the surface interval. - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT); 120 FEET. * Surface Interval. - Upon surfacing the diver has 3 and a half minutes maximum to start recompression in the chamber. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-26 SURFACE DECOMPRESSION USING OXYGEN WITH NITROX OR AIR IN WATER STOPS. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); SURFACE INTERVAL *; IN CHAMBER ON OXYGEN; NO OF AIR BREAKS; ASCENT; DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 80; 70; 60; 50; 40; 30; 50; 40. 15; 4:00; 3; NOT TO EXCEED 5 MINUTES; 10 MINUTES O2 **; 8:00; F. 30; 4:00; 3; 10; 12; 1; 50:00; M. 40; 4:00; 3; 10; 21; 2; 64:00; N. 50; 3:36; 2; 3; 10; 29; 2; 74:36; O. 60; 3:36; 2; 5; 10; 37; 2; 84:36; Z. SUPERIOR DIVING OPERATIONAL DEPTH/TIME LIMIT. 70; 3:36; 2; 7; 10; 45; 3; 99:36. 80; 3:36; 6; 10; 10; 51; 3; 112:36. 90; 3:36; 10; 14; 10; 56; 3; 125:36. DIVE PROCEDURES – TABLE 1-26 Surface Decompression Using Oxygen with Nitrox or Air in Water Stops. IN WATER PHASE; CHAMBER PHASE. * Ascent procedures; - Diver is recompressed to 50 feet in 30 seconds and is to be breathing oxygen within 3 minutes of arriving at 50 feet. - 25 feet per minute to the first stop. - Ascent time between stops & from 30 feet to surface is ONE MINUTE; - Remain at 50 feet for 10 minutes then ascend to 40 feet in 1 minute. - Decompression time as specified begins on arrival at the stop depth; - On arrival at 40 feet the first air break is given prior to starting Oxygen time at this depth. - Do not include ascent time as part of stop times; - Breath Oxygen for the time listed broken into cycles of 20 minutes interspersed with 5 minute air breaks. - Delays at earlier stops are not to be compensated for at later stops. *50/50 Nitrox Switch; ASCENT FROM 40 FEET. - Switch to 50/50 Nitrox at 100 feet during ascent; - ** On completion of total Oxygen time at 40 feet surface the diver at the rate 4 feet per minute (10 minutes). - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; - Diver to breath Oxygen during the entire ascent.. - Umbilical flush is to be formally logged. LOSS OF NITROX. * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Refer to Table 1-12 for new RGD at the end of the surface interval. - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT); 130 FEET. * Surface Interval. - Upon surfacing the diver has 3 and a half minutes maximum to start recompression in the chamber. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-26 SURFACE DECOMPRESSION USING OXYGEN WITH NITROX OR AIR IN WATER STOPS. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); SURFACE INTERVAL *; IN CHAMBER ON OXYGEN; NO OF AIR BREAKS; ASCENT; DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 80; 70; 60; 50; 40; 30; 50; 40. 13; 4:24; 3; NOT TO EXCEED 5 MINUTES; 10 MINUTES O2 **; 8:24; G. 25; 4:24; 3; 10; 11; 1; 49:24; J. 30; 4:24; 3; 10; 15; 1; 53:24; K. 35; 4:24; 3; 10; 20; 2; 63:24; N. 40; 4:00; 2; 3; 10; 24; 2; 70:00; N. 45; 4:00; 2; 4; 10; 29; 2; 76:00; O. 50; 4:00; 2; 6; 10; 33; 2; 82:00; O. 55; 4:00; 2; 7; 10; 38; 2; 88:00; Z. 60; 4:00; 2; 9; 10; 43; 3; 100:00; Z. SUPERIOR DIVING OPERATIONAL DEPTH/TIME LIMIT. 65; 4:00; 4; 11; 10; 48; 3; 109:00. 70; 3:36; 2; 7; 13; 10; 51; 3; 119:36. DIVE PROCEDURES – TABLE 1-26 Surface Decompression Using Oxygen with Nitrox or Air in Water Stops. IN WATER PHASE; CHAMBER PHASE. * Ascent procedures; - Diver is recompressed to 50 feet in 30 seconds and is to be breathing oxygen within 3 minutes of arriving at 50 feet. - 25 feet per minute to the first stop. - Ascent time between stops & from 30 feet to surface is ONE MINUTE; - Remain at 50 feet for 10 minutes then ascend to 40 feet in 1 minute. - Decompression time as specified begins on arrival at the stop depth; - On arrival at 40 feet the first air break is given prior to starting Oxygen time at this depth. - Do not include ascent time as part of stop times; - Breath Oxygen for the time listed broken into cycles of 20 minutes interspersed with 5 minute air breaks. - Delays at earlier stops are not to be compensated for at later stops. *50/50 Nitrox Switch; ASCENT FROM 40 FEET. - Switch to 50/50 Nitrox at 100 feet during ascent; - ** On completion of total Oxygen time at 40 feet surface the diver at the rate 4 feet per minute (10 minutes). - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; - Diver to breath Oxygen during the entire ascent.. - Umbilical flush is to be formally logged. LOSS OF NITROX. * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Refer to Table 1-12 for new RGD at the end of the surface interval. - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT); 140 FEET. * Surface Interval. - Upon surfacing the diver has 3 and a half minutes maximum to start recompression in the chamber. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-26 SURFACE DECOMPRESSION USING OXYGEN WITH NITROX OR AIR IN WATER STOPS. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); SURFACE INTERVAL *; IN CHAMBER ON OXYGEN; NO OF AIR BREAKS; ASCENT; DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 80; 70; 60; 50; 40; 30; 50; 40. 11; 4:48; 3; NOT TO EXCEED 5 MINUTES; 10 MINUTES O2 **; 8:48; G. 25; 4:48; 3; 10; 13; 1; 51:48; K. 30; 4:48; 3; 10; 18; 1; 56:48; L. 35; 4:24; 2; 4; 10; 23; 2; 70:24; N. 40; 4:24; 5; 7; 10; 27; 2; 80:24; N. 45; 4:24; 6; 9; 10; 33; 2; 89:24; O. 50; 4:00; 2; 7; 11; 10; 38; 2; 100:00; O. 55; 3:36; 2; 5; 9; 14; 10; 44; 3; 121:36; Z. DIVE PROCEDURES – TABLE 1-26 Surface Decompression Using Oxygen with Nitrox or Air in Water Stops. IN WATER PHASE; CHAMBER PHASE. * Ascent procedures; - Diver is recompressed to 50 feet in 30 seconds and is to be breathing oxygen within 3 minutes of arriving at 50 feet. - 25 feet per minute to the first stop. - Ascent time between stops & from 30 feet to surface is ONE MINUTE; - Remain at 50 feet for 10 minutes then ascend to 40 feet in 1 minute. - Decompression time as specified begins on arrival at the stop depth; - On arrival at 40 feet the first air break is given prior to starting Oxygen time at this depth. - Do not include ascent time as part of stop times; - Breath Oxygen for the time listed broken into cycles of 20 minutes interspersed with 5 minute air breaks. - Delays at earlier stops are not to be compensated for at later stops. *50/50 Nitrox Switch; ASCENT FROM 40 FEET. - Switch to 50/50 Nitrox at 100 feet during ascent; - ** On completion of total Oxygen time at 40 feet surface the diver at the rate 4 feet per minute (10 minutes). - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; - Diver to breath Oxygen during the entire ascent.. - Umbilical flush is to be formally logged. LOSS OF NITROX. * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Refer to Table 1-12 for new RGD at the end of the surface interval. - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT); 150 FEET. * Surface Interval. - Upon surfacing the diver has 3 and a half minutes maximum to start recompression in the chamber. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-26 SURFACE DECOMPRESSION USING OXYGEN WITH NITROX OR AIR IN WATER STOPS. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); SURFACE INTERVAL *; IN CHAMBER ON OXYGEN; NO OF AIR BREAKS; ASCENT; DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 80; 70; 60; 50; 40; 30; 50; 40. 9; 5:12; 3; NOT TO EXCEED 5 MINUTES; 10 MINUTES O2 **; 9:12; F. 20; 5:12; 3; 10; 11; 1; 50:12; J. 25; 5:12; 3; 10; 16; 1; 55:12; K. 30; 4:48; 2; 4; 10; 21; 2; 68:48; M. 35; 4:48; 4; 7; 10; 26; 2; 78:48; N. 40; 4:24; 3; 6; 9; 10; 32; 2; 92:24; N. 45; 4:00; 3; 4; 8; 12; 10; 38; 2; 108:00; Z. DIVE PROCEDURES – TABLE 1-26 Surface Decompression Using Oxygen with Nitrox or Air in Water Stops. IN WATER PHASE; CHAMBER PHASE. * Ascent procedures; - Diver is recompressed to 50 feet in 30 seconds and is to be breathing oxygen within 3 minutes of arriving at 50 feet. - 25 feet per minute to the first stop. - Ascent time between stops & from 30 feet to surface is ONE MINUTE; - Remain at 50 feet for 10 minutes then ascend to 40 feet in 1 minute. - Decompression time as specified begins on arrival at the stop depth; - On arrival at 40 feet the first air break is given prior to starting Oxygen time at this depth. - Do not include ascent time as part of stop times; - Breath Oxygen for the time listed broken into cycles of 20 minutes interspersed with 5 minute air breaks. - Delays at earlier stops are not to be compensated for at later stops. *50/50 Nitrox Switch; ASCENT FROM 40 FEET. - Switch to 50/50 Nitrox at 100 feet during ascent; - ** On completion of total Oxygen time at 40 feet surface the diver at the rate 4 feet per minute (10 minutes). - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; - Diver to breath Oxygen during the entire ascent.. - Umbilical flush is to be formally logged. LOSS OF NITROX. * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Refer to Table 1-12 for new RGD at the end of the surface interval. - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT); 160 FEET. * Surface Interval. - Upon surfacing the diver has 3 and a half minutes maximum to start recompression in the chamber. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-26 SURFACE DECOMPRESSION USING OXYGEN WITH NITROX OR AIR IN WATER STOPS. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); SURFACE INTERVAL *; IN CHAMBER ON OXYGEN; NO OF AIR BREAKS; ASCENT; DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 80; 70; 60; 50; 40; 30; 50; 40. 7; 5:36; 3; NOT TO EXCEED 5 MINUTES; 10 MINUTES O2 **; 9:36; F. 20; 5:36; 3; 10; 13; 1; 52:36; J. 25; 5:36; 3; 10; 19; 1; 58:36; L. 30; 5:12; 5; 7; 10; 23; 2; 77:12; M. 35; 4:48; 4; 6; 9; 10; 29; 2; 90:48; O. 40; 4:24; 4; 4; 8; 13; 10; 36; 2; 108:24; O. DIVE PROCEDURES – TABLE 1-26 Surface Decompression Using Oxygen with Nitrox or Air in Water Stops. IN WATER PHASE; CHAMBER PHASE. * Ascent procedures; - Diver is recompressed to 50 feet in 30 seconds and is to be breathing oxygen within 3 minutes of arriving at 50 feet. - 25 feet per minute to the first stop. - Ascent time between stops & from 30 feet to surface is ONE MINUTE; - Remain at 50 feet for 10 minutes then ascend to 40 feet in 1 minute. - Decompression time as specified begins on arrival at the stop depth; - On arrival at 40 feet the first air break is given prior to starting Oxygen time at this depth. - Do not include ascent time as part of stop times; - Breath Oxygen for the time listed broken into cycles of 20 minutes interspersed with 5 minute air breaks. - Delays at earlier stops are not to be compensated for at later stops. *50/50 Nitrox Switch; ASCENT FROM 40 FEET. - Switch to 50/50 Nitrox at 100 feet during ascent; - ** On completion of total Oxygen time at 40 feet surface the diver at the rate 4 feet per minute (10 minutes). - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; - Diver to breath Oxygen during the entire ascent.. - Umbilical flush is to be formally logged. LOSS OF NITROX. * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Refer to Table 1-12 for new RGD at the end of the surface interval. - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT); 170 FEET. * Surface Interval. - Upon surfacing the diver has 3 and a half minutes maximum to start recompression in the chamber. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-26x SURFACE DECOMPRESSION USING OXYGEN WITH NITROX OR AIR IN WATER STOPS. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); SURFACE INTERVAL *; IN CHAMBER ON OXYGEN; NO OF AIR BREAKS; ASCENT; DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 80; 70; 60; 50; 40; 30; 50; 40. 15; 5:36; 4; 6; NOT TO EXCEED 5 MINUTES; 10; 20; 1; 10 MINUTES O2 **; 59:36; NO REPET DIVES. 30; 4:48; 2; 3; 5; 10; 10; 40; 2; 88:48. SUPERIOR DIVING OPERATIONAL DEPTH/TIME LIMIT. 45; 4:00; 2; 4; 6; 12; 18; 10; 50; 2; 120:00. NOTE: NO REPETITIVE DIVES ALLOWED ON THESE TABLES. DIVE PROCEDURES – TABLE 1-26 Surface Decompression Using Oxygen with Nitrox or Air in Water Stops. IN WATER PHASE; CHAMBER PHASE. * Ascent procedures; - Diver is recompressed to 50 feet in 30 seconds and is to be breathing oxygen within 3 minutes of arriving at 50 feet. - 25 feet per minute to the first stop. - Ascent time between stops & from 30 feet to surface is ONE MINUTE; - Remain at 50 feet for 10 minutes then ascend to 40 feet in 1 minute. - Decompression time as specified begins on arrival at the stop depth; - On arrival at 40 feet the first air break is given prior to starting Oxygen time at this depth. - Do not include ascent time as part of stop times; - Breath Oxygen for the time listed broken into cycles of 20 minutes interspersed with 5 minute air breaks. - Delays at earlier stops are not to be compensated for at later stops. *50/50 Nitrox Switch; ASCENT FROM 40 FEET. - Switch to 50/50 Nitrox at 100 feet during ascent; - ** On completion of total Oxygen time at 40 feet surface the diver at the rate 4 feet per minute (10 minutes). - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; - Diver to breath Oxygen during the entire ascent.. - Umbilical flush is to be formally logged. LOSS OF NITROX. * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Refer to Table 1-12 for new RGD at the end of the surface interval. - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT); 180 FEET. * Surface Interval. - Upon surfacing the diver has 3 and a half minutes maximum to start recompression in the chamber. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-26x SURFACE DECOMPRESSION USING OXYGEN WITH NITROX OR AIR IN WATER STOPS. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); SURFACE INTERVAL *; IN CHAMBER ON OXYGEN; NO OF AIR BREAKS; ASCENT; DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 80; 70; 60; 50; 40; 30; 50; 40. 15; 5:36; 2; 4; 6; NOT TO EXCEED 5 MINUTES; 10; 20; 1; 10 MINUTES O2 **; 61:36; NO REPET DIVES. 30; 5:12; 3; 5; 8; 12; 10; 50; 2; 103:12. SUPERIOR DIVING OPERATIONAL DEPTH/TIME LIMIT. 45; 4:24; 2; 5; 10; 18; 26; 10; 50; 2; 145:24. NOTE: NO REPETITIVE DIVES ALLOWED ON THESE TABLES. DIVE PROCEDURES – TABLE 1-26 Surface Decompression Using Oxygen with Nitrox or Air in Water Stops. IN WATER PHASE; CHAMBER PHASE. * Ascent procedures; - Diver is recompressed to 50 feet in 30 seconds and is to be breathing oxygen within 3 minutes of arriving at 50 feet. - 25 feet per minute to the first stop. - Ascent time between stops & from 30 feet to surface is ONE MINUTE; - Remain at 50 feet for 10 minutes then ascend to 40 feet in 1 minute. - Decompression time as specified begins on arrival at the stop depth; - On arrival at 40 feet the first air break is given prior to starting Oxygen time at this depth. - Do not include ascent time as part of stop times; - Breath Oxygen for the time listed broken into cycles of 20 minutes interspersed with 5 minute air breaks. - Delays at earlier stops are not to be compensated for at later stops. *50/50 Nitrox Switch; ASCENT FROM 40 FEET. - Switch to 50/50 Nitrox at 100 feet during ascent; - ** On completion of total Oxygen time at 40 feet surface the diver at the rate 4 feet per minute (10 minutes). - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; - Diver to breath Oxygen during the entire ascent.. - Umbilical flush is to be formally logged. LOSS OF NITROX. * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Refer to Table 1-12 for new RGD at the end of the surface interval. - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT); 190 FEET. * Surface Interval. - Upon surfacing the diver has 3 and a half minutes maximum to start recompression in the chamber. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-26x SURFACE DECOMPRESSION USING OXYGEN WITH NITROX OR AIR IN WATER STOPS. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); SURFACE INTERVAL *; IN CHAMBER ON OXYGEN; NO OF AIR BREAKS; ASCENT; DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 80; 70; 60; 50; 40; 30; 50; 40. 15; 6:00; 2; 5; 7; NOT TO EXCEED 5 MINUTES; 10; 20; 1; 10 MINUTES O2 **; 64:00; NO REPET DIVES. 30; 4:48; 2; 4; 6; 10; 16; 10; 50; 2; 112:48. SUPERIOR DIVING OPERATIONAL DEPTH/TIME LIMIT. 45; 4:45; 2; 5; 12; 20; 30; 10; 70; 3; 167:48. NOTE: NO REPETITIVE DIVES ALLOWED ON THESE TABLES. DIVE PROCEDURES – TABLE 1-26 Surface Decompression Using Oxygen with Nitrox or Air in Water Stops. IN WATER PHASE; CHAMBER PHASE. * Ascent procedures; - Diver is recompressed to 50 feet in 30 seconds and is to be breathing oxygen within 3 minutes of arriving at 50 feet. - 25 feet per minute to the first stop. - Ascent time between stops & from 30 feet to surface is ONE MINUTE; - Remain at 50 feet for 10 minutes then ascend to 40 feet in 1 minute. - Decompression time as specified begins on arrival at the stop depth; - On arrival at 40 feet the first air break is given prior to starting Oxygen time at this depth. - Do not include ascent time as part of stop times; - Breath Oxygen for the time listed broken into cycles of 20 minutes interspersed with 5 minute air breaks. - Delays at earlier stops are not to be compensated for at later stops. *50/50 Nitrox Switch; ASCENT FROM 40 FEET. - Switch to 50/50 Nitrox at 100 feet during ascent; - ** On completion of total Oxygen time at 40 feet surface the diver at the rate 4 feet per minute (10 minutes). - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; - Diver to breath Oxygen during the entire ascent.. - Umbilical flush is to be formally logged. LOSS OF NITROX. * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Refer to Table 1-12 for new RGD at the end of the surface interval. - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT); 200 feet. * Surface Interval. - Upon surfacing the diver has 3 and a half minutes maximum to start recompression in the chamber. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-26x SURFACE DECOMPRESSION USING OXYGEN WITH NITROX OR AIR IN WATER STOPS. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); SURFACE INTERVAL *; IN CHAMBER ON OXYGEN; NO OF AIR BREAKS; ASCENT; DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 80; 70; 60; 50; 40; 30; 50; 40. 15; 6:24; 3; 4; 8; NOT TO EXCEED 5 MINUTES; 10; 20; 1; 10 MINUTES O2 **; 65:24; NO REPET DIVES. 30; 5:12; 2; 4; 7; 12; 18; 10; 60; 3; 132:12. SUPERIOR DIVING OPERATIONAL DEPTH/TIME LIMIT. 45; 5:12; 2; 6; 14; 24; 36; 10; 70; 3; 181:12. NOTE: NO REPETITIVE DIVES ALLOWED ON THESE TABLES. DIVE PROCEDURES – TABLE 1-26 Surface Decompression Using Oxygen with Nitrox or Air in Water Stops. IN WATER PHASE; CHAMBER PHASE. * Ascent procedures; - Diver is recompressed to 50 feet in 30 seconds and is to be breathing oxygen within 3 minutes of arriving at 50 feet. - 25 feet per minute to the first stop. - Ascent time between stops & from 30 feet to surface is ONE MINUTE; - Remain at 50 feet for 10 minutes then ascend to 40 feet in 1 minute. - Decompression time as specified begins on arrival at the stop depth; - On arrival at 40 feet the first air break is given prior to starting Oxygen time at this depth. - Do not include ascent time as part of stop times; - Breath Oxygen for the time listed broken into cycles of 20 minutes interspersed with 5 minute air breaks. - Delays at earlier stops are not to be compensated for at later stops. *50/50 Nitrox Switch; ASCENT FROM 40 FEET. - Switch to 50/50 Nitrox at 100 feet during ascent; - ** On completion of total Oxygen time at 40 feet surface the diver at the rate 4 feet per minute (10 minutes). - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; - Diver to breath Oxygen during the entire ascent.. - Umbilical flush is to be formally logged. LOSS OF NITROX. * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Refer to Table 1-12 for new RGD at the end of the surface interval. - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT); 210 FEET. * Surface Interval. - Upon surfacing the diver has 3 and a half minutes maximum to start recompression in the chamber. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..

Table 1-26x SURFACE DECOMPRESSION USING OXYGEN WITH NITROX OR AIR IN WATER STOPS. BOTTOM TIME (MINS); TIME TO 1ST STOP; IN WATER Stops (mins) at Depths (feet); SURFACE INTERVAL *; IN CHAMBER ON OXYGEN; NO OF AIR BREAKS; ASCENT; DECOMP TIME (MINS); REPET GROUP. Breathing Nitrox or Air. 80; 70; 60; 50; 40; 30; 50; 40. 15; 6:24; 2; 4; 5; 10; NOT TO EXCEED 5 MINUTES; 10; 20; 1; 10 MINUTES O2 **; 91:24; NO REPET DIVES. 30; 5:36; 2; 5; 9; 14; 20; 10; 60; 3; 139:24. SUPERIOR DIVING OPERATIONAL DEPTH/TIME LIMIT. 45; 5:36; 2; 6; 16; 30; 42; 10; 70; 3; 195:36. NOTE: NO REPETITIVE DIVES ALLOWED ON THESE TABLES. DIVE PROCEDURES – TABLE 1-26 Surface Decompression Using Oxygen with Nitrox or Air in Water Stops. IN WATER PHASE; CHAMBER PHASE. * Ascent procedures; - Diver is recompressed to 50 feet in 30 seconds and is to be breathing oxygen within 3 minutes of arriving at 50 feet. - 25 feet per minute to the first stop. - Ascent time between stops & from 30 feet to surface is ONE MINUTE; - Remain at 50 feet for 10 minutes then ascend to 40 feet in 1 minute. - Decompression time as specified begins on arrival at the stop depth; - On arrival at 40 feet the first air break is given prior to starting Oxygen time at this depth. - Do not include ascent time as part of stop times; - Breath Oxygen for the time listed broken into cycles of 20 minutes interspersed with 5 minute air breaks. - Delays at earlier stops are not to be compensated for at later stops. *50/50 Nitrox Switch; ASCENT FROM 40 FEET. - Switch to 50/50 Nitrox at 100 feet during ascent; - ** On completion of total Oxygen time at 40 feet surface the diver at the rate 4 feet per minute (10 minutes). - Nitrox decompression mix is not to be connected to the manifold until the diver passes 100 feet. - On surface divers umbilical is to be flushed thoroughly with air prior to starting next dive; - Diver to breath Oxygen during the entire ascent.. - Umbilical flush is to be formally logged. LOSS OF NITROX. * REPETITIVE DIVES – Are only permitted after a minimum surface interval of 6 hours; - If there is a loss of Nitrox, switch diver to Air and decompress according to table. - Previous bottom time must have been such that divers repet group was no greater than ‘O’. - Refer to table 1-10 for Repetitive Group Designator (RGD) not to exceed ‘O’. - Refer to Table 1-12 for new RGD at the end of the surface interval. - Using new RGD refer to Table 1-13 for Residual Nitrogen Time (RNT); 220 Feet. * Surface Interval. - Upon surfacing the diver has 3 and a half minutes maximum to start recompression in the chamber. NOTE: On any occasion that the Depth or Bottom Time falls within 2 feet or 2 minutes of a Depth or Time increment of a schedule use the next Deeper and/or next longer schedule..