| Job Description : SANDBAG SUPPORTS |  | Date : |
| --- | --- | --- |
| Location : | Vessel : | Job No : |
| Supervisor : | Client : |  |

| Sequence of Basic Job Steps | Potential Accidents or Hazards | Recommended Safe Job Procedure |
| --- | --- | --- |
| Hook up bags to crane | Rigging failure of bag slings Uncontrolled swing of bags | Inspect all bag slings Use tag lines to control bag till it’s secured to downline. |
| Lower bags to diver. | Entanglement Bags dropping out of sack Diver not locating load | Proper umbilical management Do not lower bags over diver. Lower before diver enters water or lower away from divers position Use light sticks |
| Set bags for support | Entrapment or entanglement Pinch points | Proper umbilical management Diver awareness bag position to avoid pinch points. Good communication between diver, supervisor and crane operator. |
| Disconnect crane from bag and recover to surface | Entrapment or entanglement | Swing crane away from diver and down line. Diver stands clear of crane till clear to surface. |
| Place bags for support | Back strain Entrapment or entanglement Bags stacked high falling on diver | Lift with legs and not back Diver awareness of umbilical and his position. Maintain a 45º angle on stacked bags. |

| ADDITIONAL SAFETY EQUIPMENT REQUIRED FOR THIS JOB |  |  |  |
| --- | --- | --- | --- |
| ___Work Vest | ___Permit | ___Fire Extinguisher | ___Cotton Gloves |
| ___Respirator | ___Fall Protection | ___Hearing Protection | ___Leather Gloves |
| ___Barricade | ___Face Shield | ___Lock-Out/Tag-Out | ___Chemical Gloves |

| ATTENDING PERSONNEL |  |  |
| --- | --- | --- |